3GPP TSG-RAN WG2 Meeting #129                                    R2-2500468

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.1.2.2

Source: 	Fujitsu

Title: 	Discussion on LCM for UE-sided Model for Beam Management Use Case

Document for:	Discussion

Introduction

In previous meetings, RAN2 made several agreements on functionality identification [1][5][6] related issues and one LS [2] has been sent to RAN1 pending for response. In this contribution, we would like to give our views on the following aspects of LCM for UE-sided model for beam management use case：

- Functionality LCM.

- Data Collection and Training.

- Performance Monitoring.

Discussion

Functionality LCM

In previous RAN2 #127bis and #128 meetings, the following agreements were made for the functionality of LCM:

[RAN2 #127Bis]

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality is signaled. FFS on the applicability reporting content.

- FFS if inference configuration can be signaled in step3.

[RAN2 #128]

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

According to both the previous long offline email discussion [2] and the RAN1 conclusions [3][4], the contents of applicability reporting may vary from one simple indicator to a bunch of detailed information of the selected functionalities/FGs, anyway it is dependent on the configuration from network side and the format of associated IDs and if UE is allowed to finalize the functionality selection based on previous information exchange, e.g., if NW configures UE to down-select pre-defined associated IDs based on UE-side additional conditions, then UE may simply report one indicator as the format of one integer; if NW is willing to retain the decision then UE may need to report more information of the corresponding associated IDs to NW. From RAN2 point of view, the signaling design may need to wait for the potential down selection from the aforementioned options.

Proposal 1 The contents of applicability reporting are subject to NW configuration and details of associated ID.

Proposal 2 Both Associated IDs and detailed information (e.g., functionality/model meta information which is not reported via step 2) can be reported to determine applicability.

Aligned with our above proposals, we believe that the inference configuration can be signaled via either step 5 or step 3 and UE may respond accordingly. In some cases (e.g., functionality switching due to mediocre performance), UE may need to embark on the inference instantly as soon as possible, a pre-configuration of inference via step 3 is necessary for saving signaling and time overhead.

Proposal 3 The inference configuration can be signaled via either step 5 or step 3.

As for non-applicable functionality reporting, multiple options are available, e.g., UE may simply report an error code to reflect the fact that the previously selected functionality is not available anymore, or UE may proactively report the preferred new functionality to NW for further decision. NW may control via measurement/report configuration or other RRC signaling for different reporting options.

Proposal 4 For non-applicable functionality reporting, UE may reactively report the fact that the current functionality is not available or proactively report more information such as preferred new functionality to NW, the reporting options are subject to NW configuration.

Data Collection and Training

There are separate agenda items for both NW and UE-side data collection, however, those discussions are mainly for the data storage, collection and reporting procedure. The LCM configuration for data collection and functionality/model training will be discussed in this paper.

In the RAN2 128 meeting, the following agreements were given:

6. For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration. 7. For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains. 8. The network can provide the data collection configuration (at any point in time), with or without UE request. 9. The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate requests for data collection. 10. FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration.

Although the above proposals indicate that NW has control over the procedure of UE-side data collection, the training entity will be on UE-side (either UE or UE-side OTT server), so NW may not have the knowledge on UE-side AI/ML functionality/model, and/or data collection requirements.

It may have at least two possibilities as follows:

- There is no information exchange (e.g., applicability reporting) for UE-side AI/ML functionalities before UE request for training. In this case, NW may need the UE preferred measurement configuration while UE must have the NW-side additional condition to make sure the training/inference consistency in the future. Therefore, UE may either send a request to simply ask NW for the indication of allowing UE to start the data collection procedure or ask for more information such as NW-side additional conditions.

- There have been information exchanges (e.g., applicability reporting) for UE-side AI/ML functionalities before UE request for training. In this case, UE already knows the current NW-side additional conditions, so at least from UE-side, there is no need to request further NW-side information, it may simply ask NW to allow it to start the data collection procedure with potential functionality related information for NW to facilitate its configuration.

Observation 1 The contents contained in UE request depend on NW-side knowledge about UE-side AI/ML functionality and data collection.

Proposal 5 In addition to asking permission to start the data collection procedure, UE request for data collection for training can optionally contain at least the following, which is subject to NW configuration:

- UE-side AI/ML functionality information.

- UE-side preferred configuration on data collection.

- Requested NW-side information (e.g., Additional conditions/associated IDs).

As for the indication that UE cannot perform data collection based on received configuration, it seems that UE may have the right to decide by itself, since this is UE-side functionality/model, so UE itself is responsible for the training procedure and quality. If UE is urgent to collect data for training, then it may report to NW for further configuration, but if it does not think it is necessary to continue data collection all the time, it may choose to keep silent. NW does not need to worry if UE can perform data collection or not.

Proposal 6 It is not mandatory to have an indication from UE to network when UE can’t perform data collection based on received configuration.

Performance Monitoring

Performance monitoring of UE-side model for beam management has been continuously discussed in RAN1 with minor progress per meeting, the following agreements were made during Rel-19 [3][4]:

Agreement [RAN1 #117] For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report. - FFS on other contents. - The report is at least configured/triggered by NW. - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report. - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring. Agreement [RAN1 #118] For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resource for monitoring. - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resource for monitoring. - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam. - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g., whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

It can be seen from the above agreement that RAN1 has not determined whether the down-selection between Type I/II monitoring or between option 1/2 of Type I monitoring should be made, nor the details of reporting from UE to NW for option 2 monitoring. RAN1 will continue discussing the remaining issues for the next several consecutive meetings.

Observation 2 The details of UE-side BM performance monitoring are still pending further RAN1 conclusions.

RAN2 needs to wait for further RAN1 input for further actions. However, no matter if NW or UE derives the metrics and what will be reported from UE to NW, RAN2 can have some discussions on at least the mechanism to trigger the performance monitoring, and the potential signaling to complete the monitoring procedure.

Proposal 7 RAN2 starts at least the following discussions while waiting for further RAN1 input.

- Mechanism to trigger the performance monitoring procedure.

- Potential signaling to complete the performance monitoring procedure.

From RAN2 point of view, the performance monitoring procedure can be triggered either periodically or event-based, e.g., for option 1, NW can configure UE to periodically report L1-RSRP or BM index to assist NW derives the monitoring metrics, or UE can trigger the procedure for certain events such as UE moving or poor performance.

Proposal 8 The performance monitoring procedure can be triggered either periodically or event based.

For the signaling of the reporting, it is natural that L1 signaling to be used as baseline for L1-RSRP or BM index for option 1 as well as the listed alternatives in option 2. MAC/RRC signaling are also valid to be used regarding the near-real-time latency requirements of performance monitoring.

Proposal 9 L1 signaling can be used as the baseline for the reports of option1 and option2. MAC and/or RRC signaling can be further studied.

Conclusion

Proposal 1 The contents of applicability reporting are subject to NW configuration and details of associated ID.

Proposal 2 Both Associated IDs and detailed information (e.g., functionality/model meta information which is not reported via step 2) can be reported to determine applicability.

Proposal 3 The inference configuration can be signaled via either step 5 or step 3.

Proposal 4 For non-applicable functionality reporting, UE may reactively report the fact that the current functionality is not available or proactively report more information such as preferred new functionality to NW, the reporting options are subject to NW configuration.

Observation 1 The contents contained in UE request depend on NW-side knowledge about UE-side AI/ML functionality and data collection.

Proposal 5 In addition to asking permission to start the data collection procedure, UE request for data collection for training can optionally contain at least the following, which is subject to NW configuration:

- UE-side AI/ML functionality information.

- UE-side preferred configuration on data collection.

- Requested NW-side information (e.g., Additional conditions/associated IDs).

Proposal 6 It is not mandatory to have an indication from UE to network when UE can’t perform data collection based on received configuration.

Observation 2 The details of UE-side BM performance monitoring are still pending further RAN1 conclusions.

Proposal 7 RAN2 starts at least the following discussions while waiting for further RAN1 input.

- Mechanism to trigger the performance monitoring procedure.

- Potential signaling to complete the performance monitoring procedure.

Proposal 8 The performance monitoring procedure can be triggered either periodically or event based.

Proposal 9 L1 signaling can be used as the baseline for the reports of option1 and option2. MAC and/or RRC signaling can be further studied.

References

[1] RAN2 127bis Chair Notes.

[2] R2-2407848 LS on applicable functionality reporting for beam management UE-sided model, 3GPP TSG RAN WG2#127	, Maastricht, Netherlands, Aug 19th – 23rd, 2024.

[3] Xiaodong's Session Notes RAN1#117 (9.1 AI&ML).

[4] Xiaodong's Session Notes RAN1#118 (9.1 AI&ML).

[5] RAN2 127 Chair Notes.

[6] RAN2 128 Chair Notes.