3GPP TSG-RAN WG2 Meeting #129                                    R2-2500469

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.1.2.3

Source: 	Fujitsu

Title: 	Discussion on LCM for Positioning Use Case

WID/SID:	NR_AIML_air

Document for:	Discussion and Decision

- Introduction

In this contribution, we would like to give our views on the LCM for positioning use cases. As agreed in the RANP meeting, both prioritized NW-side and UE-side use cases will be discussed, including the following:

- UE-side model:

  - UE-side model (Case 1)

- NW-side models:

  - gNB-side model (Case 3a)

  - LMF-side model (Case 3b)

- Discussion

  - UE-side model (Case 1)

2.1.1 Functionality-based LCM

In the RAN2 128 meeting [1], the following agreements were given:

Agreements: 1 For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability changes, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered.

And in [3], an email discussion has been set up for post meeting discussion. In this contribution, we would like to give our views on functionality-based LCM based on the above agreements and the email discussion.

Positioning Method

One outstanding issue is whether AI/ML for positioning would be introduced as a new positioning method in related specs such as TS38.305/TS37.355. As indicated in [3], we believe that the criteria for creating new positioning methods is whether the new measurement type is necessary or not for AI/ML, as it is the criteria in legacy positioning framework.

For UE-side model Case 1, it has been widely agreed in TR38.843[4] and current Rel-19 work item discussion that UE may perform DL-PRS based measurements which have not been defined in TS38.215, e.g., PDP/DP. Although the final conclusions are not yet made for down selection and compromise between sample-based and path-based measurements, it is probable that exiting measurement type is not sufficient for implementing Case 1. Therefore, we would like to have a newly defined positioning method for AI/ML positioning UE-side model case 1.

Proposal 1 A newly defined positioning method may be introduced to support AI/ML positioning case 1, since new measurement types are necessary for generating model input.

Fallback Configuration

We think it is beneficial to have one fallback scheme to guarantee the positioning accuracy in case of AI/ML failure, the most two crucial KPIs for positioning are accuracy and latency, UE may switch to another better positioning method with pre-configuration from LMF to avoid the service discontinuity and extra signaling exchange overhead.

We think the fallback strategy is also aligned with the discussion of current capability/applicability reporting procedure. LMF is the entity to decide which positioning method(s) can be used, and it may request multiple capabilities/applicability via LPP signaling, after the applicability reporting, LMF may send indications and/or configurations of more than one positioning method including AI/ML and legacy for UE.

The configuration for AI/ML and legacy can be largely shared, the similarity on configuration between AI/ML and legacy method can be one of the considerations for LMF to select the fallback.

Proposal 2 New fallback configuration (e.g., pre-configuration) for AI/ML positioning case 1 may be introduced to guarantee the accuracy and latency of positioning service.

Proposal 3 Fallback and functionality switching may be integrated into one procedure, details can be further discussed.

Inference Configuration

Similar to our parallel discussion for beam management [5], we believe the inference configuration can be signaled via either step 3 or step 5 so that more flexibility can be achieved. UE may start the AI/ML inference once the applicability is determined, without waiting for further signaling.

However, similar to the discussion for beam management in RAN2, there are still no final clarifications or conclusions on inference configuration and other configurations. The down selection between step 5 and step 3 may only be available after finalizing the detailed contents of the inference configuration.

Proposal 4 NW may signal the inference configuration in step 5 or step 3.

Unsolicited Applicability Reporting

In legacy LPP framework, UE may send unsolicited capability information upon NW request, however, the content to be reported can be decided by UE itself if no explicit configuration is given. In AI/ML, we suggest following this mechanism as a baseline.

Proposal 5 UE may decide the contents to be reported via step 4 if no explicit NW configuration is given.

2.1.2 Performance Monitoring

In RAN2 #125bis meeting, the following agreements were given:

Agreements: 1 For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). 2 “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19.

For positing case 1, both UE and LMF can be used to calculate the monitoring metrics according to RAN1’s current discussion. The feasibility of the two options is still under investigation in RAN1, so RAN2 may wait for further RAN1 progress.

According to the current LPP signaling and framework design, even if UE can make decision of the preliminary monitoring results, a final confirmation from LMF is preferred.

Proposal 6 For detailed design of functionality management, RAN2 wait for RAN1 progress on UE/LMF monitoring options of positioning case 1.

Proposal 7 For positioning case 1, even if UE can calculate the monitoring metrics and make preliminary decisions of the monitoring results, final confirmation from LMF is preferred.

2.1.3 On Questions to RAN1

In [3], another issue is whether some questions need to be sent as LS for further RAN1 discussion. However, concerning the current status of RAN1 discussion, maybe it is not a proper timing to send further questions to RAN1. Firstly, some of the potential questions are not newly emerging, e.g., granularity of the supported/applicable functionality, RAN1 has already been aware of these issues and discussed to some extent, though they found it is not easy to converge to consensus. Secondly, a common problem for all AI/ML subtopics is the time limit for discussion. There are too many issues to be discussed per meeting, so the Feature Lead in RAN1 may find it difficult to squeeze another time slot to accommodate RAN2 incoming LS. Therefore, we suggest that RAN2 does not send LS to RAN1 at this moment.

Proposal 8 RAN2 does not send LS to RAN1 at this moment on AI/ML positioning case 1 LCM questions.

  - NW-side model (Case 3a/3b)

In RAN2 #125bis meeting [2], the following agreements were given for positioning NW-side model LCM:

Agreements: For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and waits for any further inputs from other WGs. For POS, RAN2 assumes that NRPPa is used for the signaling between gNB and LMF for case 3a and 3b and the detailed signaling design is up to RAN3.

According to the above agreements, for the performance monitoring for case 3a, either gNB or LMF can derive monitoring metrics or make preliminary decisions. gNB and LMF are both NW entities, but LMF is superior to gNB during LCM procedure, so the following options are available for Case 3a monitoring decision making.

- gNB makes both the preliminary and final monitoring decisions and reports to LMF.

- gNB makes the preliminary monitoring decision while LMF makes the final monitoring decision/confirmation.

- LMF makes the preliminary and final monitoring decisions.

Further study of the details of the above three options depends on further RAN1 progress.

For case 3b, LMF can derive the performance monitoring metrics and make the monitoring decisions. Some assistance information from gNB/PRU/UE may be necessary for case 3b metrics calculation. The details depend on RAN1 progress.

Potential NRPPa signaling design for decision making, monitoring assistance information between gNB and LMF need to involve RAN3.

Proposal 9 Both gNB and LMF can make monitoring decision for case 3a. The details depend on further RAN1 progress.

Proposal 10 LMF can make monitoring decision for case 3b. RAN2 wait for further RAN1 progress on potential assistance information/data for metrics deriving and decision making of case 3b.

- Conclusion

Proposal 1 A newly defined positioning method may be introduced to support AI/ML positioning case 1, since new measurement types are necessary for generating model input.

Proposal 2 New fallback configuration (e.g., pre-configuration) for AI/ML positioning case 1 may be introduced to guarantee the accuracy and latency of positioning service.

Proposal 3 Fallback and functionality switching may be integrated into one procedure, details can be further discussed.

Proposal 4 NW may signal the inference configuration in step 5 or step 3.

Proposal 5 UE may decide the contents to be reported via step 4 if no explicit NW configuration is given.

Proposal 6 For detailed design of functionality management, RAN2 wait for RAN1 progress on UE/LMF monitoring options of positioning case 1.

Proposal 7 For positioning case 1, even if UE can calculate the monitoring metrics and make preliminary decisions of the monitoring results, final confirmation from LMF is preferred.

Proposal 8 RAN2 does not send LS to RAN1 at this moment on AI/ML positioning case 1 LCM questions.

Proposal 9 Both gNB and LMF can make monitoring decision for case 3a. The details depend on further RAN1 progress.

Proposal 10 LMF can make monitoring decision for case 3b. RAN2 wait for further RAN1 progress on potential assistance information/data for metrics deriving and decision making of case 3b.

- References

[1] RAN2 meeting Chair Note, 3GPP TSG RAN WG2 Meeting #128, 2024.

[2] RAN2 meeting Chair Note, 3GPP TSG RAN WG2 Meeting #125bis, 2024.

[3] R2-24XXXXX, Summary of LCM Procedure for Positioning Case1 (Ericsson), 2024.

[4] TR38.843.

[5] R2-2500468, Discussion on LCM for UE-sided Model for Beam Management Use Case, Fujitsu, 3GPP TSG RAN WG2 Meeting #129, Athens, Greece, February 17th-21st , 2025