3GPP TSG-RAN WG2 Meeting #129                                    R2-2500470

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.1.3

Source: 	Fujitsu

Title: 	Discussion on NW side Data Collection

WID/SID:	NR_AIML_air

Document for:	Discussion and Decision

Introduction

In last RAN2 #127, #127bis and #128 meeting [1][2][8], there were discussions on the NW side data collection of beam management use cases and several agreements were achieved, while the discussion for positioning is still pending. In this contribution, we would like to discuss the remaining issues of the detailed solutions for NW side data collection of beam management and positioning use cases for model training.

Discussion

Data Collection for Beam Management

Event-based Data Logging

In the previous meetings, the following agreements were made regarding event-based data logging:

[RAN2 #128] Focus on the following three radio condition event-based logging - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2). - Beam based events (e.g. beam becomes top 1 beam and number of measurements is less than configured value). - L1 beam level measurement. [RAN2 #127bis] Event-triggered data logging will be supported. At least radio condition-based event triggered logging will be supported. FFS the details of radio condition-based event. FFS if other events are supported.

In addition, the [POST128][019] Email discussion [9] has been set up for the above-mentioned down-selection given by #128 agreement. Combining with the LS reply from RAN1[3], we have the following discussions.

L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2).

The motivation of event-trigged data logging is to collect useful data with the minimum cost and overhead (specifically UE-side), therefore, the event should be defined as a condition to judge whether the UE can get proper data, which is useful for model training. The cell-level L3 RSRP can be considered because if L3 RSRP is lower than a threshold, basically the first priority of the UE is handover, not generating data for data collection.

As for the two proposed events, we consider A1-like is necessary, since if NW would like to collect data which the L3-RSRP is lower than one threshold, it may simply configure no event and collect all data without considering the RSRP level. So, A1-like L3 RSRP may be considered as baseline event while A2-like for further discussion.

Proposal 1 L3 serving cell measurement-based events are used to support data collection for AI/ML beam management, and A1-like event is considered to be the baseline.

Beam based events (e.g. beam becomes top 1 beam and number of measurements is less than configured value)

Some companies proposed having beam-based events such as the changing of the top 1 beam index, and some supported views are that it can keep the balance among data collected under different top 1 beams to maintain the unbiased property of a representative dataset.

However, from the motivation point of view, if we would like to enable data filtering for NW interests or UE resource saving, other events may be more appropriate (e.g., RSRP threshold, area), the change of top 1 beam may not reflect any of the specific NW interests and may not save UE resources since the top 1 beam may change rapidly in some cases.

Furthermore, even for collecting full/representative dataset point of view, the event of top 1 beam change may only be valid when there are no or minor environmental changes under the same top 1 beam, however, it is basically impossible. Therefore, it is suggested that we deprioritize the beam-based events at least for changing of top 1 beam.

Proposal 2 It is suggested that beam-based events are deprioritized to trigger data collection for AI/ML beam management.

L1 beam level measurement

At least from RAN2’s point of view, as an indicator to reflect the RSRP level, L1 beam-based measurements can be considered as the event in parallel with L3-RSRP. However, the L1-RSRP measurements are heavily dependent on UE implementation and RAN1 design, and there can be other designs to realize data filtering instead of L1 event-based data logging, so maybe RAN1 needs a systematic analysis on if L1 beam-based measurements are necessary by considering both pros and cons. So, it is suggested that RAN2 waits for RAN1 to draw further conclusions on it.

Proposal 3 For L1 beam level measurement-based events, further RAN1 conclusions are preferred.

Other events

At least UE buffer status should be considered as event, since data logging cannot be performed if UE buffer is full or almost full. NW configuration is necessary on UE further actions, e.g., report or discard the logged data.

Proposal 4 At least UE buffer status should be considered as the event to start/stop data collection.

Configurations for Data Logging and Reporting

Some remaining issues are listed below:

[RAN2 #127] 1 As the baseline approaches, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. 2 UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases.

First, the interpretations of “configurations” should be clarified. It can be the “measurement configuration” which indicates the UE what to measure based on CSI-RS/SSB, and it can also be the “data logging/reporting configuration” which indicates the UE to start/stop/suspend/resume the data logging/reporting procedure. These two “configurations” should be decoupled during the data collection procedure and UE capability defined in [4].

Concerning the “measurement configuration”, it refers to the set A/set B configuration for beam management, there may be multiple Set A/B combinations for multiple AI/ML-enabled features/FGs/models, from NW’s point of view, it may at least configure UE to perform the measurement for multiple Set A/B in parallel.

Proposal 5 For the measurement configuration of AI/ML data collection, multiple configurations can be supported to allow measurements for multiple Set A/B combinations at UE side.

And for data logging and reporting, there is no need for multiple configurations, NW can configure its own preference to UE for what contents to be captured in the data reporting. NW only needs to dynamically control the procedures of data collection, e.g., start/stop/suspend/resume of the data logging and/or reporting. For data reporting, NW may need to configure the UE to report measured data for different configurations separately, or with explicit/implicit indicator so that NW can distinguish collected data for different AI/ML-enable features/FGs/models at NW side.

Proposal 6 Data reporting for different measurement configurations should be separated and indicated so that NW can distinguish collected data for different AI/ML-enabled features/FGs/models.

As for the activation/deactivation, they are optional for the data collection procedure. UE will start/stop the data collection procedure upon configuration, and multiple events/conditions can be discussed to guarantee the flexibility to dynamically control the data collection procedure. Therefore, additional dynamic activation/deactivation signaling may not be necessary.

Proposal 7 Additional dynamic activation/deactivation may not be necessary since events/conditions can be designed to dynamically control the data collection procedure.

Availability Indicator

The availability indicator has been discussed for two meetings, and the agreements below were given:

[RAN2 #128] The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication. [RAN2 #127Bis] The UE can indicate the availability of logged data to the network to assist network to trigger UEInformationRequest. FFS trigger/definition of availability indication. and FFS how data availability indication is sent to the network.

An availability indicator can be reported prior to the full data report from the UE upon NW request to indicate if the AS buffer is full or not. The triggering of the reporting can be in both proactive and reactive ways under NW configuration, e.g., NW may explicitly send on-demand request to UE since it is NW-side model and UE reports the availability indicator if there is logged data ready for reporting; another option is that UE reports proactively on some events upon UE detection, for example, the AS buffer is full and UE has to stop logging new data.

The mechanism of MDT can be re-used as baseline for the request and report signaling of the availability indicator.

Proposal 8 Status reporting (AS buffer, power etc.) for data collection procedure is supported with the availability indicator from UE to NW under NW control, i.e., the reporting of the availability indicator may be triggered proactively or reactively with conditions configured by NW.

Proposal 9The mechanism of MDT can be re-used as the baseline to report the availability indication.

Assistance Information

To assist the network to categorize the collected data, it is important for the network to be aware of the related assistance information (e.g., additional conditions) when the data is generated and stored. There are UE-side and NW-side additional conditions. To better differentiate the characteristics of data, both additional conditions should be identified.

Observation 1 To categorize the data, it is necessary to consider both UE-side and NW-side assistance information.

In RAN1#116bis meeting, one solution was agreed for UE-side model training with the transmission of NW-sided additional conditions implicitly in term of NW associated ID(s) from network to UE. The AI/ML models will be developed at the UE side based on the collected data corresponding to the NW-sided additional conditions/associated ID(s), or both UE-side and NW-side additional conditions/associated ID(s). The details can refer to the following agreements.

Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation to NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed. - Alt.4: Model ID is determined by pre-defined rule(s) in the specification. - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

Similar principle can be applied to NW-side data collection for model training. With the UE-side and NW-side assistance information (e.g., additional conditions/associated ID(s)), the network will develop the AI model specific for these additional conditions/associated ID(s). One possible solution is that the network can send the NW-side assistance information to the UE. When the UE stores the collected data, it also includes the related NW-side assistance information. Additionally, the UE can also provide the corresponding UE-side assistance information with the collected data. The UE reports the collected data, NW-side assistance information and NW-side assistance information as the integrated data statistics.

Figure 1 UE- based solution

In option 2, the UE can only report UE-side assistance information together with the reported data. When receiving the data from the UE, the network can complement the corresponding NW-side assistance information.

Figure 2 UE- and NW-based solution

In option 3, it is also possible for the UE to provide UE-side assistance information to the network in advance. The UE can report the data without any UE-side or NW-side assistance information. When receiving the data from UE, the network complements both the UE-side and NW-side assistance information.

Fig 3 NW-based solution

It is NW-side data collection, the NW needs to filter the data according to both NW and UE-side assistance information, however, the NW has no prior knowledge of UE-side assistance information, therefore in order to facilitate the aforementioned 3 options, the key aspect needs to be specified is the transmission of UE-side assistance information (e.g., additional conditions) from UE to NW. There are basically two solutions: 1) UE provides NW with the UE-side assistance information (e.g., additional conditions) together with the data collection reporting; 2) The NW sends the UE-side assistance information (e.g., additional conditions) filtering configuration to the UE. Both solutions are reasonable, so we support having both of the two proposals below:

Proposal 10 For NW-side data collection of beam management, UE provides to the NW the UE-side assistance information (e.g., additional conditions) together with the data collection reporting.

Proposal 11 For NW-side data collection of beam management, The NW sends the UE-side assistance information (e.g., additional conditions) filtering configuration to the UE.

Other Issues

Some remaining issues given by agreements are listed below:

[RAN2 #127] - Low priority SRB will be used. FFS new SRB or use of SRB4 - For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE. FFS if other data needs to be collected based on RAN1 progress.

SRB Selection

Considering the requirements of the data reporting for AI/ML model training, it is suggested to use existing SRBs with low priority as baseline, there is no strong motivation to introduce new SRBs to support the AI/ML data collection.

Proposal 12 Existing SRBs with low priority can be taken as the baseline for data reporting.

Data Reporting Parameters

Since L1 measurement result and other data related parameters for data collection are determined by RAN1, it is suggested that RAN2 wait for RAN1 further input at this stage.

Proposal 13 For the detailed parameters of L1 data collection, RAN2 waits for the RAN1 inputs.

Data Collection for Positioning

It has been agreed in RAN1 that the collected data used for training comprises two parts:

Agreement [RAN1 #116bis] For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement [RAN1 #117] For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for the same location associated with Part B. Note: the association can be discussed

We would like to discuss the data collection for NW-side positioning models (case 3a/3b/2b) for the following three aspects:

- Part A

- Part B

- Association between Part A/B

Case 3a

Part A

The input data generation (Part A) is indicated in TR38.843[5]:

From Section 7.1.4 of TR38.843 - The following options of entity to generate other training data (at least measurement corresponding to model input) are identified: - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP

So, no matter which entity is used for case 3a model training, there may not be any RAN2 specification impact for part A data collection:

- Training at gNB: There will be no data transfer among entities, so no specification impact.

- Training at LMF: gNB->LMF data transmission is left for RAN3 discussion.

Observation 2 There may not be RAN2 impact for Part A data collection of case 3a.

Part B

The label data generation (Part B) is given in the following agreement from RAN1:

Agreement [RAN1 #116bis] For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

Again, there seems no RAN2 impacts at all if LMF will be the only entity to generate part B data.

- Training at gNB: LMF->gNB data transmission is left for RAN3 discussion.

- Training at LMF: There will be no data transfer among entities, so no specification impact.

Observation 3 There may not be RAN2 impact for Part B data collection of case 3a.

Association Between Part A/B

There might be some RAN2 impacts for the association between Part A and Part B for case 3a, since these two parts of data may be associated by the same UE/PRU, if model is trained at LMF may need to request some assistance information from UE/PRU to help the LMF identify different UE/PRUs. However, the detail for this part is far beyond current RAN2 discussion progress and cannot be proceeded unless other WGs have more input.

Observation 4 RAN2 may not be able to figure out the impact on association between Part A/B data collection for case 3a, until there is more input from other WGs.

Proposal 14 RAN2 suspends the discussion on training data collection for case 3a and wait for other WGs.

Case 3b/2b

Part A

The input data generation (Part A) is indicated in TR38.843[5]:

From Section 7.1.4 of TR38.843 - The following options of entity to generate other training data (at least measurement corresponding to model input) are identified: - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP

Specifically, for case 3b, gNB reports SRS-based channel measurements to LMF as part A, while for case 2b, UE/PRU reports PRS-based channel measurements to LMF as part A.

In legacy positioning specification, NRPPa framework defined in TS38.455 is used for communication between gNB and LMF, and LPP framework defined in TS37.355 is used for communication between UE/PRU and LMF, thus it is natural to reuse these two frameworks for the collection of part A data for case 3b/2b.

It is noticed that the channel measurement report based on SRS/PRS in NRPPa/ LPP has already been supported for legacy positioning method. In NRPPa, the procedure measurement information transfer is defined as “a procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements.” [6]:

From section 8.5 of TS38.455

Once upon the data collection triggered, LMF can configure gNB to report measurement results, and the reporting contents can be filtered by LMF, measurement quality and time stamp which defined in section 9.1.43 and 9.1.42 respectively can be re-used to report together with the channel measurements in TRP measurement results IE.

In current RAN2 discussion, it is suggested that RAN3 to provide more information on whether the signalling and procedure need to be extended or enhanced for case 3b part A data collection, meanwhile, it is suggested that RAN1 to provide more information on whether the collected contents need to be extended or enhanced, e.g., the type of the channel measurement, the format of the quality indicator and so on.

Proposal 15 To support part A data collection of case 3b, existing NRPPa procedure (e.g., Measurement information transfer) can be re-used.

Proposal 16 For the existing NRPPa procedure of part A data collection of case 3b, it is up to RAN3 on the potential enhancements of signalling and procedure, whilst it is up to RAN1 on the potential enhancements of collected data contents.

Similarly, in LPP, the location information request/provide procedure is defined as the procedure which “is to enable the server to request location measurement data and/or a location estimate from the target, and to enable the target to transfer location measurement data and/or a location estimate to a server in the absence of a request. [7]”:

From section 5.3.1 of TS37.355

Once upon the data collection triggered, LMF can configure UE to report measurement results, and the reporting contents can be filtered by LMF, measurement quality and time stamp which defined in section TS37.355 can be re-used to report together with the channel measurements in ProvideLocationInformation IE.

In current RAN2 discussion, it is suggested that RAN2 to regard the above procedure as baseline to support case 2b part A data collection, some mechanisms discussed for Beam Management may not be applicable to Positioning case 2b, e.g., the logging mechanism is not necessary for gNB, the location information request and provision is quite flexible, LMF can configure the gNB to report the data in a sample-wised way instead of logging. Meanwhile, it is suggested that RAN1 to provide more information on whether the collected contents need to be extended or enhanced, e.g., the type of the channel measurement, the format of the quality indicator and so on.

Proposal 17 To support part A data collection of case 2b, existing LPP procedure (e.g., Location information transfer) can be re-used as baseline.

Proposal 18 For the existing LPP procedure of part A data collection of case 2b, it is up to RAN1 on the potential enhancements of collected data contents.

Part B

The label data generation (Part B) is given in the following contents from TR and working assumptions from RAN1:

Working Assumption [RAN1 #117] For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location. - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Working Assumption [RAN1 #116bis] For training data generation of AI/ML based positioning Case 2b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location. - LMF Note: transfer of label and its related data is out of RAN1 scope. From Section 7.2.4 of TR38.843 For LMF-side model, the LMF is the termination point for training data.

Case 3b and 2b are both direct AI/ML positioning, therefore, their model output (ground truth labels for training) are UE locations, from the above-listed working assumptions, PRU/Non-PRU UE with estimated location/LMF are three common sources to provide part B data for case 3b and 2b.

- PRU: According to the definition in TS38.305, PRU is a special type of UE with known location, the detailed ways of PRU deployment have not been defined in specifications, however, the locations of the PRU will be known to LMF without introducing additional signalling and procedure.

- Non-PRU UE with estimated location: The data privacy has been discussed in RAN1 so the paths to deliver the estimated UE locations to LMF are pending for further study. It may either be transmitted via RAN-transparent way or follow the LPP location information transfer procedure.

- LMF: It refers to LMF obtain the ground truth labels by other means, e.g., GNSS.

Basically, LMF will be in charge of determining the ways of part B data collection of case 3b/2b, it may choose to collect data by stored PRU information, from UE reports or by other implementation ways.

Proposal 19 LMF can choose the appropriate ways to collect part B data for case 3b/2b. One feasible way to collect estimated location of non-PRU UE is via LPP procedures such as location information transfer, details pending for further RAN1 conclusions.

Association Between Part A/B

According to the above agreements, Part A and Part B of one data sample should be associated in the following two ways:

- For a same UE (PRU or Non-PRU UE): LMF can configure the contents of the measurement reporting, in legacy specification, part A (channel measurement) and part B (UE locations) can be reported together in the same reporting IE. Otherwise, LMF has the freedom to match the reported contents by itself, e.g., it can start two parallel LPP sessions towards one UE with two different positioning methods, then encapsulate part A from one session and UE location from another session into one data sample, the decisions on how to associate are up to LMF, and it can give more configurations if it needs UE to report more information to help its association, RAN2 can wait for RAN1 for the details.

- For a same location associated with Part B: LMF may know whether two UE/PRUs are grouped into “same location,” so it may classify the groups by itself. Similar to the above, LMF may give more configurations for UE/PRU to report assistance information (e.g., some indications on the adjacency), again, RAN2 can wait for RAN1 for the details.

Proposal 20 LMF may associate part A and part B data from the same PRU/UE for case 3b/2b by at least the following ways:

- LMF configures UE/PRU to report part A/B together in the same LPP reporting.

- LMF configures UE/PRU to report part A/B separately, but with assistance information to combine two reports. The assistance information may be either from UE reporting, or from LMF locally.

The details are left for further RAN1 input.

Proposal 21 LMF may associate part A for a same location associated with Part B for case 3b/2b by its definition on “same location”. There may be potential assistance information from the UE to help LMF make decisions, but the details are left for further RAN1 input.

Conclusion

In this contribution, we discuss the potential solutions for the NW-side data collection for model training in beam management and positioning. The following observations and proposals are considered:

Beam Management:

Proposal 1 L3 serving cell measurement-based events are used to support data collection for AI/ML beam management, and A1-like event is considered to be the baseline.

Proposal 2 It is suggested that beam-based events are deprioritized to trigger data collection for AI/ML beam management.

Proposal 3 For L1 beam level measurement-based events, further RAN1 conclusions are preferred.

Proposal 4 At least UE buffer status should be considered as the event to start/stop data collection.

Proposal 5 For the measurement configuration of AI/ML data collection, multiple configurations can be supported to allow measurements for multiple Set A/B combinations at UE side.

Proposal 6 Data reporting for different measurement configurations should be separated and indicated so that NW can distinguish collected data for different AI/ML-enabled features/FGs/models.

Proposal 7 Additional dynamic activation/deactivation may not be necessary since events/conditions can be designed to dynamically control the data collection procedure.

Proposal 8 Status reporting (AS buffer, power etc.) for data collection procedure is supported with the availability indicator from UE to NW under NW control, i.e., the reporting of the availability indicator may be triggered proactively or reactively with conditions configured by NW.

Proposal 9 The mechanism of MDT can be re-used as the baseline to report the availability indication.

Observation 1 To categorize the data, it is necessary to consider both UE-side and NW-side assistance information.

Proposal 10 For NW-side data collection of beam management, UE provides to the NW the UE-side assistance information (e.g., additional conditions) together with the data collection reporting.

Proposal 11 For NW-side data collection of beam management, The NW sends the UE-side assistance information (e.g., additional conditions) filtering configuration to the UE.

Proposal 12 Existing SRBs with low priority can be taken as the baseline for data reporting.

Proposal 13 For the detailed parameters of L1 data collection, RAN2 waits for the RAN1 inputs.

Positioning:

Observation 2 There may not be RAN2 impact for Part A data collection of case 3a.

Observation 3 There may not be RAN2 impact for Part B data collection of case 3a.

Observation 4 RAN2 may not be able to figure out the impact on association between Part A/B data collection for case 3a, until there is more input from other WGs.

Proposal 14 RAN2 suspends the discussion on training data collection for case 3a and wait for other WGs.

Proposal 15 To support part A data collection of case 3b, existing NRPPa procedure (e.g., Measurement information transfer) can be re-used.

Proposal 16 For the existing NRPPa procedure of part A data collection of case 3b, it is up to RAN3 on the potential enhancements of signalling and procedure, whilst it is up to RAN1 on the potential enhancements of collected data contents.

Proposal 17 To support part A data collection of case 2b, existing LPP procedure (e.g., Location information transfer) can be re-used as baseline.

Proposal 18 For the existing LPP procedure of part A data collection of case 2b, it is up to RAN1 on the potential enhancements of collected data contents.

Proposal 19 LMF can choose the appropriate ways to collect part B data for case 3b/2b. One feasible way to collect estimated location of non-PRU UE is via LPP procedures such as location information transfer, details pending for further RAN1 conclusions.

Proposal 20 LMF may associate part A and part B data from the same PRU/UE for case 3b/2b by at least the following ways:

- LMF configures UE/PRU to report part A/B together in the same LPP reporting.

- LMF configures UE/PRU to report part A/B separately, but with assistance information to combine two reports. The assistance information may be either from UE reporting, or from LMF locally.

The details are left for further RAN1 input.

Proposal 21 LMF may associate part A for a same location associated with Part B for case 3b/2b by its definition on “same location”. There may be potential assistance information from the UE to help LMF make decisions, but the details are left for further RAN1 input.

Reference

[1] R2_127bis_ChairNotes_EOM, 3GPP TSG-RAN WG2 Meeting #127bis, Hefei, China, Oct 14th – 18th, 2024.

[2] R2_127_ChairNotes_EOM, 3GPP TSG-RAN WG2 Meeting #127, Maastricht, Netherlands, Aug 19th–23rd, 2024.

[3] R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1.

[4] TS38.306.

[5]TR38.843, V2.0.0, 3rd Generation Partnership Project, Technical Specification Group Radio Access Network. Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), Dec 2023.

[6]TS38.455.

[7]TS37.355.

[8]R2_128_ChairNotes_EOM, 3GPP TSG-RAN WG2 Meeting #128, Orlando, USA, Nov. 18th – 22nd , 2024.

[9]R2-24XXXXX [POST128][019][AI PHY] NW side data collection (Nokia)