3GPP TSG-RAN WG2 Meeting #129	R2-2500472

Athens, Greece, Feb. 17th –21st, 2025

Agenda item:	8.2.2

Source: 	Fujitsu

Title: 	Discussions on AIoT Paging

Document for:	Discussion and decision

1. Introduction

According to the objective specified in the New WID [1], RAN2 needs to specify A-IoT Paging to enable DO-DTT and DT data transmission.

The following objectives are set, within the General Scope: - RAN1 scope: … - RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included - RAN3 scope: …

This contribution will provide our considerations on the Ambient IoT Paging, including:

- Paging message content and format, including subsequent paging for the same service

- Paging identifier details

2. Discussion

During Rel-19 SI phase, RAN2 has made some agreements and reported them in [2]. According to the WID, the definitions provided in the TR and decisions made during the Rel-19 SI are taken into this WI. Therefore, the discussions can be started from the decision already made in the TR.

Content and format for A-IoT paging message

In the TR, the following information can be contained in the A-IoT paging message:

- information to identify the device/group of devices, considering:

  - no any identifier

  - an identifier(s) of a single A-IoT device

  - multiple identifier(s) as requested by WID

  - a group ID

- information from which the device(s) can determine the resource(s) to be used for D2R response message(s) and random access type

- information for D2R scheduling (FFS whether MAC signalling is used)

- in case of multiple (subsequent) A-IoT paging message

  - information to avoid duplicated response from the device to a reader

- FFS on the following information:

  - information required for multiple readers case

  - information to indicate service type/command type (inventory/read/write/disable/enable) or to indicate whether there will be subsequent messages

It is observed that both A-IoT upper layer data (e.g. identifier or group ID, service type/command type) and MAC control information (e.g. random access related information) need to be contained in the A-IoT paging message.

Observation 1: A-IoT paging message needs to include both A-IoT upper layer data and MAC control information.

There are two options for the transmission of the A-IoT paging message. For example, a R2D transmission including A-IoT paging message contains both A-IoT upper layer data and MAC control information. Or, the A-IoT paging message involves two R2D transmissions, i.e. one for A-IoT upper layer data and the other for MAC control information. If two R2D transmissions are used for the A-IoT paging message, more complexity could be introduced. For example, how to determine whether the transmission of the A-IoT paging message fails and the multiple/subsequent A-IoT paging message.

Proposal 1: RAN2 to agree that a R2D transmission including A-IoT paging message contains both A-IoT upper layer data and MAC control information.

In NR, DL MAC PDU contains multiple sub-PDUs. Each sub-PDU is comprised of sub-header and corresponding MAC CE/MAC SDU. The fields in sub-header indicate the length and the type of the payload. All sub-PDUs containing MAC SDU will be placed before sub-PDUs containing MAC CE. RAN2 needs to discuss if NR-like format will be used for A-IoT paging message.

Proposal 2: RAN2 to discuss whether NR DL MAC PDU like format will be used for A-IoT paging message.

Paging identifier

In SA solutions, device ID or temporary ID can be used to support A-IoT Paginge. However, according to the WID, temporary identifier is not supported in Rel-19, unless required by SA WGs.

Regarding the identifier of A-IoT device, SA has made some interim conclusions which are captured in the TR [3]. A-IoT device is configured with a permanent Ambient IoT Device Identifier which can be assigned by an operator or by a third party. In Part1 information, as defined, ID type is included to indicate whether the identifier is assigned by an operator or by a third party. Part2 information is used to distinguish different A-IoT devices within the scope identified by the Part2 information, alought the uniqueness of Identifier is FFS.

Figure 8.2.1-1: The Operator allocated ID [3]

Figure 8.2.1-2 The third party allocated ID [3]

For the identifier of A-IoT device, there are some remaining issues in SA groups. For RAN2, we can wait for SA discussion and have no need to discuss the details of the identifier.

Proposal 3: RAN2 to wait SA discussions on the details of the paging identifier.

3. Conclusion

In this contribution, we had some discussions on the Ambient IoT Paging and made the following observation and proposals:

Content and format for A-IoT paging message

Observation 1: A-IoT paging message needs to include both A-IoT upper layer data and MAC control information.

Proposal 1: RAN2 to agree that a R2D transmission including A-IoT paging message contains both A-IoT upper layer data and MAC control information.

Proposal 2: RAN2 to discuss whether NR DL MAC PDU like format will be used for A-IoT paging message.

Paging identifier

Proposal 3: RAN2 to wait SA discussions on the details of the paging identifier.

Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#106, December 9-12, 2024

- 3GPP TR38.769, “Study on solutions for ambient IoT (Internet of Things)”

- 3GPP TR23700-13, “Study on Architecture support of Ambient power-enabled Internet of Things”, v1.2.1