3GPP TSG-RAN WG2 Meeting #129	R2-2500475

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.5.4

Source: 	Fujitsu

Title: 	Adaptation of common signal or channel

Document for:	Discussion and decision

Introduction

Rel-19 NES WID was updated and approved in RANP#105 [1]. The WID describes the following objective on the adaptation of common signal/channel transmissions.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we would like to discuss and provide our views on the adaptation of common signal/channel transmissions.

Discussion

Adaptation of SSB in time domain

Regarding RAN4 LS [4], the reply to RAN4 questions are concluded in the last RAN1 meeting.

Agreement@RAN1#119 Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Agreement@RAN1#119 Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Agreement@RAN1#119 Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Further update to be made based on RAN1#119 progress if any.

It has been agreed that the time location other than the periodicity, the frequency location and the spatial relation are the same before and after SSB adaptation. It is observed that it is only allowed to adapt the periodicity of SSB.

Regarding the applicable scenarios of SSB adaptation, the following conclusions are reached in the previous RAN1 meetings.

Conclusion(RAN1#118b) There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Conclusion(RAN1#119) There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

It has been concluded that SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell and Rel-19 idle mode UEs cannot be supported. Which means SSB adaptation can be supported only in SCell. Thus, SSB adaptation and on-demand SSB in SCell may be considered together.

Observation 1: SSB adaptation is supported only in SCell. SSB adaptation and on-demand SSB in SCell may be considered together.

For indication of SSB adaptation, the following is reached in the previous RAN1 meeting.

Agreement@RAN1#116b For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger

It has been agreed to support MAC CE signalling for on-demand SSB indication. Which means on-demand SSB with multiple candidate periodicities can be configured in RRC, and one out of periodicities can be indicated via MAC CE. Therefore, the adaptation of on-demand SSB periodicity can be realized by on-demand indication MAC CE.

If SSB adaptation and on-demand SSB operation on SCell are considered together, RAN2 may discuss whether the MAC CE of on-demand SSB indication can be utilized to adapt the SSB periodicity in AI 9.5.3. From this perspective, MAC CE-based SSB adaptation can be supported.

Proposal 1: RAN2 to consider the adaptation of SSB periodicity via on-demand SSB indication MAC-CE.

Adaptation of PRACH in time domain

In RAN1#116-bis meeting, there are the following agreements on the adaptation of PRACH in time domain [2].

Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement For adaptation of PRACH in time-domain, support the following: - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any) - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources - FFS: SSB-RO mapping for the additional PRACH resources Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode Agreement For adaptation of PRACH in spatial domain, - Study possibility of scenarios with non-uniform distribution of UEs in different beams - Note 6: Companies are encouraged to provide details on how they map UEs to different beams - Study network energy savings gain achieved by non-uniform PRACH resource allocation across SSBs for scenarios with non-uniform distribution of UEs in different beams (if any), - Assume the following framework for network energy evaluation in FR1 and companies to report at least the below settings used in the evaluation/simulation - 20ms SSB period - 30kHz SCS, DDDSU TDD pattern - Setting A: SIB1 period (20ms/40ms/160ms) - Setting B1: Cell load (Empty/low/medium) - Setting B2: Traffic model - Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2 - Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11 - Setting E1: PRACH configurations - (legacy) PRACH resources according to the following PRACH configuration for all transmitted SSBs - Case A1-1: PRACH configuration #5 (20ms) - Case A1-2: PRACH configuration #17 (10ms) - Case A2-1: PRACH configuration #0 (160ms) - (time-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration for all transmitted SSBs - Case B1: PRACH configuration #17 (10ms) - Case B2: PRACH configuration #0 (160ms) - Companies to report details of assumed time domain adaptation mechanism - (spatial-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration - Case C1: PRACH configuration #17 (10ms) - Case C2: PRACH configuration #0 (160ms) - Companies to report details of assumed spatial domain adaptation mechanism, including details of non-uniform PRACH resource allocation across SSBs - Setting F: Cat 1/Cat 2 BS as defined in TR38.864 - Setting G1: Number of SSB beams: 4,8 SSBs in a SSB burst with SSB pattern case C - Note 1: Baseline to compare is Case C1 vs Case B1/A1-1/A1-2, Case C2 vs Case B2/A2-1 - Note 2: It is up to company to report the SSB-RO mapping ratio and FDMed RO number, etc - Note 3: Other PRACH configuration index with different PRACH format other than format 0 is not precluded - Note 4: Other SSB/SIB1/RACH periodicity/PRACH resource/configuration assumptions are not precluded (up to companies to report) Other frameworks for network energy evaluation are not precluded, e.g. including for FR2

As shown above, the adaptation mechanism of additional PRACH resources for NES-capable UEs are agreed. NES-capable UEs can use both additional PRACH resources and legacy PRACH resources for legacy UEs. In high load or medium load case, the legacy and additional PRACH resources are both active, and in case of low load and low number of UE accesses, additional PRACH resources are deactivated to achieve energy saving gain.

For random access in additional PRACH resource, before discussing additional PRACH resource configuration/indication, the random access type should be discussed first.

For 4-step and 2-step random access, there are two possible options. Option 1 is to support both 4-step and 2-step random access for additional PRACH resources, and option 2 is to support 4-step random access first and deprioritize 2-step random access. 2-step random access is an optional function, and it can be applied only in good RSRP, and it may cause larger signaling overhead, such as PUSCH configuration. Therefore, Option1 has full flexibility and no restriction for random access while Option 2 has a small signaling overhead and specification impacts.

Proposal 2: RAN2 to discuss whether the following types of random access should be supported for additional PRACH resource for NES UEs.

- Option1: Support both 4-step RA and 2-step RA.

- Option2: Support only 4-step RA.

For CBRA and CFRA, there are also two possible options. Option 1 is to support both CBRA and CFRA, and Option 2 is to support only CBRA. CBRA can be applied in both in RRC_IDLE/RRC_INACTIVE mode and RRC_CONNECTED mode, however CFRA can only be applied in RRC_CONNECTED mode. PRACH adaptation for CBRA may achieve more energy saving gain than PRACH adaptation for CFRA. Furthermore, multiple PRACH configuration signalling is used for CFRA which is more complicated than CBRA. Thus, supporting only CBRA can be a straightforward approach with larger energy saving gain and minimum specification effort. However, NR supports both CBRA and CFRA procedure, and the normative work to support random access in additional PRACH resources should cover CBRA and CFRA. Therefore, the two options are provided for further discussion where option 1 has more flexibility and no restriction while Option 2 has less spec impact.

Proposal 3: RAN2 to discuss whether the following contention types should be supported for additional PRACH resource for NES UEs.

- Option1: Support both CBRA and CFRA.

- Option2: Support only CBRA.

Adaptation of paging occasions

In RAN2#127bis meeting, there were following agreements on paging adaptation [3]:

Agreements on paging adaptation

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

In RAN2#128 meeting, the following agreements are made [5]:

Agreements on paging adaptation

- Introduce value for Ns=8. FFS on 16.

- Introduce value for N= T/32. FFS on T/64, T/128, T/256.

In addition, RAN2 will revisit PEI configuration and PEI transmission aspects with the consideration of offline discussion in the next meeting.

In the following, we would like to discuss paging configuration for the Rel-19 NES UE.

Paging configuration for the NES UE

In RAN2#127bis meeting, it is agreed that legacy UE is not barred in the NES cell supporting paging adaptation and Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration. From our perspective, in addition to the existing PCCH-Config IE in SIB1 for legacy UEs, the Rel-19 paging configuration for the Rel-19 NES UEs should be introduced, and it can be transmitted in SIB1 or a new SIB in the NES cell. Based on the offline discussions in the last RAN2 meeting, the RRC signalling overhead of the Rel-19 paging configuration of Option-b) could be hundreds of bits. Since the discussions on how to minimize the RRC signalling overhead of Option-b) is not finalized, we can decide whether to use SIB1 or a new SIB later.

Proposal 4: The Rel-19 paging configuration is transmitted in SIB1 or a new SIB in the NES cell.

The next issue to be discussed is the content of the Rel-19 paging configuration. The existing PCCH-Config IE in TS 38.331 is shown as follows:

PCCH-Config ::=             SEQUENCE {

defaultPagingCycle                  PagingCycle,

nAndPagingFrameOffset               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15)

},

ns                                  ENUMERATED {four, two, one},

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

...,

[[

nrofPDCCH-MonitoringOccasionPerSSB-InPO-r16        INTEGER (2..4)               OPTIONAL  -- Cond SharedSpectrum2

]],

[[

ranPagingInIdlePO-r17                              ENUMERATED {true}            OPTIONAL,  -- Need R

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                                  OPTIONAL  -- Need R

]]

}

For Rel-19 paging configuration, at least nAndPagingFrameOffset, ns and firstPDCCH-MonitoringOccasionOfPO are needed to configure new N values and Ns values for paging adaptation. As it is required in the WID that there shall be no paging latency increase, the paging DRX cycle should be same as legacy, so the defaultPagingCycle parameter is not needed in the Rel-19 paging configuration in our view. For the other parameters, RAN2 can further discuss whether they are needed in Rel-19 paging configuration or not.

Proposal 5: In Rel-19 paging configuration, at least nAndPagingFrameOffset, ns and firstPDCCH-MonitoringOccasionOfPO are needed.

Paging transmission in POs

Paging allows the network to reach UEs in RRC_IDLE and in RRC_INACTIVE state through paging messages, and to notify UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state of system information change and ETWS/CMAS indications through short messages.

In our opinion, the NES cell should transmit short messages in both legacy POs (POs according to legacy PCCH-Config) and Rel-19 NES POs (POs according to Rel-19 NES paging configuration), so that both legacy UEs and Rel-19 NES UEs can receive short messages.

Proposal 6: Short messages are transmitted in both legacy POs and Rel-19 NES POs.

On the other hand, for the transmission of the paging messages, the gNB may need to know whether a CN paging message is for a legacy UE(s) or not, as a legacy UE cannot monitor the Rel-19 NES POs and the network can only transmit the paging message for the legacy UE in the legacy PO. Therefore, CN may need to notify the gNB whether the paged UE(s) in the CN paging message is a legacy UE or a Rel-19 NES UE. This may have some impact on RAN3 and an LS can be sent to RAN3 to trigger their discussions on the notification if necessary.

Observation 2: For the transmission of the paging messages, the gNB needs to know whether a CN paging message is for a legacy UE(s) or not.

Proposal 7: RAN2 to discuss how to provide the gNB with information on whether the paged UEs are legacy UEs or Rel-19 NES UEs.

In order for CN to know whether the paged UE is a legacy UE or a Rel-19 NES UE, a UE capability on paging adaptation can be transmitted to CN. This can be done by the following options:

- Option 1: The Rel-19 NES UE indicates its capability on paging adaptation via a NAS message to CN.

- Option 2: The Rel-19 NES UE indicates its capability on paging adaptation via the AS UE capability information message to gNB and then gNB can indicate the UE capability to CN.

If option 1 is adopted, RAN2 should send an LS to SA2/CT1/RAN3 for the NAS and NGAP signalling. Otherwise, if option 2 is adopted, UE can indicate the capability in UECapabilityInformation message as a starting point.

Proposal 8: RAN2 to discuss how to provide the NW (RAN or CN) with the paging adaptation capability information on whether the paged UEs are legacy UEs or Rel-19 NES UEs.

Conclusion

In this contribution, we have some discussions about the adaptation of common signals or channels, and the following proposals are made:

Observation 1: SSB adaptation can be supported only in SCell. Thus, SSB adaptation and on-demand SSB in SCell may be considered together.

Proposal 1: RAN2 to support MAC CE-based SSB adaptation for fast SSB adaptation.

Proposal 2: RAN2 to discuss whether the following types of random access should be supported for additional PRACH resource for NES UEs.

- Option1: Support both 4-step RA and 2-step RA.

- Option2: Support only 4-step RA.

Proposal 3: RAN2 to discuss whether the following contention types should be supported for additional PRACH resource for NES UEs.

- Option1: Support both CBRA and CFRA.

- Option2: Support only CBRA.

Proposal 4: The Rel-19 paging configuration is transmitted in SIB1 or a new SIB in the NES cell.

Proposal 5: In Rel-19 paging configuration, at least nAndPagingFrameOffset, ns and firstPDCCH-MonitoringOccasionOfPO are needed.

Proposal 6: Short messages are transmitted in both legacy POs and Rel-19 NES POs.

Observation 2: For the transmission of the paging messages, the gNB needs to know whether a CN paging message is for a legacy UE(s) or not.

Proposal 7: RAN2 to discuss how to provide the gNB with information on whether the paged UEs are legacy UEs or Rel-19 NES UEs.

Proposal 8: RAN2 to discuss how to provide the NW (RAN or CN) with the paging adaptation capability information on whether the paged UEs are legacy UEs or Rel-19 NES UEs.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson, RAN#105 meeting.

- RAN1#116bis chair’s note

- RAN2#127bis NES session chair's note

- R2-2409521_R4-2416913 LS on SSB relation in On-demand SSB and SSB adaptation on SCell

- RAN2#128 NES session chair's note