3GPP TSG-RAN WG2 Meeting #129	R2-2500480

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.7.4.2

Source: 	Fujitsu

Title: 	Discussions on DSR enhancements

Document for:	Discussion and decision

Introduction

On DSR retransmission enhancement in XR, RAN2 has made the following agreements in RAN2#127bis meeting [1]:

Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In RAN2#128 meeting, there are following agreements [2]:

Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In this contribution, we would like to discuss the format of the enhanced DSR MAC CE and the coexistence with legacy DSR.

Discussion

Enhanced DSR MAC CE format

In RAN2#127bis meeting, RAN2 agrees that a one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. In our opinion, to be simple, the format of the legacy DSR MAC CE can be reused with the R bit used as the one-bit indication. For example, the format of the enhanced MAC CE is shown in Figure 1:

Figure 1: Format of the enhanced DSR MAC CE

In figure 1, the E bit is used to indicate whether the further pair of remaining time information and buffer size information present in the enhanced DSR MAC CE is for the same LCG. For example, if the value of E bit is 0, the further pair of remaining time information and buffer size information present in the enhanced DSR MAC CE is for the same LCG; otherwise, if the value of E bit is 1, the further pair of remaining time information and buffer size information present in the enhanced DSR MAC CE is for the next reported LCG with the value of LCGi field set to 1.

Proposal 1: The format of the enhanced DSR MAC CE as shown in Figure 1 is adopted.

Another issue that can be discussed is the order of the multiple pairs of remaining time information and buffer size information present for one LCG. In our opinion, the multiple pairs can be present in ascending order of the reporting thresholds of the remaining time, which is very straightforward.

Proposal 2: For a reported LCG, the multiple pairs of remaining time information and buffer size information present are present in ascending order of the reporting thresholds.

Legacy DSR and enhanced DSR coexistence

In the last RAN2 meeting, there is an FFS issue on whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds.

In our understanding, we can always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds. For the LCG configured with multiple reporting thresholds but with only one pair of remaining time and buffer size be reported, or for the LCG configured with only one reporting threshold, the enhanced DSR format can include just one pair of remaining time information and buffer size information for this LCG. This is much simpler compared with determining whether legacy DSR or enhanced DSR to be reported based on buffer size or number of reporting threshold(s) of each LCG.

Observation 1: For the LCG configured with multiple reporting thresholds but with only one pair of remaining time and buffer size to be reported, or for the LCG configured with only one reporting threshold, the enhanced DSR format can include just one pair of remaining time information and buffer size information for this LCG.

Proposal 3: Enhanced DSR is always used in case there is at least one LCG configured with multiple reporting thresholds.

Conclusion

In this contribution, we have discussed the further details of the DSR enhancements. We have the following proposals:

Proposal 1: The format of the enhanced DSR MAC CE as shown in Figure 1 is adopted.

Proposal 2: For a reported LCG, the multiple pairs of remaining time information and buffer size information present are present in ascending order of the reporting thresholds.

Observation 1: For the LCG configured with multiple reporting thresholds but with only one pair of remaining time and buffer size to be reported, or for the LCG configured with only one reporting threshold, the enhanced DSR format can include just one pair of remaining time information and buffer size information for this LCG.

Proposal 3: Enhanced DSR is always used in case there is at least one LCG configured with multiple reporting thresholds.

References

- R2-2409215, Report from session on R18 MBS, R18 QoE and R19 XR,	Session chair (Huawei), RAN2#127bis meeting

- R2-2410915, Report from session on R18 MBS, R18 QoE and R19 XR,	Session chair (Huawei), RAN2#128 meeting