3GPP TSG-RAN WG2 Meeting #129	R2-2500481

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.7.5

Source: 	Fujitsu

Title: 	Discussions on RLC enhancements

Document for:	Discussion and decision

Introduction

On RLC retransmission enhancement in XR, there are following agreements in RAN2#128 meeting [1] for avoiding unnecessary retransmissions:

Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced Special handling to avoid PDCP control PDU discard is not needed. A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

For ensuring timely RLC retransmissions, there is following agreement:

Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

In this contribution, we would like to discuss the details on timely RLC retransmissions and avoidance of unnecessary RLC retransmissions for XR.

Discussion

Ensuring timely RLC retransmission(s) for XR

As agreed by RAN2, both autonomous retransmission and polling enhancements can be used to ensure timely RLC retransmission.

In autonomous retransmission, the RLC Tx side can perform RLC retransmissions in the consecutive transmission occasions without waiting for the status reports from the RLC Rx side, so the packet delay can be reduced and high reliability for XR traffics in case of small PDB can be achieved. In order to have less impact on the system capacity, autonomous retransmission can be applied in certain cases to save the radio resource occupation. For example, the autonomous retransmission can be triggered by the UE itself when the remaining time of the data becomes lower than a threshold. Alternatively, the autonomous retransmission can be performed according to the network request. In this option, the network can configure for which DRBs the autonomous retransmission is applicable by RRC signalling. The network can also activate/deactivate the autonomous retransmission by MAC CE based on e.g., the impact on system capacity or the situation of uplink congestion.

When the PDB of the XR traffic is reached, the autonomous retransmission in the corresponding RLC entity should be stopped to avoid the unnecessary retransmissions.

Proposal 1a: Autonomous retransmission can be triggered when the remaining time of the data becomes lower than a threshold and can be stopped when PDB of the data is reached.

Proposal 1b: Autonomous retransmission can be configured or activated/deactivated by the network.

The autonomous RLC retransmission(s) can occur in the consecutive slots or can have an interval. If positive acknowledgement is received by the transmitting side of the AM RLC entity, these autonomous RLC retransmission(s) can be stopped. If negative acknowledgement is received by the transmitting side of the AM RLC entity, the transmitting side of the AM RLC entity can continue the autonomous RLC retransmissions.

The number of the autonomous RLC retransmissions can be configured by the network or can be pre-defined in RLC. In our opinion, these autonomous RLC retransmissions should not be counted into RETX_COUNT, so that RETX_COUNT would not easily reach the maximum retransmission number (maxRetxThreshold) and RLF would not be initiated.

Proposal 2: The number of the autonomous RLC retransmissions can be configured by the network or can be pre-defined in RLC.

Proposal 3: The autonomous RLC retransmissions are not counted into RETX_COUNT.

In enhanced polling, the specification impact can be reduced with introducing small values of pollPDU or pollByte. Currently, the minimum value of pollPDU is 4 and pollBye is 1KB. For XR, the minimum value of pollPDU can be 2 and pollByte can be 0.5KB. Thus, timely status report can be received by the TX side of the AM RLC entity with the more frequent polling. In order to save the overhead of polling and status reports, the enhanced polling option can also be triggered in the UE when the remaining time of the data becomes lower than a threshold or when configured by the network via RRC signalling.

Proposal 4a: Enhanced polling can be triggered when the remaining time of the data becomes lower than a threshold.

Proposal 4b: Enhanced polling can be configured by the network.

Avoiding unnecessary retransmissions

For avoiding unnecessary RLC retransmissions, a “combined” approach has been agreed by RAN2 in RAN2#127 meeting, where Tx side stops transmissions of an outdated SDU and Rx side abandons the SDU based on a local timer. In addition, Rx side informs Tx side about the abandoned SDUs and RAN2 assume existing status report can be reused as a baseline.

The first issue that can be discussed is when the Tx side stops transmissions of an outdated SDU. In our assumption, RLC Tx side can stop retransmissions of an RLC SDU e.g., upon receiving PDCP discard indication about the discarded RLC SDU, or when the number of transmissions reaches a number configured by the network based on the PDB, or when receiving the status report from RLC Rx side indicating that the RLC SDU is ACKed. From our perspective, each option can work, so we have no strong preference.

Proposal 5: In the combined approach, Tx side stops retransmissions of an RLC SDU upon receiving PDCP discard indication or when the number of transmissions reaches a number configured by the network based on the PDB or when receiving the status report.

The next issue is how to determine the local timer at the Rx side. For the uplink XR service, the Rx side is at the network side and the local timer need not to be specified in the standard. On the other hand, for the downlink XR service, the Rx side is at the UE side, so the definition of the local timer and the related UE behaviours should be specified. In our understanding, the local timer should be configured by the network based on the PDB of the XR service, like the PDCP discard timer. How to configure the value of the local timer may depend on the network implementation. For an RLC SDU the associated local timer can be started when another RLC SDU with a higher SN is received in the Rx side. If the RLC SDU is received, the local timer associated with it can be stopped if running. When the local timer expires, Rx side can consider the associated RLC SDU as abandoned.

Proposal 6: The local timer at the Rx side (UE) is configured by the network.

Conclusion

In this contribution, we have discussed the RLC enhancements to achieve timely RLC retransmission and to avoid unnecessary retransmissions. We have the following proposals:

Proposal 1a: Autonomous retransmission can be triggered when the remaining time of the data becomes lower than a threshold and can be stopped when PDB of the data is reached.

Proposal 1b: Autonomous retransmission can be configured or activated/deactivated by the network.

Proposal 2: The number of the autonomous RLC retransmissions can be configured by the network or can be pre-defined in RLC.

Proposal 3: The autonomous RLC retransmissions are not counted into RETX_COUNT.

Proposal 4a: Enhanced polling can be triggered when the remaining time of the data becomes lower than a threshold.

Proposal 4b: Enhanced polling can be configured by the network.

Proposal 5: In the combined approach, Tx side stops retransmissions of an RLC SDU upon receiving PDCP discard indication or when the number of transmissions reaches a number configured by the network based on the PDB or when receiving the status report.

Proposal 6: The local timer at the Rx side (UE) is configured by the network.

References

- R2-2410915, Report from session on R18 MBS, R18 QoE and R19 XR,	Session chair (Huawei), RAN2#128 meeting