3GPP TSG-RAN WG2 Meeting #129	R2-2500482

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.7.6

Source: 	Fujitsu

Title: 	Discussions on XR rate control

Document for:	Discussion and decision

Introduction

On XR rate control, there are following agreements in RAN2#128 meeting [1]:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

In this contribution, we would like to discuss the details of the rate control MAC CE from gNB to UE, the potential format of the rate control MAC CE and whether to support the rate query MAC CE.

Discussion

Per QoS flow or per DRB rate control

In RAN2#128 meeting, there are two options for rate indication from the gNB to the UE on a per QoS flow level as following:

Option 1) flows are indicated by MAC CE.

Option 2) by RRC while MAC CE is per DRB.

In Option 1), the MAC CE from gNB to UE would indicate the rate control information for one or more QoS flows, and QFI related information of the QoS flows should be included in the MAC CE. At the gNB side, the QFI related information may need to be transmitted from CU to DU in CU-DU split scenario, and some impact on F1AP is foreseen. At the UE side, upon receiving the rate control MAC CE, MAC layer may need to indicate the QFI related information to SDAP layer or higher layer, since PDCP, RLC and MAC cannot see QoS flows. In our opinion, Option 1) may need more inter-layer interactions between MAC and SDAP/higher layers, which may have some specification impact or increase UE complexity. In addition, Option 1) has more overhead, since the QFI related information included in MAC CE may include QFI and PDU session ID for each indicated QoS flow.

Observation 1: MAC CE indicating QoS flows may have a higher overhead and impact on F1AP and AS layers.

In Option 2), the MAC CE from gNB to UE would indicate the rate control information for one or more DRBs, and DRB related information should be included in the MAC CE. Since DRB related information is already known at gNB-DU and MAC layer, Option 2) requires less specification impact and is simpler than Option 1), like the PDCP duplication activation/deactivation MAC CE. From our perspective, Option 2) is preferred.

Proposal 1: Rate control MAC CE from the gNB to the UE indicates rate control information per DRB and QoS flows are indicated via RRC as in legacy.

Potential rate control MAC CE format

The format of the current Recommended bit rate MAC CE is shown in the following figure. It has a fixed size indicating the recommended bit rate for only one logical channel. In this MAC CE, LCID, UL/DL, Bit rate and X bits are included.

Figure 6.1.3.20-1: Recommended bit rate MAC CE

To save the signalling overhead of indicating rate control information for multiple DRBs or QoS flows (depending on which option will be adopted by RAN2) by multiple MAC CEs where each MAC CE indicates rate control information for a single DRB/QoS flow, the rate control MAC CE can indicate the rate control information of multiple DRBs/QoS flows at a time. To be simple, a bitmap can be used at the beginning of the MAC CE, indicating the index(indices) of the DRBs/QoS flows for which the rate control information is included in the MAC CE. For each indicated DRB or QoS flow, the bit rate needs to be indicated.

In the SA4 LS [2], SA4 reply is as follows:

The range varies from 100kbps to 40Mbps. The numbers are based on the SA4 traffic characteristics study for XR services documented in 3GPP TR 26.926. Specifically, for XR split rendering and cloud gaming, a typical uplink data rate is 200 kbps. For AR conversational, a typical uplink data rate is 20Mbps. The granularity varies among existing data rate adaptation algorithms, and it can be as small as 1% for some algorithms (e.g., transport-wide congestion control as documented in 3GPP TR 26.822), yet also coarser for other algorithms (e.g., video codecs that can adapt to a given target bit rate constraint). From an application perspective, finer granularity, e.g., 1%, is generally preferred, if possible, but coarser granularity may also be sufficient, e.g., 10%.

In this LS, SA4 replies that the range of bit rate varies from 100kbps to 40Mbps and finer granularity, e.g., 1%, is generally preferred but coarser granularity may also be sufficient, e.g., 10%.

If the granularity is 1%, we suppose that 604 values are needed to indicate the bit rates from 100kbps to 40Mbps and the length of the bit rate field would be 10 bits. If coarser granularity can be used, 256 values may be enough to indicate the bit rates in this range and the length of the bit rate field can be reduced to 8 bits. If the direction (UL or DL), the bitRateMultiplier and other information also need to be indicated in the MAC CE, the rate control information for each DRB/QoS flow would be 2-byte long. An example of the potential MAC CE format is shown in Figure 1 where the length of the bit rate field is 8 bits. The length of each field in the MAC CE can be determined based on further RAN2 discussions.

Figure 1 Potential format of rate control MAC CE

Proposal 2: Rate control information is indicated by a new MAC CE (e.g., rate control MAC CE).

Proposal 3: The rate control MAC CE indicates the rate control information of multiple DRBs/QoS flows.

UL rate query MAC CE

In RAN2#128 meeting, there is an FFS on whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

In our opinion, UL MAC CE for rate query is useful in end-to-end rate control scenario like the legacy UL Recommended bit rate query MAC CE. On receiving the rate control MAC CE from the gNB, the UE can inform its peer UE of the end-to-end bit rate. Then the peer UE can send the UL MAC CE for rate query to its local gNB to ask if the bit rate can be provided. If the peer UE’s local gNB does not detect congestion, the queried bit rate can be provided. Otherwise, if the peer UE’s local gNB also detects congestion, the queried bit rate might not be provided and the local gNB can indicate the rate control information to the peer UE. Therefore, we propose to support the UL MAC CE for rate query/preference from the UE to the gNB.

Proposal 4: ULMAC CE for rate query/preference is supported.

Similar to Recommended bit rate query MAC CE, the UL rate query MAC CE from UE to NW can have the same format as the rate control MAC CE from NW to UE discussed in Proposal 2 and Proposal 3. So, we propose not to reuse the legacy Recommended bit rate query MAC CE, but to introduce a new UL MAC CE to serve this purpose.

Proposal 5: A new UL MAC CE is introduced for enhanced UL rate query for XR rate control.

Conclusion

In this contribution, we have discussed the details of the rate control MAC CE from gNB to UE, the potential format of the rate control MAC CE and whether to support the rate query MAC CE. We have the following proposals:

Observation 1: MAC CE indicating QoS flows may have higher overhead and impact on F1AP and AS layers.

Proposal 1: Rate control MAC CE from the gNB to the UE indicates rate control information per DRB and QoS flows are indicated via RRC as in legacy.

Proposal 2: Rate control information is indicated by a new MAC CE (e.g., rate control MAC CE).

Proposal 3: The rate control MAC CE indicates the rate control information of multiple DRBs/QoS flows.

Proposal 4: UL MAC CE for rate query/preference is supported.

Proposal 5: A new UL MAC CE is introduced for enhanced UL rate query for XR rate control.

References

- R2-2410915, Report from session on R18 MBS, R18 QoE and R19 XR,	Session chair (Huawei), RAN2#128 meeting

- R2-2500056, Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications, SA4. 2