3GPP TSG-RAN WG2 Meeting #129	R2-2500490

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.7.5

Source:	Nokia, Nokia Shanghai Bell

Title:	RLC Enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#128:

Agreements on AL-FEC (related to LS from SA2) - There is no consensus in RAN2 that AL-FEC ratio information is useful for the gNB for both RLC AM and RLC UM. - RAN2 understanding is that in case this information would be provided to the gNB, it is up to gNB how/whether to consider it, i.e. no impact on RAN2 specifications Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced - Special handling to avoid PDCP control PDU discard is not needed. - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report. Agreements on Timely RLC retransmissions - Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

# 2	Unnecessary RLC retransmissions avoidance

Related to agreement #5 above, received RLC ACKs are also used locally by PDCP, for instance to determine how high-numbered PDCP SDUs can be transmitted by PDCP:

NOTE 1: Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

Indeed, the PDCP specification also contains the following.

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows: - if RCVD_SN < SN(RX_DELIV) – Window_Size: - RCVD_HFN = HFN(RX_DELIV) + 1. - else if RCVD_SN >= SN(RX_DELIV) + Window_Size: - RCVD_HFN = HFN(RX_DELIV) – 1. - else: - RCVD_HFN = HFN(RX_DELIV); - RCVD_COUNT = [RCVD_HFN, RCVD_SN]. After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall: - perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT; - if integrity verification fails: - indicate the integrity verification failure to upper layer; - discard the PDCP Data PDU and consider it as not received; - if RCVD_COUNT < RX_DELIV; or - if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before: - discard the PDCP Data PDU;

That is, a received PDCP SN that is more than Window_Size (equal to half of the PDCP SN space) ahead of SN(RX_DELIV) is misinterpreted as representing a COUNT value below RX_DELIV, resulting in discarding the PDCP Data PDU with the SN. In the current specification, the PDCP transmitter is expected to take this into account by maintaining a local estimate of RX_DELIV at the peer entity, obtained as the lowest COUNT value not yet positively acknowledged. In absence of a PDCP Status report, these acknowledgements come in the form of RLC ACKs.

Observation 1: a received PDCP SN that is more than half of the PDCP SN space ahead of SN(RX_DELIV) is misinterpreted as representing a COUNT value below RX_DELIV, resulting in discarding the PDCP Data PDU with the SN.

Observation 2: to determine how high-numbered PDCP SDUs can be transmitted, the PDCP transmitter currently maintains a local estimate (lower bound) of RX_DELIV at the peer entity, which in absence of a PDCP Status report is obtained as the lowest COUNT value not yet RLC-ACKed.

By agreement #4 above, the new RLC timer can be started by a received SDU segment not yet delivered to PDCP. It follows that for an unreceived RLC SDU, the new RLC timer may start and expire earlier than PDCP’s t-Reordering, and consequently that the unreceived SDU may be RLC-ACKed before RX_DELIV at PDCP advances past that SDU. This means that RLC ACKs no longer allow the PDCP transmitter to maintain a reliable lower bound of RX_DELIV at the peer entity, also making it unable to determine how high-numbered PDCP SDUs can be transmitted.

Observation 3: accounting for possible RLC SDU segments, for an unreceived RLC SDU, the new RLC timer may expire earlier than PDCP’s t-Reordering, and consequently the unreceived SDU may be RLC-ACKed before RX_DELIV at PDCP advances past that SDU.

Observation 4: RLC ACKs no longer allow the PDCP transmitter to maintain a reliable lower bound of RX_DELIV at the peer entity, also making it unable to determine how high-numbered PDCP SDUs can be transmitted.

As a replacement, regular Status reporting would seem to be needed at PDCP.

Proposal 1: to allow the transmitting PDCP to reliably know how high-numbered PDCP SDUs it can proceed to transmit, the agreed RLC ACK no longer indicating successful delivery is combined with regular PDCP status reporting.

RAN2#127bis agreed:

- RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer

Related to the TX aspect, the following open issue was captured in the running CR:

Editor’s Note: FFS how to determine an outdated SDU for Tx side to stop transmission, e.g. based on upper layer or other mechanism.

We think the current standard already clearly defines when an SDU is outdated: when PDCP’s discardTimer (or discardTimerForLowImportance) expires. Moreover, when transmissions of an outdated SDU stop, there is no more use for the SDU and it can be discarded.

Proposal 2: the previously agreed stopping of transmissions of an outdated SDU is captured in the RLC specification in the form of unconditional discarding upon a discard indication from PDCP, when the RLC entity is configured with the combined approach for avoiding unnecessary retransmissions.

# 3	Timely RLC retransmissions

## 3.1	Autonomous retransmissions

Since autonomous retransmissions do not rely on explicit knowledge of failed RLC SDUs (and are not triggered in case of timely RLC ACKs), new conditions should be defined to trigger such retransmissions. In the following we list some of the options that have been proposed during previous meetings:

1.	Timer-based triggers: the TX RLC will start retransmitting the RLC SDUs when a timer associated with the corresponding SDU falls below a preconfigured threshold. Three possible alternatives:

a.	Relying on the existing PDCP indication of the RLC SDU becoming delay-critical. This option is coupled with DSR being configured which creates unnecessary dependencies between the features.

b.	Relying on the remaining time of a SDU based on PDCP discard timer. This option might need to factor in the shorter PDCP discard timer applied to low importance PDU in case of congestion (so that they do not get prioritized wrongly).

c.  A new RLC retransmission timer. This new timer may start upon submission of RLC PDU to MAC.

2. HARQ-aware triggers: this option relies on the knowledge from the HARQ status of first attempt of the RLC transmission and tries to initiate RLC retransmission if HARQ does not seem to be able to deliver the TB(s) originated from the RLC AM entities’ SDU(s) in a timely manner. The main benefit of this option is that the RLC will have a more informed decision for retransmissions based on the status of the first transmission which in turn will reduce the negative capacity impact of autonomous methods. One main concern that needs to be addressed for the HARQ-aware schemes is to have knowledge about the mapping between TBs and RLC SDUs. Two possible solutions for triggering timely retransmissions based on HARQ-awareness:

a.	Indication from HARQ regarding several failed retransmissions: one challenge with this solution in UL is that the UE only knows about a failed HARQ transmission once it gets a scheduling grant for a new retransmission, at which point an ARQ retransmission can be initiated together with the upcoming HARQ retransmission. Therefore, the lack of explicit HARQ feedback results in added delays for triggering RLC retransmissions which hinders the timeliness on them.

b.	Explicit indication from HARQ for last HARQ retransmission attempted failure: Based on an explicit indication from the gNB, informing the UE that the last attempt of retransmission for a certain HARQ process has failed. This requires new PHY procedures that need to be discussed in RAN1.

3.	Available PUSCH resources: relying on the bursty properties of XR traffic, a simple indication that RLC retransmissions are needed would be the scheduling of a TB after transmitting all RLC PDUs of a burst once.

The simpler trigger seems to be option 1.c (new RLC retransmission timer), while option 2 should be ruled out due its high complexity with regards to unknown mapping of RLC SDUs and TBs and also involving layer 1 coordination. Option 3 needs to be studied further

Proposal 3: for autonomous retransmissions, a new RLC retransmission timer is introduced. The availability of PUSCH resources could also be investigated further. HARQ-aware triggers are ruled out.

The main concerns expressed with autonomous retransmissions had to do with capacity implications. However, the autonomous retransmissions can be kept to a minimum provided that RLC ACKs are provided rapidly. What makes the current RLC Status PDU difficult for rapid acknowledgment is that, among the SDUs that are acknowledged, all the other SDUs are negatively acknowledged. The t-Reassembly timer avoids such premature NACKs but also holds back the ACKs.

Observation 5: among the SDUs that are acknowledged in a RLC Status PDU, all the other SDUs are always negatively acknowledged.

Since RAN2 agreed to focus specifically on timely uplink retransmissions, a complete solution to this problem would be to make it configurable whether the UE considers SDUs for retransmission when NACKed. A UE that would not, would allow the network to send RLC ACKs rapidly without any concerns about premature NACKs.

Proposal 4: to minimize autonomous retransmissions, allow the network to send frequent ACKs, by introducing a possibility to configure the UE not to consider SDUs for retransmission based on received NACKs (to eradicate the problem of premature NACKs).

Another mechanism that may hold back RLC ACKs is LCP. Currently in LCP, there is no consideration regarding prioritizing control PDUs such as RLC Status PDU. The RLC Status PDU is prioritized over data PDU only within the same bearer/LCH when the LCH is treated in LCP. As a result, during LCP, the LCHs with RLC Status PDU awaiting transmission, may end up not getting enough allocation to put such PDUs in the MAC PDU being generated for transmission, delaying transmission of these PDUs.

Proposal 5: RAN2 to discuss possible enhancements in LCP regarding high priority RLC Status PDUs.

# 4	Conclusion

This document has observed the following:

Observation 1: a received PDCP SN that is more than half of the PDCP SN space ahead of SN(RX_DELIV) is misinterpreted as representing a COUNT value below RX_DELIV, resulting in discarding the PDCP Data PDU with the SN.

Observation 2: to determine how high-numbered PDCP SDUs can be transmitted, the PDCP transmitter currently maintains a local estimate (lower bound) of RX_DELIV at the peer entity, which in absence of a PDCP Status report is obtained as the lowest COUNT value not yet RLC-ACKed.

Observation 3: accounting for possible RLC SDU segments, for an unreceived RLC SDU, the new RLC timer may expire earlier than PDCP’s t-Reordering, and consequently the unreceived SDU may be RLC-ACKed before RX_DELIV at PDCP advances past that SDU.

Observation 4: RLC ACKs no longer allow the PDCP transmitter to maintain a reliable lower bound of RX_DELIV at the peer entity, also making it unable to determine how high-numbered PDCP SDUs can be transmitted

And made the following proposals:

Proposal 1: to allow the transmitting PDCP to reliably know how high-numbered PDCP SDUs it can proceed to transmit, the agreed RLC ACK no longer indicating successful delivery is combined with regular PDCP status reporting.

Proposal 2: the previously agreed stopping of transmissions of an outdated SDU is captured in the RLC specification in the form of unconditional discarding upon a discard indication from PDCP, when the RLC entity is configured with the combined approach for avoiding unnecessary retransmissions.

Proposal 3: for autonomous retransmissions, a new RLC retransmission timer is introduced. The availability of PUSCH resources could also be investigated further. HARQ-aware triggers are ruled out.

Proposal 4: to minimize autonomous retransmissions, allow the network to send frequent ACKs, by introducing a possibility to configure the UE not to consider SDUs for retransmission based on received NACKs (to eradicate the problem of premature NACKs).

Proposal 5: RAN2 to discuss possible enhancements in LCP regarding high priority RLC Status PDUs.

Annex – Text proposal for Combined approach

### 5.2.3	AM data transfer

#### 5.2.3.2	Receive operations

##### 5.2.3.2.1	General

The receiving side of an AM RLC entity shall maintain a receiving window according to the state variable RX_Next as follows:

-	a SN falls within the receiving window if RX_Next <= SN < RX_Next + AM_Window_Size;

-	a SN falls outside of the receiving window otherwise.

When receiving an AMD PDU from lower layer, the receiving side of an AM RLC entity shall:

-	either discard the received AMD PDU or place it in the reception buffer (see clause 5.2.3.2.2);

-	if the received AMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly and t-Abandon as needed (see clause 5.2.3.2.3).

When t-Reassembly or t-Abandon expires, the receiving side of an AM RLC entity shall:

-	update state variables and start t-Reassembly or t-Abandon, respectively, as needed (see clause 5.2.3.2.4 or 5.2.3.2.X, respectively).

##### 5.2.3.2.3	Actions when an AMD PDU is placed in the reception buffer

When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

-	if x >= RX_Next_Highest:

-	update RX_Next_Highest to x+ 1.

-	if all bytes of the RLC SDU with SN = x are received:

-	reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Highest_Status:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received.

-	if x = RX_Next:

-	update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received.

-	if t-Reassembly is running:

-	if RX_Next_Status_Trigger = RX_Next; or

-	if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

-	if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size:

-	stop and reset t-Reassembly.

-	if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest> RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

-	if t-Abandon is running:

-	if RX_Abandon_Trigger = RX_Next; or

-	if RX_ Abandon _Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

-	if RX_Abandon_Trigger falls outside of the receiving window and RX_Abandon_Trigger is not equal to RX_Next + AM_Window_Size:

-	stop and reset t-Abandon.

-	if t-Abandon is configured and is not running (includes the case t-Abandon is stopped due to actions above):

-	if RX_Next_Highest> RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-Abandon;

-	set RX_Abandon_Trigger to RX_Next_Highest.

##### 5.2.3.2.X	Actions when t-Abandon expires

When t-Abandon expires, the receiving side of an AM RLC entity shall:

-	update RX_Next to the SN of the first SN >= RX_Abandon_Trigger for which not all bytes have been received;

-	discard all segments with SN < updated RX_Next;

-	if RX_Next_Highest > RX_Next + 1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the RLC SDU associated with SN = RX_Next before the last byte of all received segments of this RLC SDU:

-	start t-Abandon;

-	set RX_Abandon_Trigger to RX_Next_Highest.

## 5.4	SDU discard procedures

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity configured with t-Abandon shall discard the indicated RLC SDU along with any corresponding RLC Data PDUs.

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity not configured with t-Abandon or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity not configured with t-Abandon shall not introduce an RLC SN gap when discarding an RLC SDU.

## 7.1	State variables

<…>

The receiving side of each AM RLC entity configured with t-Abandon shall maintain the following state variables:

a) RX_Abandon_Trigger – t-Abandon state variable

This state variable holds the value of the SN following the SN of the RLC SDU which triggered t-Abandon.

<…>

## 7.3	Timers

The following timers are configured by TS 38.331 [5]:

a) t-PollRetransmit

This timer is used by the transmitting side of an AM RLC entity in order to retransmit a poll (see clause 5.3.3).

b) t-Reassembly

This timer is used by the receiving side of an AM RLC entity and receiving UM RLC entity in order to detect loss of RLC PDUs at lower layer (see clauses 5.2.2.2 and 5.2.3.2). If t-Reassembly is running, t-Reassembly shall not be started additionally, i.e. only one t-Reassembly per RLC entity is running at a given time.

c) t-StatusProhibit

This timer is used by the receiving side of an AM RLC entity in order to prohibit transmission of a STATUS PDU (see clause 5.3.4).

X) t-Abandon

When configured, this timer is used by the receiving side of an AM RLC entity in order to abandon unreceived AMD PDUs (see clause 5.2.3.2). If t-Abandon is running, t-Abandon shall not be started additionally, i.e. only one t-Abandon per RLC entity is running at a given time.