3GPP TSG-RAN WG2 Meeting #129                                                  	 R2-2500500

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.7.3 RRM measurement gaps/restrictions related enhancements

Source:	NEC

Title:	RRC-based semi-static MG skipping solution

Document for:	Discussion, Decision

Introduction

On the objective, RRM measurement gaps/restrictions related enchantments, RAN2#127bis meeting achieved following agreements:

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impacts on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

At the RAN2#128 meeting, after discussing impacts on DSR and DRX from MG skipping, following agreements were achieved.

Agreements on RRM measurement gaps impacts - No MG-specific enhancements are needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped - No MG-specific enhancements are needed for DRX operation.

The remained issue is whether there is a need to additionally have RRC-based solution. This contribution discusses the need of additional supporting RRC-based semi-static solutions.

Discussion

RRC-based semi-static solutions were discussed at RAN1#118 meeting. The basic content of RRC-based semi-static solutions agreed in [1] is shown below:

- Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions;

- FFS: Details of pattern;

- FFS: Pattern is based on periodicity, offset and duration;

- FFS: Pattern is based on a bitmap;

- whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions.

Semi-static solutions were discussed at previous RAN1 meeting, however, still many FFS remained.

Benefits and drawbacks of above RRC-based semi-basic solutions were discussed and summarized by moderator in [2]. Some of them are shown below:

Benefits:

- UE usually chooses a semi-static pattern for measurement, it results in a stable measurement pattern/period. It seems RRC-based semi-static solution is more friendly in terms of UE-implementation;

- The signalling reliability can be ensured.

Drawbacks:

- While RRC-based semi-static solutions provide a stable measurement framework, the inflexibility poses challenges with non-periodic XR traffic. For example, when XR traffic is not strictly periodic, it is difficult to predict/configure the semi-static MG skipping pattern in advance. If an inappropriate pattern is configured, frequent pattern adjustment may be needed.

- For the XR traffic, 5G may provide assistance information to gNB through NGAP TSCAI on UL and/or DL periodicity and N6 jitter information (i.e., between UPF and Data Network) associated with the DL Periodicity. Additionally, reporting of uplink assistance information (jitter range, burst arrival time, UL data burst periodicity). Such XR traffic information is assumed to be considered for DRX configuration and better scheduling of uplink resources for XR.

- Basically, the RRC-based semi-static can be applicable when the XR traffic is periodic, and the jitter ranges of the downlink and uplink are known to the gNB which can pre-configure an appropriate MG skipping pattern. However, considering the non-integer value of XR traffic periodicity and variable jitter, a mismatch between the semi-static configured MG skipping pattern and XR traffic eventually occurs.

- In our understanding, the MG can only be skipped when there are less impacts on the RRM measurement performance and some pre-conditions (for example, radio conditions) are met. Therefore, no strong concerns from signalling overhead perspective on the RAN1 agreed dynamic DCI-based MG skipping solution. In other words, we don’t see strong motivations to additionally support RRC-based semi-static MG skipping on top of dynamic DCI-based MG skipping.

Do not additionally support RRC-based semi-static MG skipping on top of dynamic DCI-based MG skipping.

Conclusion

This contribution discussed the need of additional supporting RRC-based semi-static solutions.

Observation 1 Semi-static solutions were discussed at previous RAN1 meeting, however, still many FFS remained.

Proposal 1: Do not additionally support RRC-based semi-static MG skipping on top of dynamic DCI-based MG skipping.

- References

[1] R1-2407587 Final Report of 3GPP TSG RAN WG1 #118 v1.0.0

[2] R1-2407511 Moderator summary #3 - Enabling TX/RX for XR during RRM measurements