3GPP TSG RAN WG2 #129	R2-2500501

Athens, Greece, Feb 17th–21st, 2025

Agenda Item:	8.1.2.3

Source: 	Xiaomi

Title:  	Discussion on the LCM for AI positioning case 1

Document for:	Discussion and Decision

Agreements:

1	For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline.

- 	Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF .

-	Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF.  Configuration details are FFS

2   As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause.  FFS if other fallback options are considered

Introduction

In the RAN2#128 meetings, RAN2 made some agreements for the AI positioning use case 1 as below:

There is a post meeting email discussion on the LCM procedures for AI positioning use case 1, in this contribution, we further discuss the LCM for AI positioning use case 1 which is not proposed by the email discussion.

# Discussion

## AI positioning method activation/deactivation

In legacy UE-based positioning, Location Management Function (LMF) has the full controllability of how positioning is performed at UE side, including positioning methods or instructing UE to stop positioning. For UE-based positioning, LMF can request UE to report location information by indicating the type of location information needed at network side via RequestLocationInformation. The positioning session can further be terminated by Abort procedure, where an on-going procedure (e.g. location reporting from the target device) can be stopped.

Same as legacy UE-based positioning method, LMF should also take the full responsibility of configuring, selecting or stopping positioning methods when using AI/ML.

As proposed in [1], AI/ML positioning case 1 is defined as a new positioning method, where network can directly request UE to perform AI/ML positioning case 1 in RequestLocationInformation. Upon reception of RequestLocationInformation, UE can start to perform AI/ML positioning case 1, and further reports the inference results in ProvideLocationInformation. Therefore, including AI/ML positioning case 1 in RequestLocationInformation can be considered as an activation of AI/ML positioning case 1.

Proposals from [1]: Proposal 1 Introduce AI/ML positioning Case 1 as a new positioning method. Proposal 2 Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation.

Proposal 1: LMF activates AI positioning case 1 via configuring AI positioning case 1 in RequestLocationInformation.

As for deactivation of AI/ML positioning case 1, depends on whether periodic reporting or triggered reporting is configured, deactivation method could be different. Assuming LMF requests a triggered reporting for AI/ML positioning case 1, UE only reports the inference results upon event is triggered, which is one-time shot. After reporting inference results in ProvideLocationInformation, AI/ML positioning case 1 is deactivated autonomously. On the other hand, if LMF request a periodic reporting for AI/ML positioning case 1, same as legacy UE-based positioning, abort procedure can be used to stop an ongoing procedure (e.g. periodic location reporting from the target device).

Proposal 2: For triggered reporting, AI/ML positioning case 1 is deactivated autonomously after providing inference results in ProvideLocationInformation. For periodical reporting, LMF deactivates AI/ML positioning case 1 via Abort message.

However, for a certain positioning session, multiple positioning methods may be configured, including coexistence between AI/ML positioning case 1 and other non-AI/ML positioning methods. Based on UE’s own preference/status (e.g. power state), UE may only want to stop performing AI/ML positioning case 1 while keeping other non-AI/ML positioning methods running. Therefore, it is necessary for LMF to distinguish the abort cause between AI/ML positioning method and other causes. UE can indicate an AI/ML specific abort cause to LMF. Upon reception of such abort cause, LMF terminates AI/ML positioning case 1 only, which can also achieve minimum impact to non-AI/ML positioning methods.

Proposal 3: To allow LMF abort AI/ML positioning without impacting other non-AI/ML positioning method, UE reports an AI/ML positioning specific abort cause value.

## UE side data collection

During RAN1 #116bis meeting, RAN1 made following agreements on PRS configuration for UE-side data collection:

Agreements: For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: - Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. - Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

According to the RAN1 agreements, UE may initiate on-demand PRS procedure to request PRS configuration for training data collection.

It is further observed that RAN2 agreed UE can send a request to NW for BM UE-side data collection during RAN2 #128 meeting:

For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains.

Therefore, similar to beam management UE-sided model, UE can send a request to LMF for UE-side data collection by sending an on-demand PRS request.

Proposal 4: For DL PRS configuration for training data collection, UE sends on-demand PRS request to request LMF for training data collection.

Furthermore, due to completion of data collection or other UE conditions (e.g. low power, low buffer, etc), UE may want to stop UE-side data collection. Such behavior should also be reported to LMF, where LMF can stop delivering assistance data for UE-side data collection when it receives this indication.

Proposal 5: UE sends an indication to the LMF when UE wants to stop UE-side data collection.

## Fallback configuration

- Observation 3   The way forward is that companies preferring fallback configuration should provide how much latency saving can be done; i.e motivate companies who have disagreed to introduce this option. However, additional signaling overhead with fallback configuration should be considered since assistance data for two methods may have to be provided.

- Observation 4    if fallback is requested, it is requested in Step 5 LPP request location information message.

- Observation 5   Fallback configurations should be legacy methods (e.g., DL-TDOA, DL-AoD) to ensure continuity of service.

As summarized in the post meeting email discussion [1], following observations were proposed by moderator:

In this section, we mainly discuss why fallback configuration is needed.

Assuming there is no fallback configuration, the following steps are applied when the AI positioning method can’t be performed by a UE:

Step 1: LMF configures the AI positioning method in RequestLocationInformation;

Step 2: UE reports an error with the error cause;

Step 3: LMF configures the non-AI positioning method;

Step 4: UE reports the estimated location using non-AI positioning method.

If fallback configuration (i.e. non-AI positioning method) is provided to UE together with AI positioning method, only two steps are needed when the AI positioning method can’t be performed by a UE.

Step 1: LMF configures the AI positioning method and fallback configuration simultaneously;

Step 2: If AI positioning is running, fallback configuration is not performed at UE. If AI positioning causes an error, UE switches to fallback configuration and reports the requested location using the fallback configuration instead.

The fallback configuration reduces the positioning latency because it eliminates additional steps such as LPP request location information and the LPP provide location information. As per TR 38.857 [2], the latency for LPP request location information is 23-39.5ms and the latency for LPP provide location information is 20-39.5ms. The overall latency for Step 3/4 if no fallback configuration is provided will introduce extra 43 to 79ms for signaling exchange.

Table 1: Latency performance analysis of LPP Request Location Information and LPP Provide Location information

| Step | Delay Value [ms] | Description of Latency Component |
| --- | --- | --- |
| LPP Request Location Information | 23-39.5 | Processing delays: 19 ms - UE: - TUEProc-RRCDLInfo - TUEProc-LPPLocationRe - gNB: TgNBProc-NAS/LPP - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20.5ms - UE-gNB: TUE-gNB - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF |
| LPP Provide Location Information | 20-39.5 | Processing delays: 16-19 ms - UE: - TUEProc-RRCULInfo - TUEProc-LPPLocationRe - gNB: TgNBProc-NAS/LPP - AMF: TAMFProc - LMF: TLMFProc Signalling delay:4-20.5 ms - UE-gNB: TUE-gNB - gNB-AMF: TgNB-AMF - AMF-LMF: TAMF-LMF |

Observation 1: Providing fallback configuration together with AI positioning inference configuration can save 43 to 79ms for the round-trip signaling exchange between UE and LMF.

Therefore, we have following proposals for fallback configuration.

Proposal 6: LMF provides fallback configuration to UE together with AI positioning case 1 inference configuration.

# Conclusions

In this contribution, we discussed LCM for AI positioning case 1 and provide the following observations and proposals:

Activation/Deactivation

Proposal 1: LMF activates AI positioning case 1 via configuring AI positioning case 1 in RequestLocationInformation.

Proposal 2: For triggered reporting, AI/ML positioning case 1 is deactivated autonomously after providing inference results in ProvideLocationInformation. For periodical reporting, LMF deactivates AI/ML positioning case 1 via Abort message.

Proposal 3: To allow LMF abort AI/ML positioning without impacting other non-AI/ML positioning method, UE reports an AI/ML positioning specific abort cause value.

UE-side data collection

Proposal 4: For DL PRS configuration for training data collection, UE sends on-demand PRS request to request LMF for training data collection.

Proposal 5: UE sends an indication to the LMF when UE wants to stop UE-side data collection.

Fallback configuration

Observation 1: Providing fallback configuration together with AI positioning inference configuration can save 43 to 79ms for the round-trip signaling exchange between UE and LMF.

Proposal 6: LMF provides fallback configuration to UE together with AI positioning case 1 inference configuration.

# References

- [Post128][026][AIML] LCM Procedure for Positioning Case1

- TR 38.857 Study on NR positioning enhancements