3GPP TSG-RAN WG2 #129	R2-2500503

Athens, Greece, 17 – 21 February 2025

Agenda item:		8.6.3	L1 event triggered measurement reporting

Title:		MAC CE for L1 event triggered measurement report

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the past RAN2 meetings, there was good progress on the L1 event triggered measurement. The following agreements were made in RAN2#128 [1]:

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

On top of the agreements above, there are some remaining issues about the MR MAC CE contents. In this contribution, we discuss those issues and provide our views.

# 2. Discussion

2.1	Multiple candidate beams satisfying the same event

RAN2 agreed to introduce a single MAC CE format for the event-triggered L1 measurement report. There are some cases to be clarified. For example in the Event LTM3/4/5, when the network configures many candidate cells, it is possible that multiple candidate cells satisfy the same event (almost) at the same time. More precisely, it is possible that multiple beams from more than one candidate cells satisfy the event, when LTM-CSI-SSB-ResourceSet or LTM-CSI-RS-ResourceSet includes the SSB or the CSI-RS from more than one candidate cells by referring to the currently expected configurations [2] (Fig.1 shows an example). As the event-triggered L1 measurement report is handled per beam basically, it is natural for the UE to report the L1 measurement results for those multiple candidate cells.

In addition, RAN2 agreed that the max “N” is the total number of all beams included into the MR MAC CE. Thus, we understand that the “N” is the total number of beams from multiple triggered candidate cells.

Observation 1: MR MAC CE should be able to include the L1 measurement results of multiple candidate beams from more than one candidate cells satisfying the same event at the same time, up to N beams in total.

Fig.1: Example configuration of event-triggered L1 measurement report [2]

2.2	Multiple candidate beams satisfying the different events

Another possible scenario is that multiple beams satisfy the different events (e.g. beam(s) satisfy the Event LTM3, and other beam(s) satisfy the Event LTM4) at the same time. In this scenario, more than one ltm-CSI-ReportConfigId trigger the L1 measurement reports via MR MAC CE. We see two options at least; (A) single MR MAC CE including more than one triggered L1 measurement reports, or (B) multiple MR MAC CEs included in one TB (Fig.2 shows some examples). The option (A) single MR MAC CE will work with smaller overhead, while the option (B) multiple MR MAC CE will also work with slightly larger overhead but may be simpler.

When selecting the option, it would be better to consider one potential issue in this scenario. Even though the LTM events are different each other, there may be overlaps among the beams to be reported triggered by the different events. For example, if the different events satisfy for the same candidate cell(s), it is likely that the candidate beams satisfying the events are same or partially overlapped. If there are some overlaps in the candidate beams to be reported, the option of single MR MAC CE may handle this issue better than the option of multiple MR MAC CEs. Note that how/whether to avoid duplication is also the issue to be discussed.

Observation 2: MR MAC CE format should be decided by considering the case where multiple L1 measurement reports are triggered by different events at the same time, and the potential issue of overlaps among the candidate beams to be reported.

Fig.2: Examples of MR MAC CE format

2.3	Different L1 measurement triggered at different timing

In addition, we need to consider another scenario, where one ltm-CSI-ReportConfigId #1 triggers the L1 measurement reports and while the UE is wating for UL grant to send the MR MAC CE, another ltm-CSI-ReportConfigId #2 triggers the L1 measurement reports (Fig.3 shows an example). This is also related to the options of MR MAC CE format (i.e. single or multiple MR MAC CEs) discussed in 2.2 and the multiple MR MAC CEs will be simpler in this scenario.

There can be some other scenarios. The latter triggering may be due to satisfying the event or timer expiry of event-triggered periodic report. For example, instead of the other ltm-CSI-ReportConfigId #2, the same ltm-CSI-ReportConfigId #1 may trigger the L1 measurement report when other candidate beam(s) satisfy the same event as the previous one. Yet another example is that the other ltm-CSI-ReportConfigId #3 configured with event-periodic reporting triggered earlier and due to the periodic timer expires, the L1 measurement report is triggered again. Depending on the exact case, appropriate way of sending MR MAC CE may be different and thus RAN2 should investigate on this aspect as well.

Observation 3: MR MAC CE format should be decided by considering the case where while the UE is waiting for the UL grant to send the MR MAC CE for triggered L1 measurement report, another L1 measurement report is triggered.

Fig.3: Example scenarios of triggering MR MAC CE

2.4	Truncated MR MAC CE

As agreed in RAN2#128, RAN2 introduces a truncated MR MAC CE, which is used when the UL grant is not sufficient to include the beam measurement results for the triggered L1 measurement reports. We need to consider at least following cases as discussed above with assuming the truncated MR MAC CE is required.

- Case 1: Beams of the same or different candidate cell satisfy the same event (e.g. ltm-CSI-ReportConfigId #n) at the same time.

- Case 2: Beams of the same or different candidate cell satisfy the different events (e.g. ltm-CSI-ReportConfigId #n, #m) at the same time.

- Case 3: More than one L1 measurement report triggered at different time instances is available before UL grant is received. Examples scenarios:

  - The same event triggers again the L1 measurement report by other beam(s).

  - A different event also triggers the L1 measurement report by the same or other beam(s).

  - Expiry of event-periodic timer for a different event also triggers the L1 measurement report again.

We discuss some possible approaches with assuming the single MR MAC CE option discussed in 2.2.

For the Case 1, the UE triggers the L1 measurement report for candidate cell(s), but the UL grant does not fit to include the N beams. In this case, it would be simple and sufficient that the UE includes the serving beam, the triggered beam(s) and other beams as much as possible. Among other beams, the UE can select beams in descending order in terms of triggered quantity (e.g., RSRP). One detail issue may be whether the UE should sort beams among different candidate cells, which may need additional UE complexity. This can be left for further discussion.

For the Case 2, the UE triggers the L1 measurement reports for more than one candidate cells, but the UL grant does not fit to include the N beams from the triggered candidate cells. In this case, there are some options to handle the reported beams, for examples:

- Option 1:

  - Step 1: Include serving beam and triggered beam(s) from each event (e.g. ltm-CSI-ReportConfigId #n, #m).

    - If this is not possible, it is up to the UE which triggered event(s) is selected.

  - Step 2: Include other beams (in descending order) from each event, as much as possible (up to N in total).

- Option 2:

  - Step 1: Include serving beam.

  - Step 2: Include triggered beam(s) from one triggered event (e.g. ltm-CSI-ReportConfigId #n).

    - It is up to the UE which triggered event is selected (i.e. order among events).

  - Step 3: Include up to M (<N) other beams of the selected event at Step 1.

  - Step 4: Repeat Step 2 and 3 for the other triggered event(s), as much as possible (up to N in total).

Comparing two options above, if the UL grant is sufficiently large, there will be no difference. If the UL grant is not sufficient, the Option 1 would be more useful from the network perspective. As long as the network can obtain the information on the triggered candidate cells and the triggered beams, the network can judge the following action (e.g., send the LTM cell switch command MAC CE). In addition, the Option 1 can cover the Case 1 as well. We consider the Option 1 would be better way at least as baseline.

For the Case 3, we can consider the same approach as for the Case 2. In the Option 2, the first triggered event may be prioritized at the Step 2. However, it may also depend on the trigger source of each L1 measurement report, i.e. entering condition, leaving condition or periodic timer expiry of event-periodic report. One remarks is that at any time, the UE should try to report the latest results of L1 measurements for beams to be reported.

Having discussed above, the discussion is based on the assumption that the single MR MAC CE is used for multiple triggered L1 measurement reports. If the multiple MR MAC CEs are used, the discussion may not result in the same conclusion.

Observation 4: Truncated MR MAC CE format should include the triggered beams as much as possible, while the exact format should be discussed after normal MR MAC CE format is decided or together.

Based on the discussions in the sub-sections above, we propose to discuss the (Truncated) MR MAC CE format by considering the observations 1-4.

Proposal 1: RAN2 to decide the MR MAC CE format based on the observations and consideration of the following cases and the Truncated MR MAC CE later or together:

- Case 1: Beams of the same or different candidate cell satisfy the same event

- Case 2: Beams of the same or different candidate cell satisfy the different events

- Case 3: More than one L1 measurement report triggered at different time instances is available before UL grant is received. Examples scenarios are:

  - The same event triggers again the L1 measurement report by other beam(s).

  - A different event also triggers the L1 measurement report by the same or other beam(s).

  - Expiry of event-periodic timer for a different event also triggers the L1 measurement report again.

# 3. Conclusion

In this contribution we discussed the details of MR MAC CE contents. Then we made the following observations and the proposals.

Observation 1: MR MAC CE should be able to include the L1 measurement results of multiple candidate beams from more than one candidate cells satisfying the same event at the same time, up to N beams in total.

Observation 2: MR MAC CE format should be decided by considering the case where multiple L1 measurement reports are triggered by different events at the same time, and the potential issue of overlaps among the candidate beams to be reported.

Observation 3: MR MAC CE format should be decided by considering the case where while the UE is waiting for the UL grant to send the MR MAC CE for triggered L1 measurement report, another L1 measurement report is triggered.

Observation 4: Truncated MR MAC CE format should include the triggered beams as much as possible, while the exact format should be discussed after normal MR MAC CE format is decided or together.

Proposal 1: RAN2 to decide the MR MAC CE format based on the observations and consideration of the following cases and the Truncated MR MAC CE later or together:

- Case 1: Beams of the same or different candidate cell satisfy the same event

- Case 2: Beams of the same or different candidate cell satisfy the different events

- Case 3: More than one L1 measurement report triggered at different time instances is available before UL grant is received. Examples scenarios are:

  - The same event triggers again the L1 measurement report by other beam(s).

  - A different event also triggers the L1 measurement report by the same or other beam(s).

  - Expiry of event-periodic timer for a different event also triggers the L1 measurement report again.

# References

[1] R2-2410911, Report from session on V2X/SL, R18/19 MOB, and R19 NES, Vice Chairman (Samsung)

[2] R2-250xxxx, Introduction of event-triggered measurement reporting for RRC spec (from [POST128][108] Email discussion)

[3] R2-250yyyy, Running MAC CR for enhanced mobility Ph4 (from [POST128][109] Email discussion)

# Annex for reference

# A1. RAN2 agreements

Agreements on L1 event triggered MR @RAN2#127bis

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

- MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11. The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

Agreements on L1 MR event evaluation @RAN2#127

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. 	MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

Agreements on measurement enhancements for LTM @RAN2#126

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

Agreements on measurements @RAN2#125bis

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

# A2. Running RRC CR for event-triggered measurement reporting [2]

=========================================================NEXT CHANGE=========================================================

#### –	LTM-CSI-ReportConfig

The IE LTM-CSI-ReportConfig is used to configure CSI report on the cell in which the LTM-CSI-ReportConfig is included when the field ltm-ReportConfigType is configured as periodic/semi-persistentOnPUCCH/semi-persistentOnPUSCH/aperiodic.

The IE LTM-CSI-ReportConfig is also used to configure event-triggered measurement report when the field ltm-ReportConfigType is configured as eventTriggered, within which the criteria for triggering measurement report by MAC CE as in TS 38.321 [3] is specified.

Event LTM2:	Beam of serving becomes worse than absolute threshold;

Event LTM3:	Beam of candidate cell becomes amount of offset better than the beam of serving cell;

Event LTM4:	Beam of candidate cell becomes better than absolute threshold;

Event LTM5:	Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

LTM-CSI-ReportConfig information element

-- ASN1START

-- TAG-LTM-CSI-REPORTCONFIG-START

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha                                            P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...,

eventTriggered-r19                        SEQUENCE {

eventId-r19                                     CHOICE {

eventLTM2-r19                                     SEQUENCE {

ltm2-Threshold-r19                                MeasTriggerQuantity,

hysteresis-r19                                    Hysteresis,

timeToTrigger-r19                                 TimeToTrigger,

eventEvaluationRS-Type-r19                        ENUMERATED {csi-rs, ssb}                    OPTIONAL, -- Need R

...

},

eventLTM3-r19                                     SEQUENCE {

ltm3-Offset-r19                                   MeasTriggerQuantityOffset,

hysteresis-r19                                    Hysteresis,

timeToTrigger-r19                                 TimeToTrigger,

...

},

eventLTM4-r19                                     SEQUENCE {

ltm4-Threshold-r19                                MeasTriggerQuantity,

hysteresis-r19                                    Hysteresis,

timeToTrigger-r19                                 TimeToTrigger,

...

},

eventLTM5-r19                                     SEQUENCE {

ltm5-Threshold1-r19                               MeasTriggerQuantity,

ltm5-Threshold2-r19                               MeasTriggerQuantity,

hysteresis-r19                                    Hysteresis,

timeToTrigger-r19                                 TimeToTrigger,

...

},

...

},

ltm-EventTriggeredReportContent-r19                       LTM-EventTriggeredReportContent-r19,

reportOnLeave-r19                                         ENUMERATED {enabled, disabled}                 OPTIONAL, -- Need R

ltm-EventTriggeredPeriodicReport-r19                      LTM-EventTriggeredPeriodicReport-r19           OPTIONAL, -- Need S

...

}

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...

}

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

LTM-EventTriggeredPeriodicReport-r19 ::= SEQUENCE {

reportInterval-r19                             ReportInterval,

reportAmount-r19                               ENUMERATED {r2, r4, r8, r16, r32, r64, infinity},

...

}

LTM-EventTriggeredReportContent-r19 ::=     SEQUENCE {

maxNumberOfReportedBeams-r19                   INTEGER (1..256),

allowReportAnyBeam-r19                         ENUMERATED {enabled, disabled},

reportCurrentBeam-r19                          ENUMERATED {enabled, disabled},

...

-- FFS exact value of the maximum beams that can be reported. Current value set as a placeholder for ASN1 compilation.

}

-- TAG-LTM-CSI-REPORTCONFIG-STOP

-- ASN1STOP

LTM-CSI-ReportConfig field descriptions

eventEvaluationRS-Type Indicates the type of reference signal for LTM event evaluation. csi-rs corresponds to QCL RS of the indicated joint/DL TCI state; and ssb corresponds to SSB QCLed with the QCL RS of the indicated joint/DL TCI state.

eventId Type of LTM event for triggering event-triggered measurement report as specified in TS 38.321 [3].

hysteresis Hysteresis when evaluating the entering/leaving conditions for an LTM event.

ltm-EventTriggeredMeasurement Identifier for the measurement resources for event-triggered measurement report by MAC CE as specified in TS 38.321 [3]

ltm-EventTriggeredPeriodicReport This field indicates when an LTM event is triggered, whether the event-triggered measurement report is sent periodically. When the field is absent, the event-triggered measurement report is sent once, as specified in TS 38.321 [3].

ltm-ReportConfigType This field specifies how the UE shall report the measurement results for LTM either by CSI report or by event-triggered measurement report by MAC CE.

ltm-ReportContent This field defines the content of the LTM L1 measurement report. When the ltm-ReportConfigType is set to eventTriggered, this field shall be ignored.

ltm-ResourcesForChannelMeasurement This field indicates of the index of SSB or CSI-RS in the field LTM-CSI-ResourceConfig. When ltm-ReportConfigType is set to EventTriggered, and eventId is configured as eventLTM2, this field shall be ignored.

ltm2-Threshold, ltm4-Threshold, ltm5-Threshold1, ltm5-Threshold2 Thresholds defined in the entering/leaving conditions for different LTM events.

ltm3-Offset Offset for the entering/leaving condition for event LTM3. FFS whether for this field, the actual value is field value * 0.5 dB.

reportOnLeave Indicates whether the event-triggered measurement report by MAC CE shall be triggered when leaving condition is satisfied, as specified in TS 38.321 [3].

reportSlotConfig Periodicity and slot offset (see TS 38.214 [19], clause 5.2.1.4).

reportSlotOffsetList, reportSlotOffsetListDCI-0-1, reportSlotOffsetListDCI-0-2 Timing offset Y for semi persistent reporting using PUSCH and aperiodic reporting.

timeToTrigger Time during which an entering/leaving condition needs to be consistently satisfied for triggering event-triggered measurement report by MAC CE as specified in TS 38.321 [3].

LTM-ReportContent field descriptions

nrOfReportedCells This field defines how many cells are reported within a single L1 measurement report instance.

nrOfReportedRS-PerCell This field defines how many RSs per cell are reported within a single L1 measurement report instance.

spCellInclusion This field indicates whether the UE shall include a L1 measurement report associated to the current SpCell. This field can only be configured if the current SpCell is configured as an SpCell of an LTM candidate configuration and the LTM-CSI-ResourceConfig IE associated to the LTM-CSI-ReportConfig IE includes resources for the current SpCell.

LTM- EventTriggeredReportContent field descriptions

allowReportAnyBeam Indicates whether the UE shall report the measurement results for the beams not satisfying the conditions of the events as specified in TS 38.321 [3].

maxNumberOfReportedBeams This field defines number of beams whose measurements can be reported in the event-triggered measurement report by MAC CE as specified in TS 38.321 [3].

reportCurrentBeam Indicates whether the UE shall report the measurement result of the current beam as specified in TS 38.321 [3].

LTM-EventTriggeredPeriodicReport field descriptions

reportInterval This field defines the periodicity of the event-triggered periodic measurement report.

reportAmount Number of measurement reports needs to be transmitted after the event is triggered as specified in TS 38.321 [3]. Value 'r2' means the report is sent twice, ’r3’ means the report is sent three times, and so on.

==========================================================NEXT CHANGE========================================================

#### –	LTM-CSI-ResourceConfig

The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations

LTM-CSI-ResourceConfig information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-SSB-ResourceSet-r18         LTM-SSB-ResourceSet-r18,

... ,

[[

ltm-CSI-RS-ResourceSet-r19          LTM-CSI-RS-ResourceSet-r19                     OPTIONAL  --Need R

]]

}

LTM-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

LTM-CSI-RS-ResourceSet-r19 ::=     SEQUENCE {

ltm-CSI-RS-ResourceList-r19         SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF NZP-CSI-RS-ResourceId,

ltm-CandidateIdList-r19             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF LTM-CandidateId-r18,

...

}

-- TAG-LTM-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

LTM-CSI -ResourceConfig field descriptions

ltm-SSB-ResourceSet This field indicates the resource set for LTM measurement based on SSB.

ltm-CSI-RS-ResourceSet This field indicates the resource set for LTM measuremenet based on CSI-RS. When the field is present, the UE shall ignore the field ltm-SSB-ResourceSet. Editor's NOTE: FFS it is possible to configure CSI-RS without SSB and wait for R1’s progress.

LTM-SSB-ResourceSet field descriptions

ltm-CandidateIdList This field indicates the LTM candidate configuration IDs related to the SSBs in the ltm-SSB-ResourceList. The list has the same number of entries as ltm-SSB-ResourceList. The first entry in this list shall be associated to the first entry in ltm-SSB-ResourceList, the second entry of this list shall be associated to the second entry in ltm-SSB-ResourceList, and so on.

ltm-SSB-ResourceList This field is used to indicate on SS/PBCH block resources from one or more LTM candidate cells.

LTM-CSI-RS-ResourceSet field descriptions

ltm-CandidateIdList This field indicates the LTM candidate configuration IDs related to the SSBs in the ltm-CSI-RS-ResourceList. The list has the same number of entries as ltm-CSI-RS-ResourceList. The first entry in this list shall be associated to the first entry in ltm-CSI-RS-ResourceList, the second entry of this list shall be associated to the second entry in ltm-CSI-RS-ResourceList, and so on.

ltm-CSI-ResourceList This field is used to indicate on NZP CSI-RS resources from one or more LTM candidate cells.