3GPP TSG-RAN WG2#129												     R2-2500505

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.2.2

Source: 	NEC

Title:	        Ambient-IoT Paging

Document for:	 Discussion

- Introduction

In TSG RAN Meeting #106, a new WID of solutions for Ambient IoT in NR has been approved [1]. Paging will be supported as a basic function to enable DO-DTT and DT data transmission.

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

This contribution provides our views on the paging of ambient-IoT.

- Discussion

  - Avoid duplicated response to successfully responded service

In study phase we already agreed that the reader can send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN and the duplicated response from devices for the same service request should be avoided [2].

6.3.3 A-IoT paging … It is supported that the reader can send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. The duplicated response from devices for the same service request should be avoided. The A-IoT paging message can include information to avoid this duplicated response from the device to a reader. It needs to be further discussed on how to design this information in A-IoT paging message (e.g., including stage-3 details and considering the related aspects from other WGs). Then, based on this information, the device determines whether to skip sending the response to A-IoT paging message or not (if the device had successfully responded the same service before). This information should be short and simple. This information is one ID, while it needs to be further discussed whether the ID is generated by the reader or by the core network. It needs to be further discussed on the size of this information. …

From our view, as least companies should consider whether device should skip respond to the paging under following cases:

- After the device has responded a paging; it then device receives a subsequent paging associated to the same service, which contains no identifier (intends for all devices);

- After the device has responded a paging; it then receives a subsequent paging associated to the same service, which contains the identifier of the device.

From the device's perspective, there could be the following cases of receiving subsequent paging messages:

- After the device has responded a paging; it then receives a subsequent paging associated to the same service, which contains no identifier (intends for all devices);

- After the device has responded a paging; it then receives a subsequent paging associated to the same service, which contains the identifier of the device.

For case a), a subsequent paging for all device may intends for providing resources for devices which failed to access to the reader previously. Therefore, if the device had successfully responded the same service before, the device should skip sending the response to A-IoT paging message.

Device should skip sending the response to a subsequent paging, if the subsequent paging is for all devices, and indicates a successfully responded service.

Consider the cases that devices successfully respond to the paging, reader also has the knowledge of the identities of devices which had successfully responded. On top of that, like in case b), a subsequent paging contains the identifier of the device may be used to trigger another response to the same service. In that case, device may still need to respond again to the paging.

Device should send the response to a subsequent paging regardless of whether it has been responded successfully, if the subsequent paging explicitly indicates the identifier of the device.

Another question may be how device differential “successfully responded service”. During the study phase we concluded that this determination is based on an ID information carried by paging. So, this scheme would be: on one hand, from the ID information in paging, the device may know which service the paging intends for; on the other hand, the device may store information about the service(s) which device had responded. By matching the stored information and ID information in paging, device may determine whether this is a “successfully responded service”.

The ID information in paging is used to identified a service from core network.

The determination of “successfully responded service” is based on matching the stored information about the responded services and ID information in paging.

  - Paging contents for RA type determination

In previous meeting RAN2 agreed that “For all use cases, device determines the random access type from the paging message”, and the information used for random access type determination can be explicitly or implicitly. From our perspective, at least an implicit or explicit information need to be introduced to indicate whether RA resources indicated in paging is for contention-based or contention-free. This especially applicable to the cases that the paging message is for more than one device (e.g., paging message contains group id, all devices, or multiple devices). While for the cases that paging message only includes identity for a single device, the random access resources indicated in paging is for contention-free random access.

Paging message need to implicitly or explicitly indicate whether RA resources indicated in paging is for CBRA or CFRA.

Some information can be considered to implicitly indicate the contention free random access. For example, if paging message only includes identity for a single device, RA resources indicated in paging is for CFRA.

For cases that paging message only includes identity for a single device, RA resources indicated in paging is for CFRA.

If multiple device identifiers in single paging is supported, assuming a device ID list is used to indicate multiple device IDs, the list can also implicitly indicate the resource allocation for random access. For example, each random access resource may be mapped to a device according to the ranking of the device in the device ID list. By using the mapping relationship, A-IoT devices can be scattering to different random access resources and achieve contention free random access.

Figure 2.2-1 mapping relationship between the entry of device ID list and random access resources

For contention free RA configuration when paging message contains multiple device identifiers, random access resource indicated in paging message are mapped to devices according to the ranking of the device identifier in the list.

On top of that, for contention-based random access resources, if the Solution 3 (i.e. unified solution) defined in TR38.769 is applied for the CBRA, device may further determine whether to use 2-step (carry random id with upper layer data directly) or 3-step (carry random id without upper layer data) according to RA type information in paging message. And for contention-free random access resources, since there will be no collision between different devices, performing 2-step RA procedure brings more resource utilization efficiency.

For CBRA resources, introduce information in paging message to further indicate 2-step or 3-step random access procedure, if the Solution 3 (i.e. unified solution) defined in TR38.769 is applied for the CBRA.

For CFRA resources, devices perform 2-step random access.

- Conclusion

Observation 1:	From the device's perspective, there could be the following cases of receiving subsequent paging messages:

a)	After the device has responded a paging; it then receives a subsequent paging associated to the same service, which contains no identifier (intends for all devices);

b)	After the device has responded a paging; it then receives a subsequent paging associated to the same service, which contains the identifier of the device.

Proposal 1:	Device should skip sending the response to a subsequent paging, if the subsequent paging is for all devices, and indicates a successfully responded service.

Proposal 2:	Device should send the response to a subsequent paging regardless of whether it has been responded successfully, if the subsequent paging explicitly indicates the identifier of the device.

Proposal 3:	The ID information in paging is used to identified a service from core network.

Proposal 4:	The determination of “successfully responded service” is based on matching the stored information about the responded services and ID information in paging.

Proposal 5:	Paging message need to implicitly or explicitly indicate whether RA resources indicated in paging is for CBRA or CFRA.

Proposal 6:	For cases that paging message only includes identity for a single device, RA resources indicated in paging is for CFRA.

Proposal 7:	For contention free RA configuration when paging message contains multiple device identifiers, random access resource indicated in paging message are mapped to devices according to the ranking of the device identifier in the list.

Proposal 8:	For CBRA resources, introduce information in paging message to further indicate 2-step or 3-step random access procedure, if the Solution 3 (i.e. unified solution) defined in TR38.769 is applied for the CBRA.

Proposal 9:	For CFRA resources, devices perform 2-step random access.

- References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Huawei (moderator, RAN1 Vice-Chair).

[2] TR 38.769, Study on solutions for ambient IoT (Internet of Things)