3GPP TSG-RAN WG2 Meeting #129					           R2-2500507

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.1.2.2

Source:	Samsung

Title:	Discussion on LCM for UE-sided model for beam management

Document for:	Discussion and Decision

# Introduction

RAN1 sent a reply LS on BM applicable functionality reporting [1]. In this document, we analyze the required signaling and procedure to incorporate RAN1 agreement.

# Understanding on RAN1 agreement

Based on RAN1 LS, basically, RAN1 agreed to support two different options to request applicable functionality reporting as follows.

- Option A: “one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)”

  - Expected procedure:

    - In Step 3, gNB provides one or more CSI-ReportConfig for applicable reporting.

    - In Step 4, UE provides the applicable functionality reporting by indicating one or more CSI-ReportConfig which are applicable.

    - In option A, Step 5 can be skipped because full inference related configuration is provided in Step 3.

- Option B: “One set or multiple sets of inference related parameters for applicability report only (not for inference)”

  - Expected procedure:

    - In Step 3, gNB provides one or multiple sets of inference related parameters for applicable functionality reporting.

    - In Step 4, UE provides the applicable functionality reporting by indicating one or more sets of inference related parameters which are applicable.

    - In Step 5, gNB provides CSI-ReportConfig for inference configuration.

# Discussion on functionality procedure

In this section, we would progress the functionality procedure based on RAN1 response i.e. resolving FFS points from the previous discussion.

Before discussing each step, we can also look at terminologies for functionalities from RAN1.

- The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs - The concept/terminology “functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters - The Activated functionalities may be enabled based on CSI framework.

Based on RAN1 terminologies, it is clear that applicable functionalities can be mapped to CSI-ReportConfig although it is not clear how to define supported functionalities or activated functionalities.

Proposal 1: RAN2 assume that applicable functionality is equivalent to CSI-ReportConfig for applicable functionality for BM use case.

Step 1 & 2

The main question from RAN2 is what is the granularity of supported functionality in order to design capability signaling. However, RAN1 responded that it will be discussed later. Without RAN1 input, it would be hard to progress in RAN2.

Proposal 2: RAN2 postpone further discussion on details on supported functionalities.

Step 3

In Step 3, the following is open issue.

“3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.”

Based on Section 2, it is reasonable assumption that either CSI-ReportConfig (full inference configuration) or set(s) of inference related parameter can be provided in Step 3.

Proposal 3: RAN2 agree that either “CSI-ReportConfig for inference” or “set(s) of inference related parameters” can be provided in Step 3.

Between “Step 3” and “Step 4”

The open issue in this step is:

“FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.“

Based on RAN1 agreement, both Option A and Option B may include associated ID and it appears that it is hard to mandate to include adding associated ID although it is an important parameter to identify applicable functionality. In addition, it is our understanding that exact content of NW-side additional condition associated to associated ID will not be likely concluded or defined. In that sense, it would be reasonable to leave it to implementation than specifying the case where NW-side additional condition is not provided.

Proposal 4: RAN2 agree that it is left to UE implementation the applicable functionality is decided if NW-side additional condition is not provided in step 3 unless RAN1 provides any other input.

Step 4

The open issue in this step is:

“As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). “

Based on RAN1 agreement, UE can provide applicable functionality reporting on either CSI-ReprtConfig or set(s) of inference related parameters.

Proposal 5: RAN2 agree that UE can provide applicable functionality reporting corresponding to configured CSI-ReportConfig or set(s) of inference related parameters.

Another open issue is about activate status of functionality when it includes inference configuration. This is still valid question for option A.

“RAN2 also agreed the applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state.  FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.  FFS if multiple applicable functionalities can be activated at the same time.   FFS what is the granularity of functionality.”

Regarding activation state, RAN1 informed following agreements.

- Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

UE can assume that periodic CSI report is activated if the corresponding CSI-ReportConfig (which includes all parameters for inference) is applicable. RAN1 indicates that periodic CSI Report is activated only if applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. It can be questioned when exactly UE activate periodic CSI report.

- Approach 1: UE activates periodic CSI upon transmission of RRCReconfigurationComplete.

- Approach 2: UE activate periodic CSI-ReportConfig upon reception of RRC Reconfiguration message which is same as legacy operation.

Either approach seems feasible. The benefit of approach 1 is that gNB can also activate CSI-RS resource set upon RRCReconfiguraionComplete message. However, considering time delay between RRC reconfiguration and RRC reconfiguration complete (a few tens of msec), it is not likely that gNB performs very dynamic configuration changes for CSI-RS resource set. In that sense, it is simpler to keep the legacy operation with condition of applicability of the corresponding of CSI-ReportConfig.

Proposal 6: RAN2 agree that initial status of periodic CSI report is determined with the applicability of the corresponding of CSI-ReportConfig.

As RAN1 agreed, in case of aperiodic CSI Report and semi-persistent CSI report, it can be still activated/triggered by NW via MAC/L1 signaling. Therefore, activation upon RRC reconfiguration doesn’t need to be considered.

Proposal 7: RAN2 agree that it is not necessary to define initial status for semi- persistent /aperiodic CSI report is determined with the applicability of the corresponding of CSI-ReportConfig.

# Signaling structure

RAN1 agreed to support both option A and option B for applicable reporting framework. From RAN2 point of view, it is worthwhile to discuss how signaling structure is operated to support both options.

As a starting point, we could discuss two approaches to support two options as follows.

- Approach 1: one same format to accommodate both options

  - Full CSI-ReportConfig: this is used to either request applicability reporting (in option A) or to enable inference (in option A and/or in option B)

  - Partial CSI-ReportConfig: this is used to request applicability reporting (in option B).

- Approach 2: two different formats to accommodate each option

  - CSI-ReportConfig format: this is for option A.

  - a new IE (e.g. set-InferenceConfig): this is for option B and it includes set(s) of inference parameters.

Both approaches seem feasible to support from signaling point of view. The different format approach might be clear to distinguish two different options but it would not be flexible. Based on RAN1 agreements, it is not clear why both options are needed but it seems assumed that Option A and option B could be used concurrently based on “and/or”. In different format, it may be complicated to distinguish UE operation based on different format.

Proposal 8: RAN2 agree to support one same format (CSI-ReportConfig) to support both Option A (i.e., Full CSI-ReportConfig) and Option B (i.e., Partial CSI-ReportConfig).

# Data collection

In the previous RAN2 discussion on data collection, there are two FFSs which we discuss in this section.

FFS  #1: what the UE request contains.

Although RAN1 is still under discussion, it seems likely that CSI-ReportConfig is used for configuring data collection for UE-sided model. Similarly, UE can also indicate preferred configuration to collect data given that UE (or UE server) should determine what data should be collected for training of UE side model. In addition, it would be more useful if gNB get more information from UE considering UE’s current capability/preference considering processing power/buffer status which is more dynamic than UE capability information.

Given that it is quite related to RAN1 design, we could wait for RAN1 input to define resource configuration for UE request.

Proposal 9: RAN2 agree that UE can provide preferred data collection configuration in Data collection request and wait for RAN1 to decide detailed resource configuration format.

On top of data collection configuration, it is worthwhile to discuss additional information that can be helpful. One additional information that might be helpful is time period related for data collection. Since UE doesn’t report measurement results to gNB in case of data collection for UE side mode training  and also the need of data collection is decided at UE/UE-side server, there is no clear clue when gNB release resource configuration. A specific time duration could be useful for gNB to manage resource sets overall.

Proposal 10: RAN2 agree that UE can provide preferred time duration until when resource configuration should be required.

FFS #2: on the need of UE indication when UE can’t perform data collection based on received configuration.

In the last meeting, one controversial agreement is that gNB can provide data collection configuration without UE request. It may be still working given that gNB should be able to allocate any resource sets if those are available in gNB side.

However, it is still questionable whether it is more resource efficient approach. The reason is because gNB may not know exactly what data collection configuration UE needs or more fundamentally whether UE wants or is able to perform measurements for data collection considering UE side situation e.g. processing power, memory status, etc.

In addition, once gNB configured data collection configuration, UE may want to stop data collection due to internal reason.

To avoid resource waste (i.e. UE doesn’t perform measurement even if gNB allocates resource set), it would be desirable to have a mechanism that UE can indicate stop or release configuration when UE can’t perform data collection based on received configuration.

Proposal 11: RAN2 agree that UE can indicate stop/release data collection configuration and it is up to gNB whether gNB respond to UE’s indication.

As a detailed signaling option, UAI is a more flexible option to provide UE stop/release whenever it is required. However, given that UE can also determine upon RRC reconfiguration reception, stop/release indication can be also included in RRC reconfiguration complete message.

Proposal 12: RAN2 agree that UE indication for stop/release can be indicated in UAI or RRC reconfiguration complete message.

# Conclusion

In this document, we discussed remaining FFSs based on RAN1 reply LS.

Based on the discussion, we propose the following points.

Proposal 1: RAN2 assume that applicable functionality is equivalent to CSI-ReportConfig for applicable functionality for BM use case.

Proposal 2: RAN2 postpone further discussion on details on supported functionalities.

Proposal 3: RAN2 agree that either “CSI-ReportConfig for inference” or “set(s) of inference related parameters” can be provided in Step 3.

Proposal 4: RAN2 agree that it is left to UE implementation the applicable functionality is decided if NW-side additional condition is not provided in step 3 unless RAN1 provides any other input.

Proposal 5: RAN2 agree that UE can provide applicable functionality reporting corresponding to configured CSI-ReportConfig or set(s) of inference related parameters.

Proposal 6: RAN2 agree that initial status of periodic CSI report is determined with the applicability of the corresponding of CSI-ReportConfig.

Proposal 7: RAN2 agree that it is not necessary to define initial status for semi- persistent /aperiodic CSI report is determined with the applicability of the corresponding of CSI-ReportConfig.

Proposal 8: RAN2 agree to support one same format (CSI-ReportConfig) to support both Option A (i.e., Full CSI-ReportConfig) and Option B (i.e., Partial CSI-ReportConfig).

Proposal 9: RAN2 agree that UE can provide preferred data collection configuration in Data collection request and wait for RAN1 to decide detailed resource configuration format.

Proposal 10: RAN2 agree that UE can provide preferred time duration until when resource configuration should be required.

Proposal 11: RAN2 agree that UE can indicate stop/release data collection configuration and it is up to gNB whether gNB respond to UE’s indication.

Proposal 12: RAN2 agree that UE indication for stop/release can be indicated in UAI or RRC reconfiguration complete message.

# Reference

[1] R2-2500011, Reply LS on applicable functionality reporting for beam management UE-sided model, RAN1