3GPP TSG-RAN WG2 Meeting #129	R2-2500508

Athens, Greece, Feb. Feb. 17th – 21st, 2025

Agenda Item	: 8.13.2

Source	: LG Electronics Inc.

Title	: Discussion on Relay discovery and selection for multi-hop U2N relay

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-241609 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we propose the conditions to be an intermediate (/first) L2/L3 U2N Relay UE. We investigate under which condition the discovery message is forwarded to support multi-hop relay. Also, we also propose a relay reselection scheme that can be different depending on the discovery model.

2.	Discussion

2.1 Discovery message forwarding at the intermediate Relay UE

As agreed at the last meeting agreements, NW can configure Uu RSRP upper bound for the intermediate Relay UE, but cannot configure Uu RSRP lower bound. This does not mean that the intermediate Relay UE is always out of coverage. The intermediate Relay UE can be in coverage or out of coverage depending on the configured Uu RSRP upper bound threshold. Meanwhile, according to the meeting agreements for the topologies, the RRC CONNECTED intermediate Relay UE in the different cell from the last Relay UE cannot act as an intermediate Relay UE for the last U2N Relay UE. Therefore, it is necessary to consider the conditions under which an intermediate Relay UE can act as an intermediate Relay UE for the last Relay UE belonging to a different cell from itself.

First, if the intermediate Relay UE is in a different cell from the last Relay UE, the intermediate Relay UE should be in IDLE/INACTIVE state for its own camping-on cell, before connecting to the last Relay UE. Second, the camping-on cell of the intermediate Relay UE should broadcast a discovery related SIB, which is also including TX resource pool configuration for the concerned SL frequency of the intermediate Relay UE. In Rel-17 U2N Relay operation, the U2N Relay/Remote UE in IDLE/INACTIVE state should enter RRC CONNECTED state to acquire dedicated configuration on the TX resource pool, if the broadcasted SIB includes discovery related SIB but does not include TX resource pool configuration. However, the intermediate Relay UE in Rel-19 multi-hop U2N operation, the intermediate Relay UE should not enter RRC CONNECTED state to acquire dedicated configuration if the camping-on cell of the intermediate Relay UE is different from the last Relay UE. Third, if the camping-on cell of the intermediate Relay UE broadcasts Uu RSRP upper bound via SIB, the intermediate Relay UE should satisfy the Uu RSRP threshold condition of the cell. In summary, the intermediate Relay UE cannot be in RRC CONNECTED state with the cell other than the last Relay UE. The intermediate Relay UE in the RRC IDLE/INACTIVE state should follow the SIBs broadcasted in its own the camping on cell of itself before making SL connection with the last Relay UE.

Proposal 1: The intermediate capable Relay UE in RRC_IDLE/INACTIVE state belonging to a different cell from the last U2N Relay UE can act as an intermediate Relay UE in the following cases:

- If the camping-on cell of the intermediate Relay UE broadcasts a discovery related SIB which is also including Tx resource pool configuration for the concerned SL frequency, and if the Uu-RSRP threshold for the intermediate Relay UE is broadcasted and the Uu-RSRP of the intermediate Relay UE meets the threshold, or

- If the camping-on cell of the intermediate Relay UE broadcasts a discovery related SIB which is also including Tx resource pool configuration for the concerned SL frequency, and if the Uu-RSRP threshold for the intermediate Relay UE is not broadcasted.

Proposal 2: The intermediate capable Relay UE in RRC_IDLE/INACTIVE state belonging to a different cell from the last U2N Relay UE cannot act as an intermediate Relay UE if the camping-on cell of the intermediate Relay UE does not broadcast a discovery related SIB.

Proposal 3: The intermediate capable Relay UE in RRC_IDLE/INACTIVE state belonging to a different cell from the last U2N Relay UE cannot act as an intermediate Relay UE if the camping-on cell of the intermediate Relay UE broadcasts discovery related SIB, but Tx resource pool configuration is absent for the concerned SL frequency.

2.2 Whether to use same of different L2 ID in the defined topologies

In the last meeting, we agreed that the following two topologies are available. However, whether the same or different L2 ID can be used is still an open question. More specifically, whether L2 ID(D) and L2 ID(D’) in Figure 2.5A-2 can be the same or different is an FFS part. Similarly, whether L2 ID(B) and L2 ID(B’) in Figure 2.5B-1 can be the same or different is a similar FFS part.

[EASY] Proposal 14: following cases are supported: - One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE) FFS whether the L2 ID(D) and L2 ID(D’) can be the same value in the below figure. (Figure 2.5A-2) - Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE. FFS whether the L2 ID(B) and L2 ID(B’) can be the same value in the below figure. (Figure 2.5B-1)

We don’t think it's an issue in the RAN2 whether the same or different L2 ID can be used. It’s an upper layer issue. Regardless of whether the K2 UD is the same or different L2 ID, when reporting to the gNB via SUI, the last/intermediate Relay UE needs to report the source-destination L2 ID pair. For example, in the case of Figure 2.5A-2, the last Relay UE reports the source-destination L2 ID pair such as {L2 ID(D), L2 ID(C)} and {L2 ID(D’), L2 ID(C’)} via a SUI. Similarly, in the case of Figure 2.5B-1, the intermediate Relay UE reports the source-destination L2 ID pair such as {L2 ID(B), L2 ID(A)} and {L2 ID(B’), L2 ID(A’)}. We believe that if the NW can identify the link with the pair, there will be no problem.

Proposal 4: In Figure 2.5A-2, the L2 ID(D) and LD ID(D’) can be the same or a difference value. If the last Relay UE reports the source L2 IDs and destination L2 IDs by using SUI, the last Relay UE should report the source-destination L2 ID pair to identify other links.

Proposal 5: In Figure 2.5B-1, the L2 ID(B) and LD ID(B’) can be the same or a difference value. if the intermediate Relay UE reports the source L2 IDs and destination L2 IDs by using SUI, the last Relay UE should report the source-destination L2 ID pair to identify other links.

2.3 Relay (re)selection at the intermediate Relay UE

According to the approved 23.304 CR in SA2(S2-2501295), the intermediate Relay UE shall establish a Layer-2 link with its parent intermediate Relay UE before sending the Discovery Announcement message. In Discovery model A, the intermediate Relay UE can select its parent Relay UE by its own decision. However, in the Discovery model B, the Remote UE decides the entire path toward the last Relay UE. This means that the intermediate Relay UE can select its parent intermediate Relay UE or last Relay UE when relay reselection is triggered, if the connection is made by Discovery model A.  However, the intermediate Relay UE that triggered relay reselection cannot select its parent intermediate Relay UE or last Relay UE, if the connection is made by Discovery model B.

<23.304 CR: S2-2501295> ##### 6.4.3.8.1 Layer-2 Link establishment and management Each of the 5G ProSe Intermediate UE-to-Network Relay shall establish a Layer-2 Link with its parent Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, before it can serve the 5G ProSe Remote UE, or a child Intermediate UE-to-Network Relay. The 5G ProSe Intermediate UE-to-Network Relay shall establish the Layer-2 Link with a parent relay before sending the 5G ProSe UE-to-Network Relay Discovery Announcement message. <omit> …

We believe that the whole intermediate Relay UEs and Remote UEs connected to the same last Relay UE establish the connection by using the same discovery model. This is because the RSC (Relay Service Code) value in the discovery message is only one value. Each intermediate Relay UE decides whether to forward the received discovery message or not depending on the RSC value. We understand that if the intermediate Relay UE uses an RSC for the Discovery model A, the intermediate Relay UE can only forward the discovery message with the same RSC value, i.e. the Discovery model A. Based on the SA2 approved CR, the intermediate Relay UE that has connected using Discovery model A can perform relay reselection by itself if the triggering condition is met.

Proposal 6: When relay reselection is triggered at the intermediate Relay UE, the intermediate Relay UE can select its parent relay UE by itself, if the intermediate Relay UE has an SL connection by using Discovery model A.

The condition and cause value for sending a notification message toward a Remote UE can be different depending on the discovery model of the intermediate Relay UE. If the intermediate Relay UE is connected with the parent Relay UE via Discovery Model A, the intermediate Relay UE can select a new parent relay UE if the relay reselection is triggered. The intermediate Relay UE can send a notification message with a cause value toward the Remote UE after the PC5-S/RRC connection is completed with a newly selected parent Relay UE. Upon receiving this notification message, the Remote UE may send an RRC connection establishment message toward the gNB via the current parent intermediate Relay UE. Otherwise, if the intermediate Relay UE fails to establish the PC5-S/RRC connection with a new parent Relay UE, the intermediate Relay UE sends a notification message with a cause value toward a Remote UE. Upon receiving this notification message, the Remote UE can release the current PC5 connection with the parent Relay UE and then trigger relay reselection.

Proposal 7: In the Discovery model A, if relay reselection is triggered at the intermediate Relay UE, the intermediate Relay UE sends a notification message toward Remote UE after the PC5-S/RRC connection is successfully established with the newly selected parent Relay UE. Upon receiving this notification message, the Remote UE can send RRC re-establishment message via the current parent Relay UE (i.e., no triggering relay reselection).

Proposal 8: In the Discovery model A, if relay reselection is triggered at the intermediate Relay UE, the intermediate Relay UE sends a notification message toward Remote UE after PC5-S/RRC connection has failed with a newly selected parent Relay UE. Upon receiving this notification message, the Remote UE releases the current parent Relay UE and triggers relay reselection.

If the intermediate Relay UE has a connection via Discovery model B, the intermediate Relay UE sends a notification message toward the Remote UE when the notification message sending condition occurs. For example, the intermediate Relay UE sends a notification message when it detects PC5 RLF, receives HO commend or performs cell reselection. In addition, the intermediate Relay UE can send a notification message when the upper layer of the intermediate Relay UE requests the release of the PC5-RRC connection.

Proposal 9: If the intermediate Relay UE has a SL connection using Discovery model B, the intermediate Relay UE sends a notification message toward the Remote UE as following cases:

- When detecting PC5 RLF

- When receiving HO command

- When performing cell reselection

- When failed RRC connection/resume/establishment

- When receiving the PC5-RRC release request from the upper layer

- When receiving notification message from the parent intermediate Relay UE

3.	Conclusion

Proposal 1: The intermediate capable Relay UE in RRC_IDLE/INACTIVE state belonging to a different cell from the last U2N Relay UE can act as an intermediate Relay UE in the following cases:

- If the camping-on cell of the intermediate Relay UE broadcasts a discovery related SIB which is also including Tx resource pool configuration for the concerned SL frequency, and if the Uu-RSRP threshold for the intermediate Relay UE is broadcasted and the Uu-RSRP of the intermediate Relay UE meets the threshold, or

- If the camping-on cell of the intermediate Relay UE broadcasts a discovery related SIB which is also including Tx resource pool configuration for the concerned SL frequency, and if the Uu-RSRP threshold for the intermediate Relay UE is not broadcasted.

Proposal 2: The intermediate capable Relay UE in RRC_IDLE/INACTIVE state belonging to a different cell from the last U2N Relay UE cannot act as an intermediate Relay UE if the camping-on cell of the intermediate Relay UE does not broadcast a discovery related SIB.

Proposal 3: The intermediate capable Relay UE in RRC_IDLE/INACTIVE state belonging to a different cell from the last U2N Relay UE cannot act as an intermediate Relay UE if the camping-on cell of the intermediate Relay UE broadcasts discovery related SIB, but Tx resource pool configuration is absent for the concerned SL frequency.

Proposal 4: In Figure 2.5A-2, the L2 ID(D) and LD ID(D’) can be the same or a difference value. If the last Relay UE reports the source L2 IDs and destination L2 IDs by using SUI, the last Relay UE should report the source-destination L2 ID pair to identify other links.

Proposal 5: In Figure 2.5B-1, the L2 ID(B) and LD ID(B’) can be the same or a difference value. if the intermediate Relay UE reports the source L2 IDs and destination L2 IDs by using SUI, the last Relay UE should report the source-destination L2 ID pair to identify other links.

Proposal 6: When relay reselection is triggered at the intermediate Relay UE, the intermediate Relay UE can select its parent relay UE by itself, if the intermediate Relay UE has an SL connection by using Discovery model A.

Proposal 7: In the Discovery model A, if relay reselection is triggered at the intermediate Relay UE, the intermediate Relay UE sends a notification message toward Remote UE after the PC5-S/RRC connection is successfully established with the newly selected parent Relay UE. Upon receiving this notification message, the Remote UE can send RRC re-establishment message via the current parent Relay UE (i.e., no triggering relay reselection).

Proposal 8: In the Discovery model A, if relay reselection is triggered at the intermediate Relay UE, the intermediate Relay UE sends a notification message toward Remote UE after PC5-S/RRC connection has failed with a newly selected parent Relay UE. Upon receiving this notification message, the Remote UE releases the current parent Relay UE and triggers relay reselection.

Proposal 9: If the intermediate Relay UE has a SL connection using Discovery model B, the intermediate Relay UE sends a notification message toward the Remote UE as following cases:

- When detecting PC5 RLF

- When receiving HO command

- When performing cell reselection

- When failed RRC connection/resume/establishment

- When receiving the PC5-RRC release request from the upper layer

- When receiving notification message from the parent intermediate Relay UE

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet.