3GPP TSG-RAN WG2 #129 meeting	R2-2500514

# Athens, Greece, 17th Feb – 21st Feb, 2025

Source:		NEC

Agenda item:		8.7.6	XR rate control

Title:		Uplink rate control for XR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#128 meeting, agreements [1] on XR rate control were made as below:

- RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

In this contribution, we will further discuss the remaining issues of XR rate control.

# 2. Discussion

2.1 per QoS flow recommended bit rate.

In the last RAN2 meeting, we discussed whether to indicate the recommended bit rate by RRC signaling. According to the latest WID for XR, the goal of rate control enhancement is to enable faster source rate adaptation to uplink congestion. In contrast to MAC CE-based rate indication, which is efficient, RRC signaling-based rate indication is less timely and may lead to increased delay in source rate adaptation. Furthermore, since the legacy MAC CE-based solution for rate indication has worked well, it is reasonable to retain this approach with improvements to support per-QoS flow rate indication.

Proposal 1: Per QoS flow recommended bit rate is indicated by MAC CE.

Recommended bit rate query from UE is used to check whether a bit rate recommended by its peer can be provided by the gNB. It may be initiated when the UE receives a bit rate request from its peer server or from its application layer; in other words, gNB may not get such information from CN. From this perspective, recommended bit rate query should be supported.

Given that per-QoS flow bit rate indication will be supported and legacy MAC CE for the query is at the granularity of LCH, we propose introducing a new MAC CE for the query, which is at the granularity of QoS flow. Similar to legacy MAC CE, the MAC CE can use the same format as that used for recommended bit rate indication.

Proposal 2: RAN2 supports per QoS flow Recommended bit rate query, and the MAC CE for recommended bit rate indication is reused for query.

When determining the recommended bit rate, only QoS flows that support bit rate adaptation should be considered. In the RAN2#128 meeting, it was agreed that the gNB can be indicated which QoS flows support source bit rate adaptation.

For uplink traffic, only the UE has knowledge of whether a specific service or QoS flow supports source bit rate adaptation. Therefore, the UE can report this information to the network, for example, via UAI. With it, the network can properly configure or determine the recommended bit rate for uplink traffic.

Regarding indication of QoS flows that support bit rate adaptation from CN, we think it is intended for downlink traffic. Given that we have agreed RAN2 will not discuss rate indication for DL unless WID is updated to include it by RANP, there is no need to introduce such an indication from CN unless bit rate indication for DL is supported.

Proposal 3: For uplink traffic, UE reports the information of QoS flows that support bit rate adaptation via UAI.

Proposal 4: For downlink traffic, do not introduce an indication of QoS flows that support bit rate adaptation from CN unless bit rate indication for DL is supported.

Different QoS flows may have different preferences for bit rate recommendation granularity. For example, one might favor bit rate recommendation on a per-LCH basis, while another might prefer bit rate recommendation on a per-QoS flow basis. Consequently, UE should inform the gNB of its preference of bit rate recommendation granularity for a given QoS flow. Based on this, the network can indicate the recommended bit rate in the UE's preferred format.

Proposal 5: For a QoS flow, UE informs the gNB of its preference for bit rate recommendation granularity, either on a per-LCH basis or on a per-QoS flow basis.

2.2 recommended bit rate during CA/DC.

The data of a QoS flow may be transmitted over different carriers (or LCHs) belonging to the same or different MAC entities when performing PDCP duplication or split transmission. Due to varying congestion situations in different cells, UE may receive different recommended bit rate values for the same QoS flow from different cells or cell groups (e.g., during CA/DC). In this case, RAN2 should discuss how to apply the indicated recommended bit rate.

Figure 1: recommended bit rates from different cells/CGs

For example, UE may be provided with recommended bit rate 1 of a QoS flow from cell 1 and recommended bit rate 2 of the same QoS flow from cell 2.

- For PDCP duplication, the UE may not be expected to go beyond the recommended bit rate of min (recommended bit rate 1, recommended bit rate 2) for the QoS flow.

- For split transmission, the recommended bit rate for a QoS flow may be applied to the cell or cell groups where the MAC CE is received. UE should guarantee that the bit rate of the QoS flow in each cell or cell group will not go beyond the recommended bit rate of the corresponding cell or cell groups.

Proposal 6: RAN2 to discuss how to apply the indicated recommended bit rate for a QoS flow when more than one indication is received from different cells or cell groups (e.g., during CA/DC).

# Summary

This contribution provides our analysis on the uplink rate control for XR, and has the following proposals:

Proposal 1: Per QoS flow recommended bit rate is indicated by MAC CE.

Proposal 2: RAN2 supports per QoS flow Recommended bit rate query, and the MAC CE for recommended bit rate indication is reused for query.

Proposal 3: For uplink traffic, UE reports the information of QoS flows that support bit rate adaptation via UAI.

Proposal 4: For downlink traffic, do not introduce an indication of QoS flows that support bit rate adaptation from CN unless bit rate indication for DL is supported.

Proposal 5: For a QoS flow, UE informs the gNB of its preference for bit rate recommendation granularity, either on a per-LCH basis or on a per-QoS flow basis.

Proposal 6: RAN2 to discuss how to apply the indicated recommended bit rate for a QoS flow when more than one indication is received from different cells or cell groups (e.g., during CA/DC).

# References

[1] Report of 3GPP TSG RAN WG2 meeting #128, Orlando, the USA

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.4.0