3GPP TSG-RAN WG2#129	R2-2500519

Athens, Greece, Feb. 17th – 21st, 2025

Source: 	Fujitsu

Agenda item:	8.5.3

Title:	Discussion on on-demand SIB1 procedure for NES

Document for:	Discussion and decision

# Background

In RAN2#128 meeting, RAN2 made some agreements as follows [1].

Agreements on OD-SIB1

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.

In this contribution, we would like to discuss and provide our views on the on-demand SIB1 operation from RAN2 perspective.

# Discussion

## UL WUS procedure

### UL WUS configuration

UL WUS configuration by RRC release message

In RAN2#126 meeting, the following issue was discussed and RAN2 agreed whether or not to adopt this as an option in stage 3.

2.	RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3

In RAN2#128 meeting, RAN2 discussed and reached “UL-WUS configuration in RRC release is not supported” as a WA. Because some companies think it is beneficial in terms of reducing broadcast signalling. But if the NW does not broadcast the UL WUS configuration, the NW cannot update UL WUS configuration after the NES UE is in the idle/inactive mode. In addition, the NES UE re-selected from a neighboring cell to Cell A does not have a valid UL WUS configuration. So, it imposes another mechanism to provide the UL WUS configuration, e.g., periodical SI update. Therefore, it is suspicious this option can reduce overhead of broadcast signalling for UL WUS configuration.

Proposal 1:	RAN2 to confirm the working assumption, i.e. UL-WUS configuration in RRC release is not supported.

Contents of UL WUS configuration

In RAN1#118bis meeting, there are following agreements on the parameters included in the UL WUS configuration.

Agreement For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. To which cell does the configuration applies; NES-CellId; PhysCellId. ARFCN-ValueNR. Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell. Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration: - rsrp-ThresholdSSB - prach-RootSequenceIndex - msg1-SubcarrierSpacing - restrictedSetConfig Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon Agreement For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz (working assumption)ULSubCarrierSpacing. Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. SSB-positionInBurst. tdd-UL-DL-ConfigurationCommon. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResources; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. Note: - In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB - Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA

In RAN1#119 meeting, there were following agreements on UL-WUS configuration:

Agreement The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration. Agreement At least for RO validation determination for WUS transmission, the parameter “ssb-PeriodicityServingCell” is included in the UL-WUS configuration at least for TDD system. - FFS: FDD system Agreement Confirm the working assumption below from RAN1 #118-bis to include it in the UL-WUS configuration. - (working assumption) ULSubCarrierSpacing Agreement For parameters agreed for UL WUS, whether it is mandatory or optional is for further discussion. Agreement At least for the case where SSB is transmitted on sync-raster, the indication of the quantity K_SSB (defined in TS 38.211 as subcarrier offset from subcarrier 0 in common resource block NCRBSSB to the lowest-numbered subcarrier of the SS/PBCH block) is included in the UL-WUS configuration for FDD and TDD NES cell. Agreement The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration. - Unit of the duration is in slot - FFS: Starting time

So far, it was agreed that the NES cell’s UL WUS configuration is included in a new SIB (including its own UL WUS configuration). Therefore, the new SIB in Cell A or in the NES cell should include the above RAN1 agreed parameters for UL WUS configuration.

Proposal 2:	The parameters agreed by RAN1 to be included in UL WUS configuration are included in the new SIB for UL WUS configuration.

### UL WUS transmission

RRC connected mode

In RAN#106 meeting, it was discussed whether UL WUS transmission can be also supported in RRC connected mode or not. Because in the current WID, UL WUS transmission is allowed only in RRC idle/inactive mode, then clarification is required for it. After a short offline discussion, the following way forward was endorsed for RRC connected mode [2].

On RRC Connected mode - The scope of WID will not be updated to include RRC Connected. - However, RAN2 will address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission. No need to change the scope of WI to indicate this.

Based on the above RAN plenary guideline, SIB1 request by a dedicated signaling in connected mode (i.e., RRCSystemInfoRequest message) is not pursued whereas SIB1 request to a selected cell during RLF can be discussed in RAN2.

Observation 1:	SIB1 request by a dedicated signaling in connected mode is not pursued.

In RAN2#127bis meeting, RAN2 agreed “A cell for which SIB1 request configuration is available, can periodically broadcast SIB1”, it means the NES UE may attempt to receive SIB1 if the NES UE already has UL WUS configuration corresponding to the NES cell.

It can apply to a cell selection during RRC re-establishment (i.e., timer T311 is running), if the NES UE has UL WUS configuration corresponding to the candidate NES cell, there is no reason to prevent SIB1 reception of NES cell. In this case, the NES UE just follows the legacy RRC connection re-establishment procedure. One question is whether the NES UE can trigger UL WUS transmission to the candidate cell if SIB1 is currently not broadcasted, e.g., due to on-demand mode. In this case, it should not be a trigger of UL WUS transmission, because timer T311 can be configured up to 30000ms then UL WUS configuration may be changed before cell selection, but the NES UE does not know it. In RAN2#126 meeting, RAN2 already agreed, the SIB1 request procedure is not combined with other RRC connection procedure, i.e., “RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.”. The same principal should apply to the RRC connection re-establishment on the NES cell. Hence, SIB1 request to the NES cell during RRC connection re-establishment procedure may cause more delays in re-establishment completion.

Proposal 4:	For cell selection while T311 is running, if a candidate cell is the NES cell and indicates SIB1 is periodically broadcasted, the NES UE follows the legacy RRC connection re-establishment procedure.

Proposal 5:	For cell selection while T311 is running, if a candidate cell is the NES cell and indicates SIB1 is transmitted on-demand, the NES UE should not perform SIB1 request for the candidate cell even if the NES UE has a corresponding UL WUS configuration. The NES UE considers the cell as barred as in legacy.

### On-demand SIB1 reception

On-demand SIB1 update in NES cell

In RAN2#127 meeting, it was agreed that UE keeps SIB1 updated while on the NES cell via regular SI modification procedure. An issue that can be further discussed is how long the network should transmit the on-demand SIB1 from the network modification period, as shown in the following figure.

Figure 1. On-demand SIB1 update in the NES cell

In our view, the network can transmit the on-demand SIB1 during the whole next modification period (i.e., legacy SI update) or in a time window with length equal to the RAN1 defined time window for type 0 PDCCH monitoring occasions for on-demand SIB1 reception. Regardless of the options, the UE can assume the updated SIB1 is transmitted from the next modification period. If the UE is unable to acquire the updated SIB1, the UE should consider the NES cell as barred, because the UL WUS configuration may be obsoleted due to SI update.

Proposal 5:	RAN2 to discuss how long the updated on-demand SIB1 is transmitted from the next modification period of the NES cell. The following options are considered:

- Option 1: the updated SIB1 is transmitted during whole next modification period,

- Option 2: the updated SIB1 is transmitted during time window for type 0 PDCCH monitoring occasions.

Proposal 6:	The UE considers the NES cell as barred if the UE is unable to acquire the updated SIB1.

## Access restriction

MIB configuration for NES cell

In RAN1#118bis and RAN1#119 meetings, RAN1 agreed a specific K_SSB value is used in MIB of the NES cell as follows.

Agreement For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

Agreement On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, down-select from the following options: - Option 1: Use it to indicate frequency assistance information to search SSB for Cell A - FFS: Details on the range/granularity of the frequency assistance information - FFS: Potential impact to RAN3 - Option 2: Use it to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1 - Option 3: Above feature is not supported.

RAN1 is still discussing whether the pdcch-ConfigSIB1 can be used to identify the frequency location of the nearest CD-SSB (if K_SSB > 23 for FR1 and K_SSB > 11 for FR2 but K_SSB is not equal to 30 in FR1 or 14 in FR2) or repurposed to identify the frequency location of SSB for Cell A (if K_SSB is equal to 30 in FR1 or 14 in FR2). However, according to the RRC specification, when the UE is unable to acquire the SIB1, the UE shall consider the cell as barred regardless of K_SSB value. In addition, based on the FL summary in [3], RAN1 is likely to consider a barring mechanism by K_SSB is feasible and leave it to RAN2 for further discussion.

Observation 2:	For the NES cell, RAN1 may use the K_SSB value with 23 < K_SSB ≤ 31 for FR1 and 11 < K_SSB ≤ 14 for FR2.

Observation 3:	Regardless of K_SSB value in MIB, the UE shall consider the cell as barred if the UE cannot acquire the SIB1 from the cell.

Based on the above observations, the legacy UE can be barred by the K_SSB value with ‘no SIB1 indication’ (i.e., K_SSB > 23 for FR1 and K_SSB > 11 for FR2). It means there is no additional barring bit for the legacy UE handling. For Rel-19 NES UE, it is straightforward that Rel-19 NES UE is also barred by the K_SSB value by ‘no SIB1 indication’ as in legacy if UL WUS configuration for the NES cell is not available.

Proposal 7:	RAN2 to confirm K_SSB value in MIB with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, can bar the legacy UEs. No need to introduce a separate bar indication.

Proposal 8:	Rel-19 NES UE follows the legacy barring mechanism, if the NES UE does not have a UL WUS configuration corresponding to the NES cell.

# Conclusion

In this contribution, the following proposals and observations were made:

UL WUS configuration by RRC release message

Proposal 1:	RAN2 to confirm the working assumption, i.e. UL-WUS configuration in RRC release is not supported.

Contents of UL WUS configuration

Proposal 2:	The parameters agreed by RAN1 to be included in UL WUS configuration are included in the new SIB for UL WUS configuration.

RRC connected mode

Observation 1:	SIB1 request by a dedicated signaling in connected mode is not pursued.

Proposal 4:	For cell selection while T311 is running, if a candidate cell is the NES cell and indicates SIB1 is periodically broadcasted, the NES UE follows the legacy RRC connection re-establishment procedure.

Proposal 5:	For cell selection while T311 is running, if a candidate cell is the NES cell and indicates SIB1 is transmitted on-demand, the NES UE should not perform SIB1 request for the candidate cell even if the NES UE has a corresponding UL WUS configuration. The NES UE considers the cell as barred as in legacy.

On-demand SIB1 update in NES cell

Proposal 5:	RAN2 to discuss how long the updated on-demand SIB1 is transmitted from the next modification period of the NES cell. The following options are considered:

- Option 1: the updated SIB1 is transmitted during whole next modification period,

- Option 2: the updated SIB1 is transmitted during time window for type 0 PDCCH monitoring occasions.

Proposal 6:	The UE considers the NES cell as barred if the UE is unable to acquire the updated SIB1.

MIB configuration for NES cell

Observation 2:	For the NES cell, RAN1 may use the K_SSB value with 23 < K_SSB ≤ 31 for FR1 and 11 < K_SSB ≤ 14 for FR2.

Observation 3:	Regardless of K_SSB value in MIB, the UE shall consider the cell as barred if the UE cannot acquire the SIB1 from the cell.

Proposal 7:	RAN2 to confirm K_SSB value in MIB with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, can bar the legacy UEs. No need to introduce a separate bar indication.

Proposal 8:	Rel-19 NES UE follows the legacy barring mechanism, if the NES UE does not have a UL WUS configuration corresponding to the NES cell.

# References

- R2-2410911, Report from session on V2X/SL, R19 NES and MOB

- RP-243297

- R1-2410679