3GPP TSG-RAN WG2 Meeting #129	R2-2500521

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.0

Source: 	CATT

Title: 	Discussion on CT1 LS in R2-2500004/C1-247193

Work Item:	 	Discussion and Decision

# 1.	Introduction

This contribution discusses the necessary RAN2 specification impact due to LS from CT1 in [1]:

1. Overall Description: With the work item ECRATU (Enhancement of controlling RAT utilization) CT1 is introducing an optional feature for the network to indicate RAT utilization restriction information to the UE with EMM and 5GMM procedures. The network could further indicate whether a configured RAT restriction is applicable: - only for the PLMN from which it is received; or - for the PLMN from which it is received and the equivalent PLMNs. The following access technologies can be restricted: GERAN, UTRAN, E-UTRAN, satellite E-UTRAN, NG-RAN, satellite NG-RAN. E-UTRAN access technology encompasses all terrestrial E-UTRAN cells. NG-RAN access technology encompasses all terrestrial NG-RAN cells. The UE will store the received RAT restriction in the list of "PLMNs with associated RAT restrictions", which is stored in non-volatile memory. The UE shall not consider any PLMN/access technology combination for which a RAT restriction is configured in the list of "PLMNs with associated RAT restrictions" as a candidate PLMN/access technology for PLMN selection purpose. The above aspects are taken care of in the current or future versions of TS 23.122, TS 24.301 and TS 24.501. The following aspect is not yet covered in any stage-3 specification and CT1 would like to involve RAN2: If an access technology is restricted in a PLMN, the UE shall not select any cell of the PLMN deployed in the restricted access technology during cell selection and re-selection for normal service. The restrictions are not applicable if the UE is selecting a cell for limited service. 2. Actions: To RAN WG2 group. ACTION: CT1 kindly asks RAN2 to discuss the aspects not yet covered in any 3GPP specification and check if any changes in their specifications are required and provide feedback to CT1.

2.	Discussion on the LS

As indicated in [1], CT1 introduced a list of "PLMNs with associated RAT restrictions", with the intention to prevent the UE selecting any cell of the PLMN deployed in the restricted RAT during cell selection and (re)selection procedure.

To support this RAT restriction list introduced by CT1, we find the following necessary RAN2 specification change for cell selection and reselection procedure:

- The associated restricted RAT list, if configured to the UE, needs to be additionally considered among the conditions to determine a suitable cell. Specifically, during cell (re)selection procedure, a cell is considered as a suitable cell of a PLMN, only if it is not deployed in the restricted RAT associated with the PLMN. This change of suitable cell definition applies to the cell (re)selection procedure of all the RATs concerned by the LS in [1].

  - Since the criteria of being a suitable cell is impacted, RAN2 can further discuss whether the highest ranked cell/best cell and other cells on the same frequency need to be excluded from candidates for cell reselection, if the highest ranked cell/best cell on this frequency is considered as not suitable due to being deployed in the associated restricted RAT of a PLMN.

- UE NAS needs to maintain the list of "PLMNs with associated RAT restrictions" and inform it to the UE AS for the determination of suitable cells. This needs to be captured in the Table of "Function division between NAS and AS in idle mode" (at least for NR, E-UTRAN and UTRA Specs).

Note that the impacts on suitable cell definition, as shown in above bullet a), do not prevent the UE from selecting acceptable cells for limited service, if it cannot find any suitable cell during cell (re)selection. Hence, the proposed change aligns with what is required by CT1 in the LS that "The restrictions are not applicable if the UE is selecting a cell for limited service" [1].

Based on the above observations, our proposals are given as follows:

Proposal 1: For a UE configured with the list of "PLMNs with associated RAT restrictions", a cell of a PLMN is considered as a suitable cell, only if it is not deployed in the restricted RAT associated with the PLMN.

Proposal 1a: For cell reselection, RAN2 discusses whether the highest ranked cell/best cell of a frequency and other cells of the same frequency shall not be considered as candidates for a maximum of 300 seconds, if the highest ranked cell/best cell is considered as not suitable due to being deployed in the associated restricted RAT of a PLMN.

Proposal 2: UE NAS maintains the list of "PLMNs with associated RAT restrictions" and informs it to the UE AS for suitable cell determination. Clarify this in the "Functional division between NAS and AS" at least in NR, E-UTRA and UTRA Specs.

In the CT1 LS, the E-UTRAN and NG-RAN are respectively separate from satellite E-UTRAN and satellite NG-RAN, which basically means a need to distinguish whether a cell is a TN cell or NTN cell, if detected in E-UTRA or in NR. For NR, such cell type distinction can already be supported by a UE based on the current specification [2]:

The UE can determine the network type (terrestrial or non-terrestrial) implicitly by the existence of cellBarredNTN in SIB1.

For E-UTRAN, since there is also an NTN specific barring bit (i.e. cellBarred-NTN [3]) introduced for IoT NTN, UE can rely on a similar way to distinguish whether a cell is an E-UTRAN TN cell or an IoT NTN cell (though not specified clearly in Stg.2 Spec as in NR). Actually, based on the current NR and E-UTRA Spec, there are some other means on which the UE can still rely to distinguish whether a cell belongs to TN or NTN (e.g. relying on whether essential SI for NR/IoT NTN is scheduled by the cell or not).

Therefore, current specifications can already support the UE to distinguish whether an NR/E-UTRAN cell detected is a satellite NR/E-UTRAN cell or a normal TN cell. No specification impact is needed for such distinction.

Proposal 3: RAN confirms that current specification can already enable the UE to distinguish whether an NG-RAN/E-UTRAN cell is a satellite NG-RAN/E-UTRAN cell or normal NG-RAN/E-UTRAN cell. No specification impact is needed for such distinction.

# 3.	Conclusion

The necessary RAN2 specification impacts due to CT1 LS in [1] are discussed, with the following proposals:

Proposal 1: For a UE configured with the list of "PLMNs with associated RAT restrictions", a cell of a PLMN is considered as a suitable cell, only if it is not deployed in the restricted RAT associated with the PLMN.

Proposal 1a: For cell reselection, RAN2 discusses whether the highest ranked cell/best cell of a frequency and other cells of the same frequency shall not be considered as candidates for a maximum of 300 seconds, if the highest ranked cell/best cell is considered as not suitable due to being deployed in the associated restricted RAT of a PLMN.

Proposal 2: UE NAS maintains the list of "PLMNs with associated RAT restrictions" and informs it to the UE AS for suitable cell determination. Clarify this in the "Functional division between NAS and AS" at least in NR, E-UTRA and UTRA Specs.

Proposal 3: RAN confirms that current specification can already enable the UE to distinguish whether an NG-RAN/E-UTRAN cell is a satellite NG-RAN/E-UTRAN cell or normal NG-RAN/E-UTRAN cell. No specification impact is needed for such distinction.

# 4.	Reference

- R2-2500004	UE usage of the RAT restrictions (C1-247193; contact: Apple)	CT1

- TS 38.300

- TS 36.331