3GPP TSG-RAN WG2 Meeting #129	R2-2500524

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.8.2

Source: 	CATT

Title: 	Further discussion on downlink coverage enhancements

Document for: 	Discussion and Decision

Introduction

This contribution discusses possible RAN2 impact of DL coverage enhancements.

Discussion

2.0	Potential RAN2 scope based on latest WID

The WID for NR_NTN_Ph3 was updated and approved in [1] in RAN#106 as follows:

According to the latest WID, our understanding is that potential discussion on cell/beam level DRX/DTX (which belongs to the removed bullet for the consideration of R18 NES techniques) is not within Rel-19 NR NTN phase 3 WI scope anymore. RAN2 at least needs to confirm whether this is the common understanding before carrying out any further discussions on this aspect.

Proposal 0: RAN2 clarifies whether cell/beam level DRX/DTX is still within the scope of Rel-19 NR NTN phase 3 WI according to the latest WID (considering the NES related description already removed from the WID).

2.1	Discussion on possible SMTC enhancements

At last meeting, the SMTC configuration for DL CE was discussed; there is an FFS on whether to introduce separate SMTC list for DL CE cell. The intention is to support different SMTC periodicities with corresponding SMTC configurations.

Agreements:

1.	As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

2.	RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

Companies proposing this SMTC enhancement for UE power saving mainly proposed to introduce SSB specific or location based SMTC configurations, adapting to the beam hopping pattern.

Before RAN2 makes a decision to support such kind of location based/SSB associated SMTC enhancements for DL CE purpose, the following aspects need be carefully considered:

- Technical benefit/gain. Proponents argued that with beam hopping, the SSBs of the neighbour cells may not be covered by the per-frequency SMTC configuration, but spread into different time windows, so that the UE can select to only measure the SSBs in the time window where the SSB(s) covering the UE are located. However, since RAN1 does not conclude anything about the beam hopping pattern (except for the SSB periodicity extension), the typical deployment on the SSB/SMTC in NTN with beam hopping is still unclear. This makes it unclear whether/how the SMTC configuration in Rel-19 NTN is different from legacy NR SMTC configuration, especially whether it aligns with the SSB/SMTC deployment assumed by the proponents, and also unclear how many SMTCs the UE eventually needs to measure. As a result, how much power saving gain can be achieved by this SMTC enhancement proposed, or if there is really any, is even less clear.

- Complexity and impacts to other WGs. It is straightforward that the power saving gain is at the sacrifice of measurement performance, as now it allows the UE to not measure all the SMTCs configured by the NW. Also, considering that there have already been the multiple SMTC supported in the current Spec, aiming to deal with the propagation delays of different satellites, further supporting such location based/SSB associated SMTC on top of the existing multiple SMTC mechanism can be rather complicated. Whether such RRM performance degradation as well as the extra complication can lead to potential impacts to RAN4 design may need be carefully considered. So whether this location based/SSB associated SMTC mechanism is acceptable also need be confirmed by RAN4.

- Out of WI scope or not. Per the latest WID [1], only enhancements related to extended 160 ms SSB periodicity is left within the scope of the DL coverage enhancements objective. Strictly speaking, such location based/SSB associated SMTC mechanism stems from the beam hopping and is tightly depends on the beam hopping pattern, instead of the SSB periodicity extension itself. Since whether other beam hopping enhancement than SSB periodicity extension is within the WI scope is not clear, it is unknown whether RAN1 is going to resume the discussion on and clarify to us the specific beam hopping pattern, and thus also unclear whether this discussion on SMTC is within the scope or not.

Therefore, we propose that RAN2 carefully considers and discusses the following aspects, before deciding to introduce any SMTC enhancements for power saving purpose in DL coverage enhancement work.

Proposal 1: RAN2 carefully considers and justifies the following technical aspects, before deciding to introduce any SMTC enhancement for UE power saving purpose in DL-CE work:

- Whether sufficient power saving gain can be justified;

- Whether the complexity and impacts to other WGs (e.g. RAN4) are acceptable;

- Whether such enhancement remains within the WI scope or not.

If there is positive consensus on the above aspects, we are open to further discuss the specific enhancements.

2.2	Discussion on impact of RRC procedure

The following is to discuss whether any other enhancements need to be introduced due to extended SSB periodicity of 160 ms. Note that the discussion does not involve any enhancement on cell/beam level DRX/DTX. Also note that since the extended SSB periodicity was introduced for the default SSB value, which impacts only UEs' access from RRC_IDLE/INACTIVE, the following will analyse the SI/paging/RACH related aspects.

1) System information

According to current spec [2], when the period of SSB is larger than 80 ms, the MIB is transmitted with the same periodicity as SSB, i.e., if the SSB periodicity is 160 ms, the MIB is transmitted with periodicity of 160 ms too. No spec impact is needed.

- the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms (TS 38.212 [17], clause 7.1) and it includes parameters that are needed to acquire SIB1 from the cell. The first transmission of the MIB is scheduled in subframes as defined in TS 38.213 [13], clause 4.1 and repetitions are scheduled according to the period of SSB; NOTE 1: If the period of SSB is larger than 80 ms, the MIB is transmitted with the same periodicity as that of SSB.

Observation 1: Current TS 38.331 supports the MIB broadcasted with the same periodicity as SSB when the periodicity of SSB is larger than 80 ms.

The scheduling of SIB1 relies on the SSB and CORESET multiplexing pattern, it is up to RAN1’s design to ensure that the SIB1 periodicity under different multiplexing patterns could match the SSB periodicity of 160 ms. RAN2 could wait for RAN1’s progress.

Observation 2: It is up to RAN1 whether any change is needed for the resource configuration used for SIB1 scheduling.

- the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13).

Other SI is scheduled with the SI-window mechanism. The SI-window is determined with parameters of si-WindowLength and si-Periodicity. As a reference to the current parameter value range, the SI-window can be configured to adapt to the SSB periodicity by network implementation. For example: the si-Periodicity is configured as rf16 or multiples of rf16 and the si-WindowLength is configured with a suitable value to ensure the transmission of system information to be completed during the beam coverage time of the serving cell. As another possible configuration, the SI window length can be configured with 160 ms or multiples of 160 ms, which can be supported based on current configuration as well.

Hence based on current RRC spec [2], suitable configuration for SI scheduling can adapt to the 160 ms SSB periodicity, and no extra Spec impact is foreseen from RAN2 perspective, unless RAN1 requires any enhancements.

SI-SchedulingInfo ::= SEQUENCE { schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo, si-WindowLength ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280, s2560-v1710, s5120-v1710 }, si-RequestConfig SI-RequestConfig OPTIONAL, -- Cond MSG-1 si-RequestConfigSUL SI-RequestConfig OPTIONAL, -- Cond SUL-MSG-1 systemInformationAreaID BIT STRING (SIZE (24)) OPTIONAL, -- Need R ... } SchedulingInfo ::= SEQUENCE { si-BroadcastStatus ENUMERATED {broadcasting, notBroadcasting}, si-Periodicity ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512}, sib-MappingInfo SIB-Mapping }

Observation 3: By network implementation, the SI scheduling in the current Spec can adapt to the SSB periodicity of 160 ms.

2) PCCH-Config

As specified in [3], the DRX cycle is determined by the shortest of the UE specific DRX value configured by RRC (if any), the UE specific DRX value configured by upper layers (if any) and a default DRX value broadcast in system information. The value range of all these DRX values is {rf32, rf64, rf128, rf256}. Furthermore, the paging frame and offset are configured with the parameter nAndPagingFrameOffset. When this parameter is set to oneSixteenthT, the PF period equals to 16rf. The paging configuration on paging cycle and PF can fulfil the needs for extended SSB periodicity of 160 ms, so that enhancement is not needed from RAN2 perspective.

nAndPagingFrameOffset CHOICE { oneT NULL, halfT INTEGER (0..1), quarterT INTEGER (0..3), oneEighthT INTEGER (0..7), oneSixteenthT INTEGER (0..15) },

Observation 4: The PCCH-Config in the current Spec can adapt to the extended SSB periodicity of 160 ms.

3) RACH configuration

The RACH occasion is configured per SSB with the parameter of ssb-perRACH-OccasionAndCB-PreamblesPerSSB and the mapping relationship of SS/PBCH block indexes to PRACH occasions is defined in [4]. It is up to RAN1 to check whether any enhancement is needed for PRACH related parameters due to extended SSB periodicity.

ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { oneEighth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneFourth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneHalf ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, one ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, two ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32}, four INTEGER (1..16), eight INTEGER (1..8), sixteen INTEGER (1..4) } OPTIONAL, -- Need M

Observation 5: It is up to RAN1 to check whether any enhancement is needed for PRACH configuration.

4) Common SearchSpace configuration

The NW configures some common search spaces for UE (including searchSpaceOtherSystemInformation, pagingSearchSpace and ra-SearchSpace), so that the UE can determine the time domain resource of PDCCH monitoring occasions. When SearchSpaceID = 0 is configured for UE, the UE follows the predefined tables in [4], and it is up to RAN1 to check whether any enhancement is needed to for SSB periodicity 160 ms. When SearchSpaceID other than 0 is configured for these parameters, the UE applies the configurations provided via RRC signalling. After checking the current spec, we think the current value range can fulfil the possible requirement caused by the extended SSB periodicity of 160 ms (since the monitoring periodicity of search space can be up to 2560 ms for SCS 15 kHz and 320 ms for SCS 120 kHz). It seems there is no enhancement required on the configuration of search space from RAN2  perspective.

monitoringSlotPeriodicityAndOffset CHOICE { sl1 NULL, sl2 INTEGER (0..1), sl4 INTEGER (0..3), sl5 INTEGER (0..4), sl8 INTEGER (0..7), sl10 INTEGER (0..9), sl16 INTEGER (0..15), sl20 INTEGER (0..19), sl40 INTEGER (0..39), sl80 INTEGER (0..79), sl160 INTEGER (0..159), sl320 INTEGER (0..319), sl640 INTEGER (0..639), sl1280 INTEGER (0..1279), sl2560 INTEGER (0..2559) } OPTIONAL, -- Cond Setup4

Observation 6: The current common search space configuration provided via RRC signalling can adapt to the extended SSB periodicity of 160 ms.

Based on the above observations, we found no enhancement needed from RAN2 perspective due to SSB extension to 160 ms, w.r.t. SI, paging, RACH related aspects (unless any enhancement is required by RAN1). We propose this as follows:

Proposal 2: From RAN2’s perspective, the current RAN2 spec can support the extended SSB periodicity of 160 ms. No enhancement on SI-scheduling, PCCH-Config, RACH configuration and Common SearchSpace is foreseen (unless required by RAN1).

Conclusion

In this contribution, DL coverage enhancement is further discussed with the corresponding observations and proposals listed below:

Observation 1: Current TS 38.331 supports the MIB broadcasted with the same periodicity as SSB when the periodicity of SSB is larger than 80 ms.

Observation 2: It is up to RAN1 whether any change is needed for the resource configuration used for SIB1 scheduling.

Observation 3: By network implementation, the SI scheduling in the current Spec can adapt to the SSB periodicity of 160 ms.

Observation 4: The PCCH-Config in the current Spec can adapt to the extended SSB periodicity of 160 ms.

Observation 5: It is up to RAN1 to check whether any enhancement is needed for PRACH configuration.

Observation 6: The current common search space configuration provided via RRC signalling can adapt to the extended SSB periodicity of 160 ms.

Proposal 0: RAN2 clarifies whether cell/beam level DRX/DTX is still within the scope of Rel-19 NR NTN phase 3 WI according to the latest WID (considering the NES related description already removed from the WID).

Proposal 1: RAN2 carefully considers and justifies the following technical aspects, before deciding to introduce any SMTC enhancement for UE power saving purpose in DL-CE work:

- Whether sufficient power saving gain can be justified;

- Whether the complexity and impacts to other WGs (e.g. RAN4) are acceptable;

- Whether such enhancement remains within the WI scope or not.

Proposal 2: From RAN2’s perspective, the current RAN2 spec can support the extended SSB periodicity of 160 ms. No enhancement on SI-scheduling, PCCH-Config, RACH configuration and Common SearchSpace is foreseen (unless required by RAN1).

Reference

- RP-243300 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

- 3GPP TS 38.304: "User Equipment (UE) procedures in Idle mode and RRC Inactive state"

- 3GPP TS 38.213: "Physical layer procedures for control"