3GPP TSG-RAN WG2 Meeting #129	 R2-2500526

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.8.6

Source: 	CATT

Title: 	Discussion on redirection from LTE TN to NR NTN

Document for: 	Discussion and Decision

Introduction

In RAN2 #128, the issue on redirection to NR NTN from LTE TN was discussed and agreements were made as follows [1]:

R2-2410047 Frequency priorities and redirection from LTE to NR NTN Qualcomm Incorporated discussion Rel-19 LTE_TN_NR_NTN_mob Proposal 1 Clarify that the UE ignores the dedicated frequency priority for NR NTN frequency that is associated with satellite ID. Proposal 2 Clarify that the EUTRA network does not include NR NTN frequency in redirectedCarrierInfo in the RRCConnectionRelease message. - QC thinks that at least for legacy UEs (R17 and R18) we cannot have any different behaviour, but we can discuss for R19 - Samsung thinks basic redirection should be supported without any capability. But we can discuss capability for enhancements. Xiaomi agrees. - HW thinks that redirection is not part of this WI. LG also agrees - ZTE wonders about the use case. - HW wonders where we would put the UE capability in case we introduce it. Samsung thinks this would be conveyed in the NR capability. QC/CATT do not agree. RAN2 understands that the UE behaviour is currently not clear if the network includes a NR NTN frequency in redirectedCarrierInfo in the RRCConnectionRelease message in a TN cell. We can further discuss whether we want to clarify the UE behaviour and in case whether we need to introduce a new UE capability.

This contribution addresses this FFS left on “whether we want to clarify the UE behaviour and in case whether we need to introduce a new UE capability”, and propose candidate solutions.

Discussion

At last meeting, some companies pointed out that the reference of TS38.101-5 [4] for NR NTN is not captured in TS 36.306 [3] (as shown below), and this prevents the UE from reporting the supported NR NTN band to eNB which is then unable to redirect the UE to an NR NTN frequency:

- TS 36.306 - 4.3.34.3 supportedBandListNR-SA-r15 This field indicates whether UE supports standalone NR, as specified in TS 38.331 [35], and includes the supported NR bands as defined in TS 38.101-1 [33] and TS 38.101-2 [34]. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [36].

A similar issue may exist in TS 36.331 [2] where the reference (i.e. [85]) actually only points to TS 38.101-1:

#### – FreqBandIndicatorNR The IE FreqBandIndicatorNR indicates the NR operating band as defined in TS 38.101 [85]. FreqBandIndicatorNR information element -- ASN1START FreqBandIndicatorNR-r15 ::= INTEGER (1.. maxFBI-NR-r15) -- ASN1STOP

In order to make the redirection functionality to NR NTN from LTE TN clear, there can be the following options taken into consideration by RAN2:

- Option 1: Correct TS 36.306/TS 36.331 since Rel-17 to enable UE capability reporting of supported NR NTN band (i.e. add reference TS 38.101-5 for NR NTN bands in TS 36.306/TS 36.331), and further clarification on UE behavior in spec is not needed.

In this option, it is based on the assumption that the Rel-17 LTE UE, which supports also NR NTN, can perform cell reselection measurement on NR NTN frequency without NR satellite assistance information, when it camps on LTE TN according to TS 36.304. From the UE perspective, it has the ability to support redirection to NR NTN from LTE TN. The correction does not require Rel-17 UE to support more capability like the IDLE capability we are introducing now for Rel-19. Hence it is feasible to provide the correction from Rel-17 for TS 36.306 and TS 36.331 to clarify that UE capability report for NR NTN bands is supported, e.g.:

- TS 36.306 - 4.3.34.3 supportedBandListNR-SA-r15 This field indicates whether UE supports standalone NR, as specified in TS 38.331 [35], and includes the supported NR bands as defined in TS 38.101-1 [33] ,and TS 38.101-2 [34] and TS 38.101-5 [xx]. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [36].

#### – FreqBandIndicatorNR The IE FreqBandIndicatorNR indicates the NR operating band as defined in TS 38.101 [85] and TS 38.101-5 [xx]. FreqBandIndicatorNR information element -- ASN1START FreqBandIndicatorNR-r15 ::= INTEGER (1.. maxFBI-NR-r15) -- ASN1STOP

Based on this option, when UE receives a dedicated NR NTN frequency RedirectedCarrierInfo:

- If the UE supports the Rel-19 LTE TN to NR NTN IDLE mode mobility enhancement and the eNB provides the NR NTN satellite assistance information in SIB24 and SIB33, the UE performs cell reselection measurement according to TS36.304 by using the satellite assistance information (with how to use the assistance information left to UE implementation without need of specified UE behavior).

- If the UE does not support the Rel-19 LTE TN to NR NTN IDLE mode mobility enhancement (including the Rel-17/18 UEs) or the eNB does not provide the NR NTN satellite assistance information in SIB24 and SIB33, the UE may perform cell reselection measurement according to TS36.304 without NR NTN satellite assistance information.

No further clarification on UE behavior is needed in the spec, depending on the IDLE mode capability of the UE's own as shown in above two bullets.

- Option 2: Admit that redirection to NR NTN from LTE TN is not supported before Rel-19 pursuing no Rel-17/18 spec change, and make necessary enhancement (i.e. add reference TS 38.101-5 for NR NTN bands in TS 36.306/TS 36.331 and/or include satellite ID in RedirectedCarrierInfo) to support this feature from Rel-19.

Due to the reason that a Rel-17/18 LTE UE cannot report supported NR NTN band per current TS 36.306/TS 36.331, we may have to admit that redirection to NR NTN from LTE TN is currently not supported if no Rel-17/18 Spec change is pursued. This is with the assumption that network shall not configure UE to redirect to NR NTN, as it cannot get the NR NTN capability of the UE per the current Spec. A confirmation on the chair notes may be needed, if companies want to follow this way.

Then, we could provide correction to support redirection to NR NTN since Rel-19 in this option. Considering the discussion in previous meetings, the following corrections could be taken into consideration:

    - Option 2-a: Only add the reference of TS38.101-5 in TS 36.306/TS 36.331 since Rel-19 (with no additional capability signaling needed)

      - In this way, the Rel-19 UE will follow the two-bullet behaviours as clarified for Option 1, when it supports or does not support the Rel-19 LTE TN to NR NTN IDLE mode capability of using the NR NTN satellite assistance info in SIB33.

    - Option 2-b: Introduce a new capability signaling for LTE TN to NR NTN redirection purpose and also a satellite ID(s) in RedirectedCarrierInfo.

      - Note that in the endorsed TS 36.306 CR we only introduced one new UE capability without signaling for this LTE TN to NR NTN IDLE mob feature as agreed in RAN2 #127.

- Option 3: Confirm redirection to NR NTN from LTE TN is not supported.

If no consensus can be made to above option 1 or 2, we may have to admit that redirection from LTE TN to NR NTN is not supported, mainly due to the reason that no NR NTN band can be reported to the eNB which then assumes the UE does not support NR NTN. Following this option, RAN2 may further discuss whether any restriction in TS 36.331 is needed to prevent an NR NTN frequency being configured in RRCConnectionRelease for redirection or only a clarification in the chair notes is sufficient.

As a summary, we propose that RAN2 addresses this redirection issue by down-selecting above options.

Proposal 1: To resolve the remaining issue of redirection from LTE TN to NR NTN, RAN2 down-selects the following options:

  - Option 1: Correct TS 36.306/TS 36.331 since Rel-17 to enable UE capability reporting of supported NR NTN band (i.e. add reference TS 38.101-5 for NR NTN bands in TS 36.306/TS 36.331), without Spec change on UE behavior.

  - Option 2: Admit that redirection to NR NTN from LTE TN is not supported before Rel-19 pursuing no Rel-17/18 spec change, and make necessary enhancement to support this feature from Rel-19 with either sub-option below:

  - Option 2-a: Only add the reference of TS38.101-5 in TS 36.306/TS 36.331 since Rel-19 (with no additional capability signaling needed);

  - Option 2-b: Introduce a new capability signaling for LTE TN to NR NTN redirection purpose and also a satellite ID(s) in RedirectedCarrierInfo.

  - Option 3: Confirm redirection to NR NTN from LTE TN is not supported. FFS whether to introduce any restriction in TS 36.331 (e.g. NR NTN frequency cannot be configured in RRCConnectionRelease) or only clarify this in the chair notes.

Conclusion

In this contribution, the issue of redirection to NR NTN is discussed, with the following proposal:

Proposal 1: To resolve the remaining issue of redirection from LTE TN to NR NTN, RAN2 down-selects the following options:

  - Option 1: Correct TS 36.306/TS 36.331 since Rel-17 to enable UE capability reporting of supported NR NTN band (i.e. add reference TS 38.101-5 for NR NTN bands in TS 36.306/TS 36.331), without Spec change on UE behavior.

  - Option 2: Admit that redirection to NR NTN from LTE TN is not supported before Rel-19 pursuing no Rel-17/18 spec change, and make necessary enhancement to support this feature from Rel-19 with either sub-option below:

  - Option 2-a: Only add the reference of TS38.101-5 in TS 36.306/TS 36.331 since Rel-19 (with no additional capability signaling needed);

  - Option 2-b: Introduce a new capability signaling for LTE TN to NR NTN redirection purpose and also a satellite ID(s) in RedirectedCarrierInfo.

  - Option 3: Confirm redirection to NR NTN from LTE TN is not supported. FFS whether to introduce any restriction in TS 36.331 (e.g. NR NTN frequency cannot be configured in RRCConnectionRelease) or only clarify this in the chair notes.

Reference

- Draft_RAN2_128_Meeting_Report_v1

- 3GPP TS 36.331: "Universal Terrestrial Radio Access (UTRAN); Radio Resource Control (RRC)"

- 3GPP TS 36.306:"Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities"

- 3GPP TS 38.101-5:"NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements"