3GPP TSG-RAN WG2 Meeting #129	R2-2500528

Athens, Greece, February 17th – 21st, 2025

Source:	CATT

Title:	Further consideration on UL capacity enhancements

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In RAN2#128 meeting [1], agreements were made for UL capacity enhancements as follows:

Agreements:

1.	Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

2.	Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

3.	The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

4.	There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

5.	The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

6.	There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

7.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

9.	The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

11.	Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

12.	RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

In this document, we further discuss the remaining issues of UL capacity enhancements in IoT NTN, with our observations and proposals provided.

Discussion

Terminology

In order to facilitate discussion in this document, the following terminologies are used for CB-Msg3 EDT in this document.

- CB-Msg3-EDT Occasion: the time occasion to transmit CB-Msg3. In the time domain, it corresponds to UL transmissions in N consecutive UL slots with N = NRepNslots, where NRep is the repetition number which can be determined by the CE level (NRep=2 in Figure 1), and Nslots is the length of basic resource unit, e.g. slot number within one RU in NB-IoT. In frequency domain, the maximum frequency resource number is 4 in Figure 1.

- Replica: a number of consecutive transmissions within a CB-Msg3-EDT occasion according to the CE level. The number of the consecutive transmissions is NRep. Each replica covers consecutive resources in time domain for repetitions (as determined by RSRP for intended CE level).

- CB-Msg3 window: the timer duration in which the UE monitors PDCCH for network response for CB-Msg3.

- CB-Msg3-RNTI: The RNTI used to scramble PDCCH to schedule network response for CB-Msg3.

Figure 1: Example for illustration of CB-Msg3-EDT occasion

Initiation

It has been agreed that existing CE level selection procedure will be reused for CB-Msg3 at least for the initial selection. But it is still FFS on whether the same thresholds are used. From our perspective, DSA is used as one complementary solution to increase the UL capacity for IoT NTN, but not a new mechanism used to further improve the UL coverage. Hence, it is not necessary to introduce new CE levels or new RSRP thresholds for CB-Msg3.

Proposal 1: Reuse the existing CE levels and RSRP thresholds for CB-Msg3 procedure.

For the size of pending UL data, it has been agreed that only if it is less than the configured maximum TBS, can the UE trigger CB-Msg3 EDT procedure. Basically, the size of the pending data is more related to the traffic. It can be seen that the EDT TBS range for both eMTC and NB-IoT are the same. Furthermore, the TBS configured for EDT ranges from 328 bits to 1000 bits for both eMTC and NB-IoT based on current specification [2] which cover the possible cases for UL transmission. Therefore, the maximum TBS for CB-Msg3 can be configured the same as in legacy EDT (i.e. per CE level).

Proposal 2: The maximum TBS can be configured in the same way as the legacy EDT (i.e. per CE level).

In EDT, apart from RSRP and data volume, the UE also needs to decide whether it is allowed to initiate EDT via checking the information in SIB2(-NB), e.g., cp-EDT(-5GC), up-EDT(-5GC). For CB-Msg3, SIB2(-NB) can be used to include the CB-Msg3 related configuration. Considering that the CB-Msg3 specific configuration is needed to include e.g. resource pool configuration, number of replicas for DSA, etc., the UE considers the eNB supporting CB-Msg3 if the configuration is present.

Proposal 3: CB-Msg3 specific configuration is included in one field contained in SIB2(-NB). The presence of this CB-Msg3 specific field indicates that the UE is allowed to initiate CB-Msg3 EDT in the cell.

Resource allocation for CB-Msg3 EDT

In this section, we discuss the remaining aspects on resource allocation mechanism for CB-Msg3 EDT with DSA. In RAN2#127bis meeting, it was assumed that CB-Msg3 EDT cell specific PUSCH resources are associated with the number of repetitions and RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission. According to the assumption, it can be seen that the CB-Msg3 PUSCH resources can be configured with association to different CE levels. From the network perspective, the network can identify the repetition number based on the PUSCH resource where the UE actually used for transmission. Considering that the PUSCH resource will be shared among multiple UEs to perform CB-Msg3 transmission, it is a pool of PUSCH resources that is actually needed for each CE level. This is proposed as follows:

Proposal 4: Introduce per CE level resource pools which include the CB-Msg3 EDT cell specific PUSCH resources. UE selects the resource pool associated with the intended CE level, according to the RSRP criterion.

Regarding the resource structure in the resource pool, it can be configured like PUR, for example defining the periodicity and timing offset. However, considering that multiple UEs will contend for the resources to perform the transmission of CB-Msg3 EDT, the resource density may be different from PUR. Anyway, the details are in RAN1 scope on how the physical resource structure should be designed in the resource pool for CB-Msg3 EDT.

Proposal 5: The details on physical resource structure of CB-Msg3 EDT in the resource pool (e.g. whether to reuse the existing RU in NB-IoT or design other resource granularity) are left to RAN1.

Once the UE determines a CE level for CB-Msg3 EDT, the UE selects the resources for the replica transmissions in the resource pool associated with the selected CE level.

Proposal 5a: UE selects the PUSCH resources for replica transmissions in the resource pool associated with the selected CE level.

We assume that there is no intention to introduce any uplink control signaling to indicate the resource allocation related information to the eNB for the decoding of a replica. To this end, besides the configuration of PUSCH resources themselves, we think the following parameters should also be configured at per CE level and thus associated with the corresponding resource pool:

- The number of repetitions should be configured for each CE level and thus associated with the corresponding resource pool;

- An MCS should be configured for each CE level and thus associated with the corresponding resource pool.

Then, for the decoding of a replica, when the eNB receives Msg3 on a PUSCH resource from a resource pool, it can determine that the MCS and the number of repetitions (for combination) associated with the resource pool are used for the replica's transmission, and performs the decoding accordingly.

Proposal 6: Each CE-level is configured with a "number of repetition" which is associated to the resource pool of the corresponding CE-level.

Proposal 6a: Each CE-level is configured with an MCS level, which is associated to the resource pool of the corresponding CE-level.

Proposal 6b: For a CB-Msg3 EDT transmission, UE applies the "number of repetition" and MCS associated with the selected CE level.

It was already agreed "Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection". Considering that the UE’s movement is not rapid w.r.t. the NTN cell coverage, we can assume, during the transmission of each Replica, the CE level is not changed, so upon the initiation of CB-Msg3, the UE does not need to choose CE level again before each replica transmission.

Proposal 7: The CE level selected upon the initiation of CB-Msg3 EDT will be used for each replica transmission for this CB-Msg3 EDT procedure.

CB-Msg3 transmission

Once the resource has been selected, the UE can perform CB-Msg3 transmission. The UE can deliver the corresponding UL grant, HARQ information to the HARQ entity. Each grant is still corresponding to the transmission occasion of a repetition, similar to legacy UL transmission.

Proposal 8: The CB-Msg3 transmission procedure follows the legacy mechanism with grant based handling.

Network response reception

### 2.5.1	Window handling for NW response reception

For multiple replica transmission, it is still unclear how to maintain CB-Msg3 window to guarantee that the UE can monitor PDCCH for the response of each replica. That is, it is still FFS on whether one or multiple (i.e. one for each replica) PDCCH monitoring window(s) will be used for PDCCH monitoring. Considering that the network does not know which CB-Msg3-EDT occasion is selected for different replica and whether subsequent replica transmission is performed, the following options are provided.

Option 1: CB-Msg3 window is started for each Replica transmission respectively (i.e. multiple windows).

In this option, the UE will not consider the CB-Msg3 transmission terminated, when CB-Msg3 window expires for a replica other than the last one. The UE monitors PDCCH scrambled by the CB-Msg3-RNTI calculated according to the corresponding replica within the CB-Msg3 window per replica. In detail, the UE monitors both CB-Msg3-RNTIs when the CB-Msg3 windows of 2 adjacent replicas overlaps, and the UE does not need to monitor the PDCCH scrambled by the CB-Msg3-RNTI of a replica when the CB-Msg3 window of this specific replica expires. The network configures the timer length, without necessarily taking the replica resource interval into consideration.

Figure 2: CB-Msg3 window running in Option 1

Option 2: The UE starts only one CB-Msg3 window for all the replicas.

In this option, only one window is maintained. Since there is no boundary of each replica for PDCCH monitoring, when the CB-Msg3-window overlaps, the UE has to monitor the CB-Msg3-RNTIs for all the replicas. This brings extra complexity to the procedure of a "blind" PDCCH detection.

Figure 3: CB-Msg3 window running in Option 2

Comparing these 2 options, it can be found that Option 1 is friendlier to blind detection procedure, especially when the CB-Msg3-window overlaps. Therefore, it is proposed to handle the window for NW response as follows.

Proposal 9: A CB-Msg3 window is started for each Replica transmission respectively, and the UE only monitors PDCCH scrambled with the corresponding CB-Msg3-RNTI for the corresponding replica within the CB-Msg3 window.

Proposal 10: UE considers the CB-Msg3 transmission failure upon the CB-Msg3 window expires for the last replica.

It has been agreed that UE starts a window for NW response reception taking UE-eNB RTT into account after the end of all repetition of CB-Msg3 PUSCH transmission. For DSA case, the CB-Msg3 window should also be started after the end of all repetition of CB-Msg3 PUSCH transmission of the each replica taking UE-eNB RTT into account.

Proposal 11: For DSA case, after the end of all repetition of CB-Msg3 PUSCH transmission of each replica, UE starts the corresponding window for NW response reception, taking UE-eNB RTT into account.

### 2.5.2	RNTI calculation

It has been agreed that the RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for CB-Msg3 EDT transmission. But it is FFS on whether only one RNTI or multiple RNTIs can be used for PDCCH monitoring. Considering that the network does not know how many replicas the UE has selected, the RNTI should be computed for each replica.

Proposal 12: CB-Msg3-RNTI should be computed for each Replica respectively in CB-Msg3 EDT.

RAN1 sent LS [3] and replied to the OCC support with CB-Msg3:

The OCC solution that RAN1 is working on may be applied to CB-Msg3 NPUSCH format 1 single tone at least for OCC length 2, for the subcarrier spacing(s) for which CB-Msg3 is supported, when the power imbalance is small. The design details for the OCC solution are ongoing.

According to the LS, it can be seen that OCC may be applied to CB-Msg3 NPUSCH format single tone. Hence, the OCC index can also be taken into account to RNTI calculation.

Proposal 13: OCC index can be considered for CB-Msg3 RNTI calculation for NPUSCH format single tone at least for OCC length 2.

Contention resolution

In CB-Msg3 EDT, if the UE selects the same PUSCH/NPUSCH, there is no way to distiguish them except for contention resolution. It has been agreed that broadcast signalling is used to deliver the CB-Msg3 configuration. In this case, the UE could calcuate CB-Msg3-RNTI based on the resource associated with the PUSCH occasion used for CB-Msg3 EDT transmission. Similar to Msg4 reception, the UE monitors the PDCCH scrambled by the CB-Msg3-RNTI. Then, in order for contention resolution, a CB-Msg3 contention resolution MAC CE should be introduced and included in the DL message as a NW response to the UE. If the CB-Msg3 contention resolution MAC CE corresponds to the beginning part of the UL message (e.g. first 48 bits), the UE will consider the conention resolution for this CB-Msg3 transmission successful.

Proposal 14: For CB-Msg3 EDT, the similar mehcanism like Msg4 in RACH can be reused for contention resolution, e.g., if the beginning part (e.g. 48 bits) in the DL message matches the corresponding part of the UL message sent by the UE, contention resolution is considered successful for CB-Msg3 by the UE.

There is a need to handle the exceptional case that the UE may not receive any response from the network after the CB-Msg3 transmission. To this end, the UE just starts PDCCH monitoring for Msg4 scheduling during CB-Msg3 window. If the Msg4 corresponding to its identity is not received within the window, UE considers the CB-Msg3 transmission for corresponding replica failure; if CB-Msg3 window of the last replica expires, UE considers this CB-Msg3 transmission procedure failure, and perform the failure handling. If the CB-Msg3 EDT fails, the UE could re-attemp CB-Msg3 transmsision procedure, fallback to RA-EDT or declare CB-Msg3 transmission failure to upper layers.

Proposal 15: If the CB-Msg3 window of the last replica expires, the UE will assume this CB-Msg3 transmission procedure fails. RAN2 further discusses whether the UE re-attemps another CB-Msg3 transmission, falls back to RA-EDT or declares CB-Msg3 transmission failure to upper layers in this case.

- 3.	Conclusion

In this document, we analyse the potential impacts for UL capacity enhancements in IoT NTN, and find the following and proposals:

Proposal 1: Reuse the existing CE levels and RSRP thresholds for CB-Msg3 procedure.

Proposal 2: The maximum TBS can be configured in the same way as the legacy EDT (i.e. per CE level).

Proposal 3: CB-Msg3 specific configuration is included in one field contained in SIB2(-NB). The presence of this CB-Msg3 specific field indicates that the UE is allowed to initiate CB-Msg3 EDT in the cell.

Proposal 4: Introduce per CE level resource pools which include the CB-Msg3 EDT cell specific PUSCH resources. UE selects the resource pool associated with the intended CE level, according to the RSRP criterion.

Proposal 5: The details on physical resource structure of CB-Msg3 EDT in the resource pool (e.g. whether to reuse the existing RU in NB-IoT or design other resource granularity) are left to RAN1.

Proposal 5a: UE selects the PUSCH resources for replica transmissions in the resource pool associated with the selected CE level.

Proposal 6: Each CE-level is configured with a "number of repetition" which is associated to the resource pool of the corresponding CE-level.

Proposal 6a: Each CE-level is configured with an MCS level, which is associated to the resource pool of the corresponding CE-level.

Proposal 6b: For a CB-Msg3 EDT transmission, UE applies the "number of repetition" and MCS associated with the selected CE level.

Proposal 7: The CE level selected upon the initiation of CB-Msg3 EDT will be used for each replica transmission for this CB-Msg3 EDT procedure.

Proposal 8: The CB-Msg3 transmission procedure follows the legacy mechanism with grant based handling.

Proposal 9: A CB-Msg3 window is started for each Replica transmission respectively, and the UE only monitors PDCCH scrambled with the corresponding CB-Msg3-RNTI for the corresponding replica within the CB-Msg3 window.

Proposal 10: UE considers the CB-Msg3 transmission failure upon the CB-Msg3 window expires for the last replica.

Proposal 11: For DSA case, after the end of all repetition of CB-Msg3 PUSCH transmission of each replica, UE starts the corresponding window for NW response reception, taking UE-eNB RTT into account.

Proposal 12: CB-Msg3-RNTI should be computed for each Replica respectively in CB-Msg3 EDT.

Proposal 13: OCC index can be considered for CB-Msg3 RNTI calculation for NPUSCH format single tone at least for OCC length 2.

Proposal 14: For CB-Msg3 EDT, the similar mehcanism like Msg4 in RACH can be reused for contention resolution, e.g., if the beginning part (e.g. 48 bits) in the DL message matches the corresponding part of the UL message sent by the UE, contention resolution is considered successful for CB-Msg3 by the UE.

Proposal 15: If the CB-Msg3 window of the last replica expires, the UE will assume this CB-Msg3 transmission procedure fails. RAN2 further discusses whether the UE re-attemps another CB-Msg3 transmission, falls back to RA-EDT or declares CB-Msg3 transmission failure to upper layers in this case.

- 4.	Reference

- R2-2410913	Report from Break-out session on NR-NTN and IoT-NTN	Session Chair (ZTE Corporation)

- TS 36.331

- R2-2500010	Reply LS on OCC for CB-msg3 NPUSCH