3GPP TSG-RAN WG2 Meeting #129                                    	R2-2500531

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.8.6

Source:	Qualcomm Incorporated

Title:	Frequency priorities and redirection from LTE to NR NTN

Document for:	Discussion and Decision

# Introduction

In RAN2#128, following agreements were made.

Agreements:

1.	RAN2 understands that the UE behaviour is currently not clear if the network includes a NR NTN frequency in redirectedCarrierInfo in the RRCConnectionRelease message in a TN cell. We can further discuss whether we want to clarify the UE behaviour and in case whether we need to introduce a new UE capability.

2.	Add a clarification in Stage 2 that only LTE to NR NTN is supported (and not eMTC NTN to NR NTN). Also add in the field description that NeighSatelliteInfoNR-r19 is only signalled in a LTE TN cell but not IoT NTN cell

In this document, we discuss the solution for dedicated NR NTN frequency priority and redirection to NR NTN frequency.

# Discussion

The frequency priority can be configured via dedicated signaling in RRCConnectionRelease message. In Rel-19 now when SIB33 provides satellite information for the NR carrier frequency, then the frequency may be associated with NR NTN but the UE may not be supporting the new feature LTE to NR NTN mobility.

[[	freqPriorityListNR-r15				FreqPriorityListNR-r15		OPTIONAL		-- Need ON

]]

FreqPriorityListNR-r15 ::=		SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR-r15

FreqPriorityNR-r15 ::=			SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

cellReselectionPriority-r15			CellReselectionPriority,

cellReselectionSubPriority-r15		CellReselectionSubPriority-r13		OPTIONAL		-- Need OR

}

### 5.2.4 Cell Reselection evaluation process #### 5.2.4.1 Reselection priorities handling Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease or RRCEarlyDataComplete message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information.

As per TS 36.304, the UE has to act upon it when priorities are provided in dedicated signaling. Then if the priority is higher than the current cell, the UE has to perform the measurement of the frequency. However, as per the RAN2 agreement, the inter-RAT measurements for EUTRA TN to NR NTN mobility is optional without UE capability signaling. Additionally RAN2 made following understanding:

RAN2 understands that the UE behaviour is currently not clear if the network includes a NR NTN frequency in redirectedCarrierInfo in the RRCConnectionRelease message in a TN cell. We can further discuss whether we want to clarify the UE behaviour and in case whether we need to introduce a new UE capability.

Therefore, in the first place, the frequency priority may not be relevant here. The EUTRA network should not provide dedicated frequency priorities for NR NTN frequency without proper satellite assistance information and without knowing UE’s GNSS is ON or not. It may be better to clarify in TS 36.304 that UE ignores dedicated frequency priority for NR NTN frequency. It is also possible to assume the Rel-16 behavior, i.e., just assume the provided frequency is TN frequency and network is expected not to include the NTN frequency.

- Add clarification that the UE assumes priorities of NR frequencies provided in dedicated signalling are associated with TN band frequencies.

In similar case, the network can also provide the redirection information to NR frequency via dedicated signaling. In this case also, the EUTRA cell should not include the NR NTN frequency in RedirectedCarrierInfo.

RedirectedCarrierInfo ::=			CHOICE {

eutra								ARFCN-ValueEUTRA,

geran								CarrierFreqsGERAN,

utra-FDD							ARFCN-ValueUTRA,

utra-TDD							ARFCN-ValueUTRA,

cdma2000-HRPD						CarrierFreqCDMA2000,

cdma2000-1xRTT						CarrierFreqCDMA2000,

...,

utra-TDD-r10						CarrierFreqListUTRA-TDD-r10,

nr-r15								CarrierInfoNR-r15,

nr-r17								CarrierInfoNR-r17

}

CarrierInfoNR-r15	::= SEQUENCE {

carrierFreq-r15					ARFCN-ValueNR-r15,

subcarrierSpacingSSB-r15			ENUMERATED {kHz15, kHz30, kHz120, kHz240},

smtc-r15							MTC-SSB-NR-r15				OPTIONAL		-- Need OP

}

CarrierInfoNR-r17	::= SEQUENCE {

carrierFreq-r17					ARFCN-ValueNR-r15,

subcarrierSpacingSSB-r17			ENUMERATED {kHz15, kHz30, kHz120, kHz240, kHz480, spare1},

smtc-r17							MTC-SSB-NR-r15				OPTIONAL		-- Need OP

}

### 5.2.7 Cell Selection at transition to RRC_IDLE or RRC_INACTIVE state For NB-IoT cell selection at transition to RRC_IDLE state is defined in clause 5.2.7a. At reception of RRCConnectionRelease message or RRCEarlyDataComplete message to move the UE into RRC_IDLE or RRC_INACTIVE, UE shall attempt to camp on a suitable cell according to redirectedCarrierInfo, if included in the RRCConnectionRelease message or RRCEarlyDataComplete message. If the UE cannot find a suitable cell, the UE is allowed to camp on any suitable cell of the indicated RAT. If the RRCConnectionRelease message or RRCEarlyDataComplete message does not contain the redirectedCarrierInfo UE shall attempt to select a suitable cell on an EUTRA carrier. If no suitable cell is found according to the above, the UE shall perform a cell selection starting with Stored Information Cell Selection procedure in order to find a suitable cell to camp on. When returning to RRC_IDLE or RRC_INACTIVE state after UE moved to RRC_CONNECTED state from camped on any cell state, UE shall attempt to camp on an acceptable cell according to redirectedCarrierInfo, if included in the RRCConnectionRelease message. If the UE cannot find an acceptable cell, the UE is allowed to camp on any acceptable cell of the indicated RAT. If the RRCConnectionRelease message does not contain redirectedCarrierInfo UE shall attempt to select an acceptable cell on an EUTRA carrier. If no acceptable cell is found according to the above, the UE shall continue to search for an acceptable cell of any PLMN in state any cell selection.

As per TS 38.304, if a NR NTN frequency is included in RedirectedCarrierInfo, the UE shall measure the frequency and attempt to camp on the cell. However, this is not correct for UEs not supporting the EUTRA TN to NR NTN mobility. As network wouldn’t know the UE’s capability, the EUTRA network should not include NR NTN frequency in redirectedCarrierInfo in the RRCConnectionRelease message.

In addition, while connected in TN, UE will not turn GNSS. The lack of GNSS positioning and satellite information associated with the redirected NR NTN carrier is certain to result in UE’s power consumption and redirection failure.

Such NR NTN carrier redirection from TN EUTRA works only if there is a mechanism for UE to turn ON GNSS and additional satellite information is provided in the RRCConnectionRelease message. We think this is not in the scope of the WI. Therefore, we think the UE should expect the NR frequency included in redirectedCarrierInfo in the RRCConnectionRelease message is a TN band frequency.

- Add clarification that the UE assumes that the NR frequency included in redirectedCarrierInfo in the RRCConnectionRelease message is a TN band frequency.

# Conclusion

Following proposals are made.

Proposal 1	Add clarification that the UE assumes priorities of NR frequencies provided in dedicated signalling are associated with TN band frequencies.

Proposal 2	Add clarification that the UE assumes that the NR frequency included in redirectedCarrierInfo in the RRCConnectionRelease message is a TN band frequency.