3GPP TSG-RAN WG2 Meeting #129                                    	R2-2500532

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.8.2

Source:	Qualcomm Incorporated

Title:	Default extended SSB periodicity

Document for:	Discussion and Decision

# Introduction

In RAN plenary#106, maximum SSB periodicity extension up to 160ms is considered and the DL coverage enhancement objective is updated in WID [1]:

  - Link level enhancements are to be specified for the following channels:

    - PDCCH at least for CSS (except for Type-3) via PDCCH repetition

    - PDSCH with Msg4 via PDSCH repetition

    - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

  - System-level enhancements are to be specified for the following:

    - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

      - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms

In this document, we discuss the RAN2 aspects of the default SSB periodicity that is larger than 20ms.

# Discussion

Currently the SSB extension periodicity of 160ms is already supported as specified in TS 38.331. However, it is to note that for initial access, the UE would not be aware of such extension from a cell and by default the assumption for UE is that the SSB periodicity is 20ms. This also impacts the MIB periodicity and default SIB1 repetition periodicity. Following is specified in TS 38.213.

- For initial cell selection, a UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames.

This would mean by default the UE would be searching the SSB with the assumption of 20ms SSB periodicity and it can impact UE’s action for cell search is the SSB is not detected in the next 20ms. However, if a UE is able to detect the SSB, it can access the cell and eventually transition to RRC_CONNECTED. Therefore, we think change of default SSB periodicity up to 160ms does not mean the legacy UEs cannot be supported, though some unforeseen impacts could be possible such as RLM and cell reselection due to no SSB signal. A new barring procedure for this purpose is not possible as barring information itself would be in SIB1.

A new barring procedure is not needed for a cell that is using new default SSB periodicity of larger than 20ms.

cell measurement:

Even though the UE can access a cell that is operating with default SSB periodicity of 160ms, the UE might be unnecessarily performing frequency search or even trying to measure the neighbor cell at wrong occasions. To avoid this, a cell should indicate the default SSB periodicity in SIB1. This helps UE to optimize the measurements and SSB detection. In addition, information of neighbor cells on the periodicity of larger than 20ms should also be considered to assist UE in neighbor cell measurements.

Currently, network can indicate SSB periodicity in SIB1. This would be sufficient for the extended SSB periodicity.

#### –	ServingCellConfigCommonSIB

The IE ServingCellConfigCommonSIB is used to configure cell specific parameters of a UE's serving cell in SIB1.

ServingCellConfigCommonSIB information element

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMONSIB-START

ServingCellConfigCommonSIB ::=      SEQUENCE {

downlinkConfigCommon                DownlinkConfigCommonSIB,

uplinkConfigCommon                  UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

supplementaryUplink                 UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                           OPTIONAL, -- Need S

ssb-PositionsInBurst                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))                                   OPTIONAL  -- Cond FR2-Only

},

ssb-PeriodicityServingCell          ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160},

Existing signaling ssb-PeriodicityServingCell in serving cell common configuration is sufficient to indicate the default SSB periodicity of a cell.

Existing SMTC configuration is sufficient to assist UE for neigbor cell measurements.

Once a UE connects to the cell, then network can configure measurement object for neighbour cells and perform handover. In this case, the network may choose to prioritize measurement and handover of legacy UEs to the legacy cells or cells that do not use the default SSB periodicity of, say, 160ms. For this purpose, it would be beneficial for network to know the UE’s capability to support the default SSB periodicity of larger than 20ms.

Introduce a UE capability whether the UE supports default SSB periodicity of larger than 20ms.

## 5.2 System information ### 5.2.1 Introduction System Information (SI) is divided into the MIB and a number of SIBs and posSIBs where: - the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms (TS 38.212 [17], clause 7.1) and it includes parameters that are needed to acquire SIB1 from the cell. The first transmission of the MIB is scheduled in subframes as defined in TS 38.213 [13], clause 4.1 and repetitions are scheduled according to the period of SSB; NOTE 1: If the period of SSB is larger than 80 ms, the MIB is transmitted with the same periodicity as that of SSB. - the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

# Conclusion

Following proposals are made.

Proposal 1	A new barring procedure is not needed for a cell that is using new default SSB periodicity of larger than 20ms.

Proposal 2	Existing signaling ssb-PeriodicityServingCell in serving cell common configuration is sufficient to indicate the default SSB periodicity of a cell.

Proposal 3	Existing SMTC configuration is sufficient to assist UE for neigbor cell measurements.

Proposal 4	Introduce a UE capability whether the UE supports default SSB periodicity of larger than 20ms.

# References

[1] RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”.