3GPP TSG-RAN WG2 Meeting #129                                    	R2-2500533

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.9.2

Source:	Qualcomm Incorporated

Title:	Switching of S&F mode

Document for:	Discussion and Decision

- Introduction

In RAN2#128 meeting, following agreements were made.

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

In this document, we provide further details for solution of switching S&F mode and normal mode.

- Discussion

Figure 1 An example of switching of normal mode and S&F mode

Whenever the satellite establishes feeder link connection (feeder link becomes available), it is also possible that a cell may want to exit from the S&F mode operation (for example, switch the S&F mode to transparent architecture mode or only eNB onboard mode). In this case, the UEs supporting S&F mode operation may also support normal mode operation, in which case, these UEs may not need to take any action (e.g., cell reselection) due to the switch.

However, if cellBarred-NTN-r17 is changed or the S&F mode indication is changed, the existing procedure should follow to update the SIB1, which requires UE to take action on updating the system information. This requires network sending paging message adding signaling overhead. While in RRC_CONNECTED, the UE does not need to worry about the cell barring change.

Therefore, we think there should be a time information provided when such switching occurs and this should be indication that SIB1/SIB31 may change or S&F indication may change. The UE should be given enough time to acquire the SIBs, for example, running timer T318. However, the connected mode UEs may stay in the same cell until the network releases to IDLE mode.

For switching time from S&F mode to normal mode, reuse existing t-Service and add indication that it is associated with the S&F mode switch (not service link or feeder link switch).

When S&F mode to normal mode switch time is indicated, discuss how to update the system information without paging if barring bit or S&F mode indication is changed for new UEs.

Whenever the satellite loses feeder link connection, cell supporting S&F mode should make sure that S&F indication is included in SIB. In this case, the cell can release all UEs that do not support S&F mode operation in RRC_CONNECTED to IDLE mode. In this case, the cell may also be releasing the legacy UEs.

The SIB may need to be updated to provide updated information of feeder link or S&F mode indication and next switch time. We think it is very important to avoid paging notification to eMTC and NB-IoT UEs. The existing cell stop time mechanism can be used to inform the legacy UEs that they need to find other cells. This can be useful for both cases, i.e., when feeder link will be lost or when feeder link will be available.

For new UEs, a new indication (probably in SIB19) can be introduced to indicate that the cell stop time is associated with the update of S&F mode operation, i.e., at cell stop time, the cell loses feeder link and operates in S&F mode.

For switching time from normal mode to S&F mode, the existing cell stop time (t-Service) reused.

For legacy UEs, the existing behavior of t-Service applies when normal mode is switched to S&F mode at t-Service.

For new UEs, additional indication is provided to associate the existing cell stop time with the switching time from normal mode to S&F mode.

When feeder link becomes available, the connection with MME onboard may be torn down and instead the MME on the ground can be used. When operating in normal mode, the satellite may include eNB only on board or may switch to transparent payload mode. In this case, it can support any NTN capable UE. Similarly, the cell can also switch back to S&F mode when feeder link is lost and use MME onboard satellite.

RAN2 can assume that the cell operating in S&F mode uses MME on board and cell operating in normal mode uses MME on the ground. Whether MME on board can operate in normal mode is decided by SA2.

The UE behavior at the cell switch also needs to be clarified. The legacy UEs will follow the existing behavior for t-Service, if t-service is used. For new UEs, the behavior can be optimized. The UEs in RRC_CONNECTED mode may not need to take any action and stay in the cell even after the switch. It is up to network to release the UEs properly when there is no data transmission is expected. UE can do so without updating the system information, however, the RTT needs to be updated if the ULSRP changes, e.g., transparent mode was used.

For UEs in RRC_IDLE, it can be left to UE whether to update the SIB1 and SIB31 or trigger the cell reselection procedure at the switch time. This should be discussed.

Clarify procedure for new UEs in RRC_CONNECTED and RRC_IDLE when the S&F mode switching happens.

- Conclusion

Following observation and proposals are made:

Proposal 1	For switching time from S&F mode to normal mode, reuse existing t-Service and add indication that it is associated with the S&F mode switch (not service link or feeder link switch).

Proposal 2	When S&F mode to normal mode switch time is indicated, discuss how to update the system information without paging if barring bit or S&F mode indication is changed for new UEs.

Proposal 3	For switching time from normal mode to S&F mode, the existing cell stop time (t-Service) reused.

Proposal 4	For legacy UEs, the existing behavior of t-Service applies when normal mode is switched to S&F mode at t-Service.

Proposal 5	For new UEs, additional indication is provided to associate the existing cell stop time with the switching time from normal mode to S&F mode.

Observation 1.	RAN2 can assume that the cell operating in S&F mode uses MME on board and cell operating in normal mode uses MME on the ground. Whether MME on board can operate in normal mode is decided by SA2.

Proposal 6	Clarify procedure for new UEs in RRC_CONNECTED and RRC_IDLE when the S&F mode switching happens.