3GPP TSG-RAN WG2 Meeting #129                                    	R2-2500534

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.9.3

Source:	Qualcomm Incorporated

Title:	CB-Msg3 and Msg4 enhancements

Document for:	Discussion and Decision

- Introduction

In RAN2#128, following agreements were made.

Agreements:

1.	Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

2.	Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

3.	The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

4.	There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

5.	The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

6.	There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

7.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

9.	The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

11.	Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

12.	RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

In this document, we provide details for DSA solution.

- Discussion

## Diversity Slotted Aloha (DSA)

How DSA works is explained in [1]. For DSA, the network should configure a number of replicas (i.e., K) allowed for transmission. There could be N EDT occasions configured for UE to choose randomly and UE selects randomly K occasions (i.e., a Msg3 transmission and K replicas) out of N occasions. Following was agreed in RAN2#128.

- The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

- There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

If K = 1 (i.e., number of replicas = 0), then UE will have only one PUSCH resource to select. If K = 2, then UE will choose randomly 2 PUSCH resources in different time occasions and transmit one original msg3 transmission and one replica of the Msg3 transmission as shown in figure 2 below. It is to note that at a time occasion, a UE may randomly choose different frequency domain or OCC domain resources, for example in figure 2, the frequency domain resources at subframe 2 and 6 can be different.

Figure 1 DSA operation with K = 1

However, we think the DSA configuration such as mixing the K = 1 (SA) and K > 1 (DSA) should be possible for new UEs, for example, resources for two CE levels configured one with K = 1 for CE level 0 and K = 2 for CE level 1. However, UEs not supporting DSA feature should not use the resource with K =1 for cleaner procedure, e.g. support of group RNTI.

UEs not supporting DSA feature do not use new DSA CB-Msg3 resource configured with K =1.

Based on the configuration of DSA resource UE knows the network’s capability. Based on the use of the DSA resource, network knows the UE’s capability to support DSA.

Based on the configuration of DSA resource UE knows the network’s capability. Based on the use of the DSA resource, network knows the UE’s capability to support DSA.

Transmission occasion group

For DSA, the network configures a group of transmission occasions for UE to transmit replicas in. Consider the example where network configures N = 2 transmission occasions (TO1 and TO2) in a group for UEs to transmit K=2 replicas. Suppose the gap between TO1 and TO2 is large, the contention resolution window for TO1 starts much earlier than TO2, it is possible for UEs with successful CB-Msg3 transmission in TO1 to receive a successful Msg4. However, DSA gain will be marginal. The TOs must be well separated and sparse in time for this to occur which limits the total throughput of EDT in the system. To increase throughput these TOs must be closer in time. A UE with successful transmission only in TO2 will receive a success Msg4 at least 1 RTT after the end of TO2. So, reducing the gap between TO1 and TO2 reduces the worst case delay without affecting the throughput. Hence having TOs closer together within a group is essential to reduce overall delay.

Figure 2 Example of DSA with too sparse TOs

Figure 3 Example of DSA with closely sparse TOs

Therefore, to assist network in the proper configuration of the TOs and response windows taking maximum benefit of the DSA, the starting point for the N DSA transmission occasions can be defined. The gap between the TOs in the group can be defined as offset. The stage 3 detail on the value range of offset can be discussed later or can be checked with RAN1 considering the need for sparse TOs. Similarly, the network may want to configure the gap between two groups which can be different than the gap between two TOs within a group. For this purpose, the periodicity of the DSA groups can be defined.

The starting point of the first transmission occasion of the group of replicas is configured by the network. The transmission occasions for replicas within the group are defined w.r.t. the first transmission occasion.

Periodicity of the DSA transmission occasion groups can be configured.

RNTI calculation

In RAN2#128, following agreement was made.

- For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

It should be clear how the Msg4 transmission is performed. If there is multiple PDCCH monitoring windows (i.e., one window per replica), the RNTI to be monitored will be different for each window. This would require UE to monitor multiple RNTIs as windows may overlap. In addition, this approach of operation would have some issues on maintaining multiple PDCCH monitoring windows:

- As shown in figure 2, if the UE receives rescheduling associated with the first window (i.e., first Msg3 replica transmission), then the benefit of DSA is not achieved as the successful Msg4 response associated with the second window (i.e., second replica of the Msg3) may collide with or may be scheduled after the retransmission grant of the first Msg3 replica.

- If network is able to decode only one replica of the Msg3 transmission, then the network can transmit the Msg4 response corresponding to the only one Msg3 transmission that is successfully decoded. However, the other Msg3 transmission replicas may not be decoded, and network may unnecessarily schedule the retransmission of the replica Msg3 transmission that was not decoded successfully. This results in unnecessary resource wastage.

For resolving issue#1 above, the network can wait until the resource occasion N (i.e., last transmission occasion of the group) to decide on retransmission. Then UE will always receive the successful response first.

This can be achieved with single PDCCH monitoring window. The other benefit of single PDCCH monitoring window is that network can take maximum benefit from the Msg4 multicast enhancements as a single RNTI associated with the window would be sufficient. Single RNTI per window will also resolve the risk of RNTI collision and UE’s complexity on monitoring multiple RNTIs.

A single RNTI specific to a PDCCH monitoring window is used. FFS on details for RNTI derivation.

A single PDCCH monitoring window is considered.

For resolving the second issue#2 above, at occasion N, the network may also be able to figure out whether retransmission is needed based on the RSRP level. If the second copy is decoded, then if the network knows resource for first copy(say resource A), the network can decide to not issue a retransmission for resource A if it can determine the decoding failure of the first copy was likely due to poor RSRP or not due to contention.

When a UE transmits a copy of the CB-Msg3 in a transmission occasion, a mechanism is defined to help network determine the location of the other copy(ies). FFS on details of how to do it.

Contention resolution

In our view, existing contention resolution timer should be used for the PDCCH monitoring window. After every replica transmission the CR timer can be started or restarted considering the last repetition plus UE-eNB RTT as shown in figure 2. To take maximum benefit of Msg4 multicast enhancement, the CR timer can be defined to be started after the last replica transmission occasion plus UE-eNB RTT. However, for having a possibility to improve the delay for some UEs by responding the Msg4 response earlier, the CR timer can be started after the first replica transmission plus UE-eNB RTT. The issue with this approach is that subsequent replica transmission occasion may overlap with the CR timer but a rule can be defined to prioritize the replica transmission over Msg4 monitoring. The contention resolution can still be handled in the same existing way, i.e., by the use of contention resolution timer and contention resolution MAC CE in Msg4. There can be two options to maintain contention resolution timer.

The contention resolution timer is used as PDCCH monitoring window. It is started or restarted after the last repetition of the first replica plus UE-eNB RTT.

For Msg4, existing contention resolution procedure can be used, i.e., contention resolution timer and contention resolution MAC CE.

## CB-Msg3 transmission attempt

It should be discussed how to handle the fallback scenario. It may not be efficient to keep trying only DSA resource or CB-Msg3 transmission which is based on PUSCH resource. It can be based on resource being unavailable when CE level is changed during re-attempt or reaching a maximum number of attempts or new data arrival during re-attempt.

Following was agreed in the RAN2#128 meeting:

- Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

- The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

- There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

Since the existing RSRP threshold is to define the initial PRACH CE level, same threshold may not be suitable for determining the CE level for the RSRP threshold and existing number of CE levels defined for PRACH can be supported for CB-Msg3.

New RSRP threshold to determine the CE level for CB-Msg3 transmission is defined.

Number of CE levels for CB-Msg3 is same as that of PRACH CE levels.

There should also be a limit on how many attempts the UE is allowed for the CB-Msg3 transmission if the UE declares the contention resolution failure or contention resolution timer expires. After the limit, the UE can fallback to normal 4 step RACH-based procedure, e.g., RACH or EDT. We think this can be configured by network via system information.

If CB-Msg3 transmission fails, network configures via system information the maximum number of re-attempts allowed for CB-Msg3 transmission before falling back to 4 step RACH-based procedure.

The maximum TBS size depends on the coverage level. Therefore, we think the pending UL data size can be different for each CE level to make decision whether the CB-Msg3 can be used.

Maximum TBS size for CB-Msg3 is different for different CE level.

After CB-Msg3 transmission, if the network wants UE to move to RRC_CONNECTED mode, the same RNTI cannot converted to the C-RNTI. It is because the RNTI is derived from the CB-Msg3 resource and should not be reused in RRC_CONNECTED. Therefore, the network should provide a new C-RNTI (e.g. by RRC message) to be used when moved to RRC_CONNECTED.

After CB-Msg3 transmission, if a UE is moved to RRC_CONNECTED, a new C-RNTI is provided to the UE.

Also, it is possible that network wants UEs to take back off or fall back to the 4 RACH procedure. We this option should also be possible for the network.

Support network indication of backoff or fallback to 4 step RACH procedure.

## Msg4 enhancements

The second objective of the EDT enhancement is to reduce the congestion/overhead for transmission of Msg4 to several UEs. For example, many UEs may initiate EDT and network may exhaust the PDCCH or PDSCH resources while scheduling Msg4 (which comprises a PDCCH and at least one PDSCH) to each individual UE. There may be cases where network may not have any downlink response data and network would need to schedule Msg4 just including RRCEarlyDataComplete message (without DL NAS PDU) and contention resolution MAC CE of multiple UEs.

Instead of sending individual Msg4’s in such instances, a “multiplexed Msg4” to several UEs may be considered. An example procedure where multiplexed transmissions to several UEs using a single PDSCH are performed exists in case of Random-Access response. As shown in figure 3, network can schedule multiple RARs in a single MAC PDU.

Figure 4 TS 36.321 Figure 6.1.5-4: Example of MAC PDU consisting of a MAC header and MAC RARs

Multicast Msg4 (multi-user Msg4 multiplexing, similar to RAR or NR 2-step RACH response) or multi-user Msg4 scheduled by a single DCI (as in multi-TB Msg4 scheduling) can be studied as a solution.

Figure 5 Msg4 enhancement for EDT

In this case there could be several possible options for the enhancements, some examples being:

- Reusing a PUR-like L1 ACK procedure

  - The L1 ACK may also be used to resolve contention between UEs

  - The L1 ACK can move UE back to RRC_IDLE

- Reusing concept of RAR-like multiplexing (e.g., of multiple users RAPIDs in a single RAR)

  - A single PDU can multiplex contention resolution MAC CEs (or Msg4) of more than one UE.

- Others such as existing Multi-TB scheduling for Msg4 could also be studied for multi-user scenario.

A PUR-like L1 ACK concept, as a response to the EDT transmission, can be considered.

The Msg4 enhancements can be on the top of existing 4 steps EDT procedure as shown in figure 3, or applicable for the case of CB-msg3 / DSA as described for the first objective.

The enhancements done for Msg4 transmission can be applicable to normal 4 step RACH or CB-Msg3 or DSA transmission.

## Aspects related to RAN1

Similar to what was done in EDT, the introduction of CB-Msg3 may include some small specification updates in RAN1 specifications. For instance, the following should be considered by RAN1:

- Design of search spaces for response.

- Derivation of CB-Msg3 RNTI

- Aspects related to retransmission of msg3-CB (e.g. DCI design)

- TBS size per CE level

- Any other feedback on msg3-CB transmission (e.g. DMRS design, scrambling of NPUSCH, periodicity of TOs within a group etc.)

Send LS to RAN1 informing potential alignment work associated with the CB-Msg3 transmission for DSA and non-DSA.

- Conclusion

Following proposals are made:

Proposal 1	UEs not supporting DSA feature do not use new DSA CB-Msg3 resource configured with K =1.

Proposal 2	Based on the configuration of DSA resource UE knows the network’s capability. Based on the use of the DSA resource, network knows the UE’s capability to support DSA.

Proposal 3	The starting point of the first transmission occasion of the group of replicas is configured by the network. The transmission occasions for replicas within the group are defined w.r.t. the first transmission occasion.

Proposal 4	Periodicity of the DSA transmission occasion groups can be configured.

Proposal 5	A single RNTI specific to a PDCCH monitoring window is used. FFS on details for RNTI derivation.

Proposal 6	A single PDCCH monitoring window is considered.

Proposal 7	When a UE transmits a copy of the CB-Msg3 in a transmission occasion, a mechanism is defined to help network determine the location of the other copy(ies). FFS on details of how to do it.

Proposal 8	The contention resolution timer is used as PDCCH monitoring window. It is started or restarted after the last repetition of the first replica plus UE-eNB RTT.

Proposal 9	For Msg4, existing contention resolution procedure can be used, i.e., contention resolution timer and contention resolution MAC CE.

Proposal 10	New RSRP threshold to determine the CE level for CB-Msg3 transmission is defined.

Proposal 11	Number of CE levels for CB-Msg3 is same as that of PRACH CE levels.

Proposal 12	If CB-Msg3 transmission fails, network configures via system information the maximum number of re-attempts allowed for CB-Msg3 transmission before falling back to 4 step RACH-based procedure.

Proposal 13	Maximum TBS size for CB-Msg3 is different for different CE level.

Proposal 14	After CB-Msg3 transmission, if a UE is moved to RRC_CONNECTED, a new C-RNTI is provided to the UE.

Proposal 15	Support network indication of backoff or fallback to 4 step RACH procedure.

Proposal 16	Multicast Msg4 (multi-user Msg4 multiplexing, similar to RAR or NR 2-step RACH response) or multi-user Msg4 scheduled by a single DCI (as in multi-TB Msg4 scheduling) can be studied as a solution.

Proposal 17	A PUR-like L1 ACK concept, as a response to the EDT transmission, can be considered.

Proposal 18	The enhancements done for Msg4 transmission can be applicable to normal 4 step RACH or CB-Msg3 or DSA transmission.

Proposal 19	Send LS to RAN1 informing potential alignment work associated with the CB-Msg3 transmission for DSA and non-DSA.

- References

[1] R2-2408896, “Discussion on EDT enhancements”, Hefei, China, Oct 14th – 18th, 2024.