3GPP TSG-RAN WG2 Meeting #129                                    	R2-2500536

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.17

Source:	Qualcomm Incorporated

Title:	Discussion on new NB-IoT NTN TDD mode

Document for:	Discussion and Decision

- Introduction

A new work item is approved in [1] to integrate the 90ms TDD frame structure into the existing NB-IoT NTN FDD system.

In this document, we provide details on RAN2 specific aspects that need to be addressed.

- Discussion

The current iridium system deployed in the 1616-1626.5 MHz MSS band [2] uses a framing with a periodicity of 90ms with the following structure (also depicted in Figure 1):

- Simplex slot (20.32ms) – used for downlink-only, ring and messaging channels

- 4 uplink slots (8.28ms long each one)

- 4 downlink slots (8.28ms long each one)

- Guard periods in between

Figure 1 An example of low duty cycle operation with new TDD frame structure, i.e., there will be many invalid DL frames

As DL slot is 8.28ms, it can support max 8 consecutive DL slots to include all essential broadcast signals PSSS, SSS, PBCH and SIB1 given that the 8 consecutive downlink subframes shall include subframes {0, 4, 5, 9}. Considering these constraints, currently RAN1 is working on specifying the NB-IoT NTN TDD pattern using FDD procedure (as introduction of TDD in NTN is not considered). However, there are also higher layers work needed to support the new frame structure.

The new frame structure will not be backward compatible. In addition, we think a new band indicator will be defined for such system. Therefore, no legacy UEs or UE not supporting such new frame structure will not support the new band. Therefore, NB-IoT NTN TDD cell should be considered as a new cell which cannot be accessed by legacy UEs and no additional barring mechanism would need to be introduced.

RAN2 assume the cell with new TDD frame structure is called NB-IoT NTN TDD cell which is not accessible by legacy UEs and no new barring mechanism is needed.

With the new NB-IoT TDD cell, the length of frame will be 90ms, i.e., 9 SFN and current H-SFN does not wrap around at 90ms. In addition, the frame structure will result in extremely low duty cycle operation as shown in Fig 1, i.e., there is likely that only 1 SFNs (or two partial SFNs) out 9 SFNs would be only valid DL SFN or valid UL SFN.

Because of such constraints, we think RAN2 should further discuss following aspects.

- SFN wrap around issue and its consequence on calculation for starting SFN for SI scheduling, eDRX, PUR etc.

- PDCCH period and PDCCH subframe definition for timers

- Increased delay in Paging, initial access and SI acquisition

- UE’s DRX state.

H-SFN wrap around issue:

Currently H-SFN is configured from 0 to 1023 and wraps around after 1024 H-SFNs. 1024 H-SFN translates to 1048576 SFN and 1048576 SFN is not multiple of 9 SFN which is the new TDD frame periodicity. Suppose SFN x in a H-SFN is the downlink SFN. Then in the next H-SFN, the SFN x will become invalid SFN and different SFN y will be downlink SFN.

Currently the minimum SI modification boundary is defined in terms of H-SFN, i.e., satisfying the equation (H-SFN * 1024 + SFN) mod 4096 = 0. This will result in falling the SI modification boundary in different SFN after H-SFN wrap around. The SI-window starts at the subframe #0 in the radio frame for which (H-SFN * 1024 + SFN) mod T = FLOOR(x/10) + Offset where T is the si-Periodicity of the concerned SI message and, Offset is the offset of the start of the SI-Window (si-RadioFrameOffset). Similar problem occurs on the start of the SI window. Currently NB-IoT uses H-SFN for RA-RNTI, SIB scheduling/modification boundary, PUR, eDRX, SC-PTM, MCCH channel and they may be impacted by the H-SFN wrap around issue.

Currently RAN1 is also investigating the issue. The wrap around issue at SFN level is also being discussed. If SFN level wrap around is adopted, it will have larger impact to RAN2 specifications due to use of SFN for start of timers. We think both SFN level and H-SFN level wrap around solutions should be studied by RAN2.

Study the impact on starting SFN for RA-RNTI, SIB scheduling/modification boundary, PUR, eDRX, SC-PTM, MCCH channel due to H-SFN wrap around issue.

PDCCH period and PDCCH subframe

Currently many timers for NB-IoT are configured in the unit of PDCCH period (PP), such as DRX timers, RAR window, contention resolution timer. The PP duration is based on the gap between two consecutive NPDCCH search spaces determined by  𝑮.𝑹_𝒎𝒂𝒙  (≥𝟒);𝑮∈{𝟏.𝟓, 𝟐, 𝟒, 𝟖, 𝟏𝟔, 𝟑𝟐, 𝟒𝟖, 𝟔𝟒}  𝒎𝒔. The definition of PP and how to count the PDDCH subframe is specified in TS 36.321.

Therefore, it is possible when a timer is configured in number of PP, then all or some of the PDCCH search spaces may fall in the invalid DL SFN due to low duty cycle operation. The way the PDCCH period and PDCCH subframe is used for the timer duration calculation may need to be updated by RAN2.

RAN2 can study the scenario where the PDCCH monitoring falls on the invalid downlink SFN. For example, example the paging occasion may fall on the invalid DL SFN increasing the paging delay. For the system information acquisition, the SI window may also fall on the invalid DL SFN and this might impact the SI acquisition and initial access delay. The RAR window and contention resolution timer may fall on the invalid DL SFN making initial access delay worse. The DRX timers may fall on the invalid DL SFN impacting the UE power consumption and QoS significantly.

Similarly, the scenarios where UL transmission occasion falling on the in valid uplink SFN such as PRACH occasion should also be considered.

Study the impact of invalid DL and UL SFNs on Paging delay, SI acquisition, random access and DRX timers.

UE’s DRX state

As explained, the new NB-IoT NTN TDD cell operates in very low duty cycle, for example, only 8 ms out of 90 ms can be active time for PDCCH monitoring and rest UE may stay inactive. Even though the UE is in active time based on DRX inactivity timer or some other events, the UE would not need to monitor the PDCCH during invalid DL SFN duration of 90ms frame periodicity. During invalid SFN duration, whether the UE is in DRX inactive state and whether it impacts the RLM monitoring can also be discussed. However, any requirements to meet for measurements due to many invalid SFNs should be discussed in RAN4.

Discuss the UE’s DRX state during invalid SFN duration.

- Conclusion

Following proposals are made:

Proposal 1	RAN2 assume the cell with new TDD frame structure is called NB-IoT NTN TDD cell which is not accessible by legacy UEs and no new barring mechanism is needed.

Proposal 2	Study the impact on starting SFN for RA-RNTI, SIB scheduling/modification boundary, PUR, eDRX, SC-PTM, MCCH channel due to H-SFN wrap around issue.

Proposal 3	Study the impact of invalid DL and UL SFNs on Paging delay, SI acquisition, random access and DRX timers.

Proposal 4	Discuss the UE’s DRX state during invalid SFN duration.

- Reference

[1] RP-242415, New WID on introduction of IoT-NTN TDD mode

[2] ICAO TECHNICAL MANUAL FOR IRIDIUM AERONAUTICAL MOBILE SATELLITE (ROUTE) SERVICE (available here)