3GPP TSG-RAN WG2 Meeting #129	R2-2500546

Athens, Greece, 17 – 21 February 2025

Agenda item:	7.0.2.15

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRC segmentation coordination for QoE reporting in NR-DC

WID/SID:	NR_QoE_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The inter-gNB coordination concerning RRC segmentation for QoE reporting on SRB4 and/or SRB5 in NR-DC scenario was discussed in RAN3. However, no consensus was reached on whether it is needed to exchange the segmentation status for SRB4/5 between MN and SN. Consequently, an LS [1] has been sent to RAN2 for further clarification with below questions.

- Question 1: Can the UE receive an AppLayerMeasConfig-r17 IE for MCG configuration containing rrc-SegAllowedSRB5-r18 parameter? - Question 2: Can the UE receive an AppLayerMeasConfig-r17 IE for SCG configuration containing rrc-SegAllowedSRB4-r17 parameter? - Question 3: If the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18 parameter, how does the UE treat its RRC segmentation state for SRB5 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB5 or does the UE consider that the RRC segmentation function for SRB5 shall be disabled? - Question 4: If the UE receives from the SN the AppLayerMeasConfig-r17 parameter which does not include the rrc-SegAllowedSRB4-r17 parameter, how does the UE treat its RRC segmentation state for SRB4 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB4 or does the UE consider that the RRC segmentation function for SRB4 shall be disabled?

In this document, we would like to provide our view on the issues.

# 2	Discussion

For a UE in NR-DC, both SRB4 (direct SRB between the MN and the UE) and SRB5 (direct SRB between the SN and the UE) can be configured simultaneously for QoE reporting. In order to allow the transmission of application layer measurement reports which exceed the maximum PDCP SDU size, the network can inform the UE whether the MN allows RRC segmentation of MeasurementReportAppLayer message sent by the UE via SRB4 and whether the SN allows RRC segmentation of MeasurementReportAppLayer message sent by the UE via SRB5. The segmentation allowance for SRB4 and SRB5 is controlled by rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5, respectively, under the same IE (AppLayerMeasConfig).

In NR-DC, according to current RRC specification, the IE AppLayerMeasConfig can be used by MN and/or SN to indicate configuration of application layer measurements for the corresponding node.  Furthermore, both the rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 are optional fields with Need R, which means if the field absent, the UE releases the current value hence disables the QoE report segmentation in the corresponding SIB4/SIB5.

AppLayerMeasConfig information element -- ASN1START -- TAG-APPLAYERMEASCONFIG-START AppLayerMeasConfig-r17 ::= SEQUENCE { measConfigAppLayerToAddModList-r17 SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayer-r17 OPTIONAL, -- Need N measConfigAppLayerToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayerId-r17 OPTIONAL, -- Need N rrc-SegAllowedSRB4-r17 ENUMERATED {enabled} OPTIONAL, -- Need R ..., [[ rrc-SegAllowedSRB5-r18 ENUMERATED {enabled} OPTIONAL, -- Need R idleInactiveReportAllowed-r18 ENUMERATED {enabled} OPTIONAL -- Need R ]] } Need R; Release Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE releases the current value..

Observation 1: According to the current RRC spec, the IE AppLayerMeasConfig (including both rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5) can be used by MN and/or SN to indicate configuration of application layer measurements for the corresponding node.

Observation 2: For rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5, since they are defined as Optional NEED R, the UE releases the current value if it receives a message with the field absent.

For NR-DC, we understand that one of the principles in the signalling design is to keep independent configurations between MN and SN. E.g., The CG configurations are within OCTET STRINGs as transparent containers, and MN/SN decides on their own configurations. However, it seems current ASN.1 design for QoE configurations via AppLayerMeasConfig cannot support the independent configuration for SRB4 and SRB5 segmentation allowance. If MN configures the SegAllowedSRB4 while leaves the SegAllowedSRB5 as absent, the UE should regard the SRB5 segmentation as disabled (due to NEED R). If SN configures the SegAllowedSRB5 while leaves the SegAllowedSRB4 as absent, the UE should regard the SRB4 segmentation is disabled.

Observation 3: In NR-DC, the current design for IE AppLayerMeasConfig implies that configuring SRB4 or SRB5 segmentation on one node will impact the segmentation enabling/disabling on the other node.

Therefore, due to the current AppLayerMeasConfig configuration, coordination between the MN and SN is required regarding the SRB4 and SRB5 segmentation status. This ensures that one node can properly configure rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 within the same QoE configuration (AppLayerMeasConfig). Otherwise, it seems that the specification is broken on how the MN or SN should determine the segmentation status for both SRB4 and SRB5.

Observation 4: Based on the current design for IE AppLayerMeasConfig, it requires coordination between the MN and SN regarding the SRB4 and SRB5 segmentation status.

Alternatively, RAN2 may redesign the RRC IE for the SRB4/SRB5 segmentation configuration hence MN and SN can configure the status for SRB4 and SRB5 independently. However, this kind of change would be NBC change. We think it should be avoided at this later stage.

Observation 5: Independently configuring the SRB4 and SRB5 segmentation status in MN and SN may require a redesign of the RRC IE for segmentation allowance, which would be a NBC change.

RAN2 may aim to retain the current design for the QoE configuration in the RRC specification and introduce coordination between the MN and SN for the SRB4 and SRB5 segmentation status. That means MN and SN need to know each other’s configuration to ensure they don’t cause issues. One simple approach is for one node to report its own SRB segmentation status to the peer node. This can be achieved either via an RRC inter-node message (INM) or via an XnAP message.

Proposal 1: In NR-DC, one node shall report its own SRB segmentation status to the peer node, which can be done either via RRC INM message or XnAP message.

Based on above analysis, we think RAN2 can acknowledge the issue raised by RAN3 on the SRB4/5 segmentation status exchange. As RAN3 already prepared the CRs for both 37.340 [2] and 38.423 [3], it seems a good way-forward to let RAN3 to decide on the signalling preference for XnAP or RRC INM message.

Proposal 2: RAN2 acknowledges the issue raised by RAN3 and confirms that the exchange of SRB4/5 segmentation status is required in NR-DC.

Proposal 3: Reply to the RAN3 LS and let RAN3 to decide whether the XnAP message will be used for the SRB4/5 segmentation status exchange.

# 3	Conclusion

This document has made the following observations:

Observation 1: According to the current RRC spec, the IE AppLayerMeasConfig (including both rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5) can be used by MN and/or SN to indicate configuration of application layer measurements for the corresponding node.

Observation 2: For rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5, since they are defined as Optional NEED R, the UE releases the current value if it receives a message with the field absent.

Observation 3: In NR-DC, the current design for IE AppLayerMeasConfig implies that configuring SRB4 or SRB5 segmentation on one node will impact the segmentation enabling/disabling on the other node.

Observation 4: Based on the current design for IE AppLayerMeasConfig, it requires coordination between the MN and SN regarding the SRB4 and SRB5 segmentation status.

Observation 5: Independently configuring the SRB4 and SRB5 segmentation status in MN and SN may require a redesign of the RRC IE for segmentation allowance, which would be a NBC change.

And proposed the following:

Proposal 1: In NR-DC, one node shall report its own SRB segmentation status to the peer node, which can be done either via RRC INM message or XnAP message.

Proposal 2: RAN2 acknowledges the issue raised by RAN3 and confirms that the exchange of SRB4/5 segmentation status is required in NR-DC.

Proposal 3: Reply to the RAN3 LS and let RAN3 to decide whether the XnAP message will be used for the SRB4/5 segmentation status exchange.

# Reference

[1] R3-247888 LS on QMC Coordination for RRC Segmentation in NR-DC

[2] R3-247655 Introduction to coordination for QoE RRC segmentation in NR-DC

[3] R3-247606 Corrections to QMC support in NR-DC for RRC Segmentation