3GPP TSG-RAN WG2 Meeting #129	R2-2500548

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on UL capacity enhancement for IoT NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

As one target of WID[1], support of capacity enhancement for UL needs to be studied and specified. One of the motivations is to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction. More specifically, it includes enhancement to transmit Msg3 without Msg1/Msg2 and enhancement to efficiently deliver Msg4.

- Support of Capacity enhancements for uplink - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In RAN2-127bis meeting, RAN2 agreed to continue working on a Contention-Based (CB) msg3 EDT mechanism as well as the Diversity Slotted-Aloha (DSA).

RAN2-127bis meeting Agreements: - RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19. - RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed. - At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. - CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA) - RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA) - At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission - The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA) - CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In RAN2-128 meeting, RAN2 progressed well on how to support CB-Msg3 in both SA and DSA cases but many FFS remain.

RAN2-128 meeting Agreements: - Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) - Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels - The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels) - There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) - The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific - There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) - For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time - For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs) - The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource. - RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

In this contribution, we provide further view on the remaining issues for CB-msg3 EDT, DSA as well as the enhancement for Msg4.

# 2	Discussion

## 2.1	CE level for CB-Msg3 EDT

It was agreed in RAN2-128 meeting that the UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS. But it is open whether the maximum TBS threshold is the same or different for different CE levels. The maximum TBS threshold for CB-Msg3 is similar to the threshold used to decide whether UE can trigger the legacy EDT transmission. As defined in TS36.331 section 5.3.3.1b, the UE can initiate legacy CP or UP EDT when the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS signalled in edt-TBS. The edt-TBS is defined as per-CE level, which is given in RACH-CE-LevelInfo-r13. Since the legacy EDT and CB-Msg3 EDT are similar from the Msg3 data transmission point of view, we believe the same principle used for legacy EDT can be applied to CB-Msg3 EDT. The per-CE level TBS threshold enables the flexibility to configure different thresholds considering the repetition request and resource cost for different CE levels.

edt-TBS Largest TBS for Msg3 for a CE level applicable to a UE performing EDT. Value in bits. Value b328 corresponds to 328 bits, b408 corresponds to 408 bits and so on. Additionally, value b1000or456 corresponds to 1000 bits for CE levels 0 and 1, and 456 bits for CE levels 2 and 3. See TS 36.213 [23].

Observation 1: The TBS threshold for legacy EDT is defined as per Coverage Enhancement level.

Proposal 1: The maximum TBS for CB-Msg3 can be defined differently for different Coverage Enhancement (CE) levels.

In legacy EDT, there is some optimization to enable UE to select smaller TBS than edt-TBS for Msg3.  The UEs can adapt their TBS selection based on their specific coverage conditions and pending data size, improving reliability and efficiency (e.g., to avoid padding bits to be transmitted in vain). If the UE selects a smaller TBS, the number of UL transmission repetitions should be adapted as well.

edt-SmallTBS-Enabled Value TRUE indicates UE performing EDT is allowed to select TBS smaller than edt-TBS for Msg3 in the corresponding CE level, as specified in TS 36.213 [23]. Excerpt from 36.213 section 16.3.3: … - if the UE is configured with higher layer parameter edt-SmallTBS-Enabled set to 'false', - the TBS is given by higher layer parameter edt-TBS - otherwise, - the UE selects a TBS from the allowed TBS values according to Table 16.3.3-2 - the repetition number for Msg3 is the smallest integer multiple of L value that is equal to or larger than where is the selected TBS for Msg3, and is given by higher layer parameter edt-TBS

For CB-Msg3, it worth to discuss whether the similar mechanism should be introduced. Or alternatively, NW may configure multiple TBS for a CE level hence UE can select the TBS based on its pending data size to avoid unnecessary bits padding.

Proposal 2: RAN2 to discuss whether to introduce a flexible TBS selection for CB-Msg3 based on UE’s pending data size.

For the initial CE level selection procedure for CB-Msg3, RAN2 agreed to reuse the existing CE level selection for the RACH procedure while it is FFS whether a separate set of thresholds is needed. In the RACH procedure, the CE level is determined based on the measured RSRP compared to thresholds configured by upper layers.

Excerpt from 36.321 section 5.1.1: - if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage: … - else if the RSRP threshold of enhanced coverage level 2 configured by upper layers in rsrp-ThresholdsPrachInfoList and the measured RSRP is less than the RSRP threshold of enhanced coverage level 2 and the UE is capable of enhanced coverage level 2 then: - the MAC entity considers to be in enhanced coverage level 2; - else if the measured RSRP is less than the RSRP threshold of enhanced coverage level 1 as configured by upper layers in rsrp-ThresholdsPrachInfoList then: - the MAC entity considers to be in enhanced coverage level 1; - else: - the MAC entity considers to be in enhanced coverage level 0;

The principle can be illustrated in Figure1 below.

Figure 1: Example parameters setting for 3 Coverage Levels for NB-IoT in legacy RACH procedure

To determine three CE levels in RACH procedure for NB-IoT, only two thresholds should be defined in rsrp-ThresholdsPrachInfoList. If the UE’s measured RSRP is less than the RSRP threshold of enhanced coverage level 2 (prachRsrpThreshold2 in Figure 1), the UE will select CE level 2. In other words, if the measured RSRP is above the minimum RSRP for cell selection/reselection (e.g., q-RxlevMin) but is less than the RSRP threshold of enhanced coverage level 2, the UE will select CE level 2 for RACH procedure.

Observation 2: For NB-IoT RACH CE level selection, the UE selects CE level 2 when the measured RSRP is between the minimum RSRP for cell selection/reselection and the threshold for enhanced coverage level 2.

The principle of Observation 2 may not be suitable for CB-Msg3 CE determination. As the CB-Msg3 will directly transmit the PUSCH via Msg3, the RSRP-based CE level selection should ensure the uplink configuration in terms of MCS and resources is suitable for the current radio conditions. Depending on how the network defines the MCS and PRB resources for a CE level, there should be a dedicated RSRP threshold that determines whether CB-Msg3 can be used in the resource for the CE level. Furthermore, the RSRP requirement for RACH and PUSCH can be different due to the decoding requirements may be different in network side. So, we propose to have a separate set of thresholds for CB-Msg3 CE level selection. The UE can select a CE level if the measured RSRP is above the RSRP threshold for the corresponding CE level.

Figure 2: Example parameters setting for 3 Coverage Levels for CB-Msg3 procedure

Proposal 3: Introduce a separate set of thresholds specifically for the CE level selection of CB-Msg3.

Proposal 4: The UE selects a CE level only if the measured RSRP exceeds the RSRP threshold defined for that specific CE level.

In last RAN2 meeting, the discussion for CE level determination is for the initial selection. Another important issue to consider is the CE level increase after consecutive CB-Msg3 failures. I.e., if the CB-Msg3 fails multiple times in the selected CE level, whether the UE should increase the CE level hence it can have multiple CB-Msg3 transmission retries using the next CE level. In our understanding, if the transmission failures are caused by RF conditions, it is reasonable to have a higher CE level which is typically configured with higher number of repetitions to support more robust transmission scheme. However, if the transmission failures are caused by resource collision due to the limited CB-Msg3 resource, it seems not reasonable to use the next CE level which may consume more resources.

Observation 3: In case of consecutive CB-Msg3 transmission failures, the increase of the CE level should take into account whether it is caused by poor RF conditions or resource collisions.

Due to the limited CB-Msg3 resource, the PUSCH resource collision may be worse than the legacy RACH procedure considering more preamble resource (e.g., 64) can be allocated for a RO than the PUSCH resource which can be allocated for CB-Msg3 occasion. To control the unnecessary CE level increase due to CB-Msg3 PUSCH resource collision instead of bad RF conditions, we think the UE should check its RF condition when it decides to increase the CE level. If the UE’s measured RSRP is close to the CB-Msg3 threshold for current CE level (e.g., within the offset of the threshold), the UE could increase the CE level. Otherwise, if the UE’s measured RSRP is far above the CB-Msg3 threshold for current CE level (which means UE is still in good RF conditions), the UE can stay at current CE level for more attempts to fight against the resource collision.

Figure 3: Example parameters setting for CE level increase for CB-Msg3 procedure

Proposal 5: If consecutive CB-Msg3 failures exceed the threshold, the UE should check its measured RSRP to decide whether to increase its CE level.

Proposal 6: Introduce an offset for CB-Msg3 CE level increase. The UE may move to next CE level only if the measured RSRP is within the offset range of the current CE level's CB-Msg3 threshold.

Regarding how many CE levels can be supported for CB-Msg3 transmissions, we think it can reuse the legacy mechanism. That is, 4 CE levels for eMTC and 3 CE levels for NB-IoT.

Proposal 7: Support up to 4 CE levels for eMTC CB-Msg3 and 3 CE levels for NB-IoT CB-Msg3.

In last RAN2 meeting, there was some discussion on whether a different minimum threshold should be defined in case CB-msg3 EDT is combined with OCC. Based on the LS response from RAN1 [2], it seems the detailed design for the OCC is still ongoing and whether the OCC can be applied to Msg3-EDT depends on the power imbalance. RAN2 may wait for more RAN1 progress to decide whether and how to introduce the parameters for OCC in different CE level.

RAN1 LS response: The OCC solution that RAN1 is working on may be applied to CB-Msg3 NPUSCH format 1 single tone at least for OCC length 2, for the subcarrier spacing(s) for which CB-Msg3 is supported, when the power imbalance is small. The design details for the OCC solution are ongoing.

Proposal 8: RAN2 to wait for further progress in RAN1 to decide whether and how to introduce OCC parameters for different CE levels.

## 2.2	Contention Resolution for CB-Msg3 EDT

For CB-Msg3 EDT transmission, since there is no Msg2 to schedule the Msg3 transmission, the TC-RNTI to be used for Msg3 transmission and Msg4 scheduling cannot be allocated by the eNB and indicated in RAR. In the RAN2-127bis meeting, it was agreed the RNTI used to schedule Msg4 is derived from the PUSCH resource corresponding to the Msg3 transmission occasion while it is not clear whether the derived RNTI can be used for Msg3 transmission. According to TS36.213, in legacy, the TC-RNTI assigned by RAR can be used for scrambling initialization of Msg3 NPUSCH transmission. For an RRC idle UE which triggers the CB-Msg3 transmission, it is natural to use the RNTI derived based on the PUSCH resource for the Msg3 transmission.

Excerpt from TS 36.213: ### 16.5.1 UE procedure for transmitting format 1 narrowband physical uplink shared channel … If a Temporary C-RNTI is set by higher layers, the scrambling initialization of NPUSCH corresponding to the Narrowband Random Access Response Grant in Clause 16.3.3 and any NPUSCH retransmission(s) for the same transport block is by Temporary C-RNTI. Otherwise, the scrambling initialization of NPUSCH corresponding to the Narrowband Random Access Response Grant in Clause 16.3.3 and any NPUSCH retransmission(s) for the same transport block is by C-RNTI.

Proposal 9: The RNTI used for scrambling initialization of PUSCH transmission for CB-Msg3 is derived based on the resource associated to the PUSCH occasion used for the transmission.

In legacy 4-step RACH, the RACH collisions are handled using the contention resolution scheme, which relies on uniquely identifying the UEs in Msg3 (i.e., first 48 bits of CCCH SDU) and NW acknowledgement in Msg4. In legacy PUR mechanism (Msg3 transmission without Msg1/RAR), there is no need for contention resolution since the PUR resource allocated via RRCConnectionRelease message is dedicated to a specific UE (i.e., contention free resource). For CB-Msg3 transmission in shared resource, there may occur Msg3 collision between UEs. Therefore, the contention resolution scheme should be supported. The most straightforward way is to reuse the contention resolution scheme in 4-step RACH.

Proposal 10: Reuse the contention resolution scheme in legacy 4-step RACH for CB-Msg3 EDT transmission.

After the end of all repetition of CB-Msg3 PUSCH transmission, it was agreed in RAN2-128 meeting that the UE starts a window for response reception taking UE-eNB RTT into account. However, it is FFS if we need to consider additional delay e.g. for the processing time.

In TS36.321, the start time of the Msg3 response window were defined in both the legacy RACH and PUR, in which 4 ms were considered in PUR while it is not added in RACH contention resolution. For CB-Msg3, we tend to keep the specification simple and align the UE behavior with the contention resolution timer as defined in legacy RACH procedure.

Proposal 11: The UE starts a window for response reception in the subframe containing the last PUSCH repetition plus UE-eNB RTT, without needing to consider additional delays (e.g., for the processing time).

TS 36.321 5.1.5 for Contention Resolution: Once Msg3 is transmitted, the MAC entity shall: - if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage: - if Msg3 is transmitted on a non-terrestrial network: … - start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission of the bundle in the subframe containing the last repetition of the corresponding PUSCH transmission plus UE-eNB RTT. TS 36.321 5.4.7 for Msg3 response window in PUR: After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer: - if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT.

## 2.3	CB-Msg3 EDT Fallback

In legacy EDT, the CP-EDT and UP-EDT procedure can be used by network to trigger RRC connection establishment (i.e. fallback to RRC Connection setup). For example, if the MO data trigger further downlink information from application, then NW may move the UE to RRC connected mode.

Figure 4: CB-Msg3 CP-EDT fallback to RRC connection establishment

For CB-Msg3 EDT, the same mechanism can also be applied.

Proposal 12: The CB-Msg3 EDT can fallback to legacy RRC connection establishment procedure if NW decides to move the UE in RRC_CONNECTED mode.

If the UE moves to RRC Connected mode, the RNTI derived from the resource associated with the CB-Msg3 EDT PUSCH occasion cannot be used as the dedicated C-RNTI for the UE for further dedicated communications between the UE and the NW. This is because it may be derived based on the PUSCH resource used for CB-Msg3 transmission by another UE later on, while the UE is still in RRC Connected mode. Therefore, a dedicated C-RNTI needs to be allocated and indicated in Msg4 for each UE that needs to establish the RRC connection and perform the further communication with the eNB.

Proposal 13: For CB-Msg3 EDT, a dedicated RNTI needs to be indicated in Msg4 for UE that fallback to legacy RRC connection establishment.

As described in section 2.1, the CB-Msg3 procedure may fail due to, e.g., RF conditions or collision. After a number of CB-Msg3 transmission failures, the UE may be fallback to legacy RACH procedures such as 4-step RACH or legacy RACH-based EDT procedure. Furthermore, it is important to ensure that the UE may have a successful data transmission within a time limit, assuming that the UE has periodic data transmission requirement as applied for the legacy PUR. For PUR, compared to 4-step RACH or RACH-based EDT procedure, Msg1/Msg2 can be skipped. Therefore, if there is PUR resource configured for the UE, the fallback from CB-Msg3 EDT to PUR may also be considered, to save the UL data transmission delay.

Proposal 14: The UE may fallback from CB-Msg3 EDT to 4-step RACH, EDT or PUR.

## 2.4	CB-Msg3 EDT Collision Control

In the RAN2-128 meeting, it was agreed the cell-specific PUSCH resource for CB-Msg3 transmission is configured at least via system information. For each of the CE levels, the configured CB-Msg3 resource is shared between UEs and multiple UEs may transmit Msg3 simultaneously, thus it may cause a collision which will bring some adverse consequence such as the increased signalling overhead, additional UE power consumption and waste of the resources. Since the CB-Msg3 EDT resource configuration is provided to UE via SIB, the NW has no idea how many idle UEs will use the resource simultaneously which may cause unexpected Msg3 collision. To avoid very high packet loss due to a high collision rate, we believe the NW should have a mechanism to control the possible Msg3 collision in order to mitigate the Msg3 collision impact. The legacy access barring check (as defined in TS36.331 clause 5.3.3.11) to control the RACH collision can be used as baseline.

Proposal 15: Since the cell-specific CB-Msg3 EDT resource is provided via SIB, RAN2 to introduce a mechanism to control Msg3 collisions. The legacy access barring check mechanism to control RACH collisions can be used as a baseline. FFS on the details.

## 2.5	Power Control for CB-Msg3 EDT

For PUSCH transmission, UE shall determine its UL transmission power as specified in TS36.213. In legacy PUR, the eNB configures the UL power control parameter for the PUSCH transmission in the PUR-Config via RRC. Then the UE determines the PUSCH transmission power based on the configured parameter (e.g., p0-UE-NPUSCH and alpha), as specified in TS36.213 section 16.2.1.

For NPUSCH (re)transmissions corresponding to the random access response grant if enhanced random access power control is not applied, and for all other NPUSCH transmissions except for NPUSCH (re)transmission corresponding to preconfigured uplink resource, when the number of repetitions of the allocated NPUSCH RUs is greater than 2: [dBm] otherwise PPUSCH,c(i)=min&PCMAX,c(i),&10log10(MNPUSCH,c(i))+PO_NPUSCH,c(j)+αc(j)⋅PLc+ΔTF,c(i)) [dBm]

For the PUSCH transmission for CB-Msg3 EDT, the UE needs to determine UL transmission power for direct PUSCH transmission as well.  Similar to PUR, we think the dedicated parameter (e.g., p0-UE-NPUSCH and alpha) for CB-Msg3 EDT transmission should be defined to faciliate the PUSCH transmission power calculation.

Proposal 16: Define dedicated parameters (e.g., p0-UE-NPUSCH and alpha) for CB-Msg3 EDT transmission.

Furthermore, in legacy Msg3 UL transmission power determination, there is a mechanism to dynamically adjust the target power at the receiver side (P0_NPUSCH). i.e., based on the number of preamble transmissions attempted before the UE receives the current UL grant (RAR) to transmit message 3 (e.g., PREAMBLE_TRANSMISSION_COUNTER_CE as defined in TS36.321). The UE increases its UL transmission power after multiple attempts, and according to the power ramping step, which is beneficial to increase the successful PUSCH decoding rate.

- the MSG3_RECEIVED_TARGET_POWER is set to preambleInitialReceivedTargetPower + (PREAMBLE_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep;

For CB-Msg3 transmissions, as there is no preamble attempts in Msg1 (since no Msg1 is transmitted), the UE may fail multiple times for Msg3 transmissions attempts due to RF conditions or resource collisions. In this case, it may be also beneficial to consider the number of failed attempts to decide the UL transmission power and potential transmit power ramping.

Proposal 17: RAN2 to discuss whether the number of failed CB-Msg3 attempts should be considered for UL transmission power determination.

## 2.6	CB-Msg3 replicas transmission in DSA

For DSA, the UE is allowed to transmit more than one copy of Msg3 on the contention-based resources (i.e., more than one replica). The NW individually decodes each replica without soft combining. If any of the replicas is successfully decoded by NW, the NW can successfully receive the packet accommodated in Msg3.  Firstly, we understand the DSA mechanism is not the same as a reattempt of the Msg3 transmission after a failure, i.e., it is expected in DSA that all or at least multiple replicas of Msg3 may be transmitted before UE declaring the contention resolution failure.

Observation 4: It is expected in DSA that all or at least multiple replicas of Msg3 may be transmitted before UE declaring the contention resolution failure.

For CB-Msg3 replicas, RAN2 agreed there will be a configurable time window for DSA CB-Msg3 occasion selection. It is FFS on the details, for example when the time window starts. In our understanding, after network configures the time window, it is UE who selects the occasions for Msg3 transmission while there is no need for network side to maintain the window (e.g., when the NW configured window for replicas transmission should be started and stopped). This is because for DSA there is no requirement for replicas combination in network. Based on that, we think the UE should start the window once it starts to transmit the first replica.

Proposal 18: The NW configures a time window for the transmission of DSA CB-Msg3, within which the UE is expected to complete the transmission of the required number of replicas. Once the UE starts transmitting its first replica, it initiates the time window for selecting DSA CB-Msg3 occasions.

## 2.7	Msg4 reception for DSA

After Msg3 replica transmission, in the RAN2-128 meeting discussion, it is quite open on how UE monitor the Msg4 PDCCH for different replicas.

The first question is whether one or multiple PDCCH monitoring window(s) should be started for the Msg4 reception. For the first Msg3 replica, the UE should start a window for response reception in the subframe containing the last PUSCH repetition (of the first replica) plus UE-eNB RTT in order to receive the possible Msg4 response for the first replica. In one way (Option#1), the UE may apply the window with quite long duration to cover the whole period to receive possible Msg4 reception responding to the first or subsequent Msg3 replicas. However, this may cause UE and NW misalignment because NW may miss the first Msg3 replica reception hence UE and NW have different understanding on when the window is started and the window length. To avoid the NW and UE misalignment, the Msg4 reception window should be restarted after each Msg3 replica.

Observation 5: To avoid misalignment between the NW and UE on the window start and window length, the Msg4 reception window should be started and restarted after each replica.

In the other way (Option#2), the UE may start an individual Msg4 reception window for each of the replica. As each Msg4 reception window is (re)started for each Msg3 replica, there is no NW and UE misalignment issue no matter whether NW decoded a certain replica successfully. In our understanding, both Option#1 and Option#2 can work but it seems the Option#2 is a simpler way-forward because it is easy for window modelling and less impact to the specification.

Proposal 19: The UE starts an individual Msg4 reception window for each Msg3 replica.

RAN2 agreed that the number of replicas for DSA configured by the NW can be: 1 (SA), 2, 3, 4. This means the UE may start up to four Msg4 reception windows for DSA. In legacy, if the Msg4 reception window expires while no PDCCH for Msg4 has been received, the UE may consider the Contention Resolution failure. For DSA with multiple started Msg4 reception windows, it seems natural to consider that the UE should declare Contention Resolution failure only if no PDCCH for Msg4 has been received when all the Msg4 reception windows have expired.

Proposal 20: The UE considers a Contention Resolution failure only if no PDCCH for Msg4 is received when all Msg4 reception windows have expired.

As RAN2 agreed the RNTI used to schedule Msg4 is derived from the resource used for CB-Msg3 transmission, it may cause trouble in DSA in which multiple RNTIs may be derived based on multiple Msg3 replicas.

Figure 5: Msg3 transmission and Msg4 monitoring in DSA using Msg3 resource derived RNTI

As illustrated in Figure 5, the RNTI X, Y and Z may be derived based on the individual Msg3 replica resources used for replica #1, #2 and #3. On one hand, it may cause the UE to monitor multiple RNTI(s) for Msg4 after it sends multiple replicas. Since the maximum replicas can be up to 4, the UE may monitor 4 RNTIs for Msg4. On the other hand, NW should avoid duplicated and redundant Msg4 to different replica of Msg3 as different RNTI may be derived for different replica of same Msg3. As NW cannot identify the replica of the same UE just based on RNTI, NW has to store and compare the decoded Msg3 SDU contents from different replicas to identify the same UE, but it may result in complex NW implementation.

Observation 6: Due to up to 4 RNTIs can be derived in DSA for different replicas, it may cause impacts to both UE and NW: 1) UE has to monitor up to 4 RNTIs for Msg4 reception. 2) NW has to store and compare the Msg3 content for up to 4 RNTIs to avoid duplicated Msg4 delivery.

In our understanding, the DSA impact to UE and NW should be minimised as much as possible. Though multiple RNTIs may be derived in DSA, the UE needs to monitor only one RNTI for Msg4 reception. I.e., one RNTI should be selected for all the replicas with the same Msg3 content.

Proposal 21: UE monitors only one RNTI for the Msg4 corresponding to all possible replicas with the same Msg3 content.

Regarding how the UE and NW have the common understanding on the RNTI to be used for Msg4 reception, there may have two options:

- Option1:  The RNTI for Msg4 reception is derived from the first replica occasion, if the UE and NW have the common understanding of the replica pattern.

- Option2:  The UE includes the RNTI information for Msg4 reception in the Msg3 replicas.

Both options enable the NW to avoid duplicated Msg4 delivery. Since the discussion on whether and how network can inference the positions of the DSA replicas sent by UE is still ongoing (excerpt as below), the conclusion may impact whether Option1 above can be adopted. How to derive the RNTI used for Msg4 reception can be further disussed.

R2-2410875 Implicit pointer for locating DSA replicas for EDT of CB-Msg3 DLR, ESA, Inmarsat, Viasat discussion Proposal 1: RAN2 to consider the use of an implicit pointer to allow the network infer the positions of the DSA replicas sent by a UE over the available set of EDT occasions. Can continue the discussion in the next meeting

Proposal 22: RAN2 to further discuss how to decide the RNTI used for Msg4 reception corresponding to CB-Msg3 using DSA.

## 2.8	CB-Msg3 replicas transmission early termination

In RAN2-128 meeting, RAN2 reached an assumption that there will be scenarios where it’s possible that a UE receives a CB-msg4 before it transmits all replicas, hence the UE shall stop transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity.

In our understanding, this is quite important aspect to support the early termination of the replica transmissions once Msg3 is successfully decoded by NW. Firstly, as agreed in last meeting, the replica occasions are distributed in a NW configured window hence the distance between the replicas may span for quite long duration. The NW may send the Msg4 response during the Msg3 span once it decoded a Msg3 successfully. Secondly, due to repetitions may be configured together with DSA to extend the UL coverage for IoT, the resource utilization efficiency may be reduced with the increase of the replica number. When the NW successfully decodes a Msg3, it’s better to respond with Msg4 as early as possible hence UE can stop the remaining Msg3 replica transmissions.

RAN2-128 agreements: - RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started) - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

Observation 7: It is a system resource waste if NW decoded a Msg3 successfully while UE continues to transmit all the remaining Msg3 replica transmissions.

Proposal 23: RAN2 confirm the assumption that, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity.

## 2.9	Efficient Msg4 delivery

On the objective of efficient delivery of msg4, according to the WID [1], the intention is to identify methods to de-couple the UL from the DL as much as possible hence unlock the additional UL capacity potential. We understand it is to avoid the PDCCH/PDSCH congestion which may impact the UL scheduling as well as the UL capacity. Therefore, how to reduce the Msg4 overhead should be studied.

In legacy, a dedicated Msg4 with both PDCCH and PDSCH is always needed for the corresponding Msg3 for contention resolution. To reduce the Mgs4 overhead, one possible enhancement is to reduce the PDCCH usage for Msg4 scheduling. For example, it seems a possible optimization way-forward to support one Msg4 to deliver contention resolution result for multiple UEs to save the scheduling PDCCH at least. The other possible enhancement is to reduce the PDSCH overhead for Msg4 content delivery. For example, some of the IEs in Msg4 can be reduced or reused, especially considering the case when multiple UEs can be scheduled in a single Msg4.

Proposal 24: To support efficient delivery of Msg4, RAN2 to consider below aspects:

- aspect1: Multiplex several UE contention-resolution identities into one Msg4.

- aspect2: Reduce common IEs if multiple UEs are addressed in one Msg4.

# 3	Conclusion

This document has made the following observations and proposals:

CE level for CB-Msg3 EDT:

Observation 1: The TBS threshold for legacy EDT is defined as per Coverage Enhancement level.

Proposal 1: The maximum TBS for CB-Msg3 can be defined differently for different Coverage Enhancement (CE) levels.

Proposal 2: RAN2 to discuss whether to introduce a flexible TBS selection for CB-Msg3 based on UE’s pending data size.

Observation 2: For NB-IoT RACH CE level selection, the UE selects CE level 2 when the measured RSRP is between the minimum RSRP for cell selection/reselection and the threshold for enhanced coverage level 2.

Proposal 3: Introduce a separate set of thresholds specifically for the CE level selection of CB-Msg3.

Proposal 4: The UE selects a CE level only if the measured RSRP exceeds the RSRP threshold defined for that specific CE level.

Observation 3: In case of consecutive CB-Msg3 transmission failures, the increase of the CE level should take into account whether it is caused by poor RF conditions or resource collisions.

Proposal 5: If consecutive CB-Msg3 failures exceed the threshold, the UE should check its measured RSRP to decide whether to increase its CE level.

Proposal 6: Introduce an offset for CB-Msg3 CE level increase. The UE may move to next CE level only if the measured RSRP is within the offset range of the current CE level's CB-Msg3 threshold.

Proposal 7: Support up to 4 CE levels for eMTC CB-Msg3 and 3 CE levels for NB-IoT CB-Msg3.

Proposal 8: RAN2 to wait for further progress in RAN1 to decide whether and how to introduce OCC parameters for different CE levels.

Contention Resolution for CB-Msg3 EDT:

Proposal 9: The RNTI used for scrambling initialization of PUSCH transmission for CB-Msg3 is derived based on the resource associated to the PUSCH occasion used for the transmission.

Proposal 10: Reuse the contention resolution scheme in legacy 4-step RACH for CB-Msg3 EDT transmission.

Proposal 11: The UE starts a window for response reception in the subframe containing the last PUSCH repetition plus UE-eNB RTT, without needing to consider additional delays (e.g., for the processing time).

CB-Msg3 EDT Fallback:

Proposal 12: The CB-Msg3 EDT can fallback to legacy RRC connection establishment procedure if NW decides to move the UE in RRC_CONNECTED mode.

Proposal 13: For CB-Msg3 EDT, a dedicated RNTI needs to be indicated in Msg4 for UE that fallback to legacy RRC connection establishment.

Proposal 14: The UE may fallback from CB-Msg3 EDT to 4-step RACH, EDT or PUR.

CB-Msg3 EDT Collision Control:

Proposal 15: Since the cell-specific CB-Msg3 EDT resource is provided via SIB, RAN2 to introduce a mechanism to control Msg3 collisions. The legacy access barring check mechanism to control RACH collisions can be used as a baseline. FFS on the details.

Power Control for CB-Msg3 EDT:

Proposal 16: Define dedicated parameters (e.g., p0-UE-NPUSCH and alpha) for CB-Msg3 EDT transmission.

Proposal 17: RAN2 to discuss whether the number of failed CB-Msg3 attempts should be considered for UL transmission power determination.

CB-Msg3 replicas transmission in DSA:

Observation 4: It is expected in DSA that all or at least multiple replicas of Msg3 may be transmitted before UE declaring the contention resolution failure.

Proposal 18: The NW configures a time window for the transmission of DSA CB-Msg3, within which the UE is expected to complete the transmission of the required number of replicas. Once the UE starts transmitting its first replica, it initiates the time window for selecting DSA CB-Msg3 occasions.

Msg4 reception for DSA:

Observation 5: To avoid misalignment between the NW and UE on the window start and window length, the Msg4 reception window should be started and restarted after each replica.

Proposal 19: The UE starts an individual Msg4 reception window for each Msg3 replica.

Proposal 20: The UE considers a Contention Resolution failure only if no PDCCH for Msg4 is received when all Msg4 reception windows have expired.

Observation 6: Due to up to 4 RNTIs can be derived in DSA for different replicas, it may cause impacts to both UE and NW: 1) UE has to monitor up to 4 RNTIs for Msg4 reception. 2) NW has to store and compare the Msg3 content for up to 4 RNTIs to avoid duplicated Msg4 delivery.

Proposal 21: UE monitors only one RNTI for the Msg4 corresponding to all possible replicas with the same Msg3 content.

Proposal 22: RAN2 to further discuss how to decide the RNTI used for Msg4 reception corresponding to CB-Msg3 using DSA.

CB-Msg3 replicas transmission early termination:

Observation 7: It is a system resource waste if NW decoded a Msg3 successfully while UE continues to transmit all the remaining Msg3 replica transmissions.

Proposal 23: RAN2 confirm the assumption that, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity.

Efficient Msg4 delivery:

Proposal 24: To support efficient delivery of Msg4, RAN2 to consider below aspects:

- aspect1: Multiplex several UE contention-resolution identities into one Msg4.

- aspect2: Reduce common IEs if multiple UEs are addressed in one Msg4.

# Reference

[1] RP- RP-243278	Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2] R1-2410895	Reply LS on OCC for CB-msg3 NPUSCH