3GPP TSG-RAN WG2 Meeting #129	R2-2500556

Athens, Greece, 17 – 21 February 2025

Agenda item:	7.8.1

Source:	Nokia

Title:	carrierBandwidth configuration for less than 5MHz

WID/SID:	NR_FR1_lessthan_5MHz_BW-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In last RAN2 meeting we sent LS to RAN4 regarding carrierBandWidth configuration in R2-2409409. In order to hopefully find common understanding how UE handles carrierBandwidth in case less than 5MHz operation we would like to provide status of RAN4 specification. RAN4 responsed to RAN2 in R4-240382:

# 2	Discussion

In RAN4 it has been specified for band n100 following:

- PCell or SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3.

- PCell or SCell with 20 PRB transmission bandwidth configuration within 5 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3.

Therefore, UE can determine 12 or 20 PRB transmission bandwidth configuration should be used instead of 15 or 25 PRB transmission bandwidth configuration, respectively, even though carrierBandwidth is configured to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling.

Thus it is feasible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling.

Observation 1: It is possible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling.

The LS exchange was initiated from the paper R2-2409055 in which it was stated:

As noted for observation 1 – in RAN4 specification it is already clear that if 15/25PRB are indicated in carrierBandWidth then UE can determine while operating on band n100 that instead 12/20PRB is applicable.

And like RAN4 responded in the response LS:

RAN4 would like to thank RAN2 for their LS R2-2409409.

RAN4 confirms that it is feasible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

In addition, RAN4 would like to provide the background that 12 PRB and 20 PRB transmission bandwidth configuration are two special cases only for band n100, and only can be configured with SS Block frequency positions in additional GSCN parameters defined in TS 38.101-1 Table 5.4.3.1-3.

Therefore, in these two special cases UE can determine 12 or 20 PRB transmission bandwidth configuration should be used instead of 15 or 25 PRB transmission bandwidth configuration, respectively, even though carrierBandwidth is configured to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

Thus we propose

Proposal 1: Keep existing LessThan5MHz condition

# 3	Conclusion

Observation 1: It is possible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling.

Proposal 1: Keep existing LessThan5MHz condition