3GPP TSG-RAN WG2 Meeting #129													     R2-2500558

Athens, Greece, 17-21 Feb, 2025

Agenda item:	8.10.2

Source:	Apple

Title:	Outstanding issues of MRO

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide or views on the remaining details that need to be discussed on SON for LTM.

# 2   	Discussion

### 2.1 	LTM

In RAN2#128 an issue of LTM switch followed by BFR brough up in [2] was discussed [1]. There was no consensus on how to proceed and the issue was postponed with the intention to rediscuss it in RAN2#129.

To recap, according to [2], there two somewhat different cases:

- LTM cell switch (to the correct cell) failure due to wrong beam

- Successful LTM cell switch, shortly followed by BFR

These cases are illustrated in figures 1 and 2 below (borrowed from [2]).

While the cases are different, the root cause is the same – LTM cell switch indicates a wrong beam. Since the root cause is the same, a potential corrective action by the network should also be the same.

First off, we would like to remind everybody that in Rel-18 only intra-CU LTM is supported. This means, among other things, that while the source and the target cells are different, they are controlled by the same CU and generally it is very easy for the network to exchange information (e.g. configuration information) between the target and the source cells.

Observation 1: in Rel-18 only intra-CU LTM is supported, therefore it is very easy for the network to exchange information (e.g. configuration information) between the target and the source cells.

In particular, there has been defined (in RAN3) TRP INFORMATION REQUEST/RESPONSE F1-AP signaling, which allows CU to be aware of beam configurations of both DUs involved in an LTM cell switch. Therefore, such misconfiguration which would result in LTM failure due to wrong beam configuration is:

- Very rare, and

- Can be corrected by the network using existing F1-AP signaling

Observation 2: since CU is aware of both DU’s beam configurations, such errors should be very rare.

Finally, even if (either of the) two issues being discussed occurs, the CU would know it (even without new signaling).

Observation 3: the CU should be able to detect such errors; if there doubts RAN2 can send a LS to RAN3 to confirm.

Proposal 1: no air interface signaling enhancements are needed to address the LTM followed by BFR issues, as they can be addressed by network signaling (in RAN3).

### 2.2	CHO

For CHO with candidate SCGs, RAN2#128 agreed:

Agreements - RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements. - Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

First off, with regards to the agreement on time information to be included in SHR and SCGFailureInformation, an obvious solution is to re-use the existing IEs timeSinceSHR  and timeSCGFailure.

Observation 4: existing IEs can be reused for time information to be included in SHR and SCGFailureInformation.

Naturally, both SHR and SCGFailureInformation already contain measurements results through MeasResultNR, which in turn already contains the indication of choCandidate. Therefore, it appears that no new IEs are needed for CHO with candidate SCGs.

Observation 5: both SHR and SCGFailureInformation already contain measurements results and choCandidate indication.

Proposal 2: re-use existing IEs for the agreement to log measurement results of PCells and PSCells and the time information in SHR and SCGFailureInformation.

# 3	Proposals

Observation 1: in Rel-18 only intra-CU LTM is supported, therefore it is very easy for the network to exchange information (e.g. configuration information) between the target and the source cells.

Observation 2: since CU is aware of both DU’s beam configurations, such errors should be very rare.

Observation 3: the CU should be able to detect such errors; if there doubts RAN2 can send a LS to RAN3 to confirm.

Proposal 1: no air interface signaling enhancements are needed to address the LTM followed by BFR issues, as they can be addressed by network signaling (in RAN3).

Observation 4: existing IEs can be reused for time information to be included in SHR and SCGFailureInformation.

Observation 5: both SHR and SCGFailureInformation already contain measurements results and choCandidate indication.

Proposal 2: re-use existing IEs for the agreement to log measurement results of PCells and PSCells and the time information in SHR and SCGFailureInformation.

# 5	References

[1]		RAN2#128 meeting minutes

[2]		R2-2409754, Further considerations on MRO	ZTE