3GPP TSG-RAN WG2 Meeting #129	R2-2500562

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.13.4

Source: 	China Telecom

Title:	Discussion on service continuity for multi-hop relay

WID/SID:	NR_SL_relay_multihop

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, the following agreements on service continuity in multi-hop relay were reached for the first priority scenarios.

Agreements in RAN2#128 - For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused. - For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching. Spec impact will be minimised. - For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure. This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE. - The intra-gNB switching from indirect to direct path procedure defined in TS 38.300 for single hop case can be used as the baseline for multi-hop indirect to direct path switching. - Event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported. - The intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure. - The following measurement events can be reused in multi-hop indirect to single-hop indirect path switching: - Event Y2 - Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported. - Figure 1 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to direct path switching procedure to be captured in TS 38.300. - Figure 2 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to single-hop indirect path switching to be captured in TS 38.300.

In this contribution, we would like to share our opinions on service continuity for second priority scenarios in multi-hop U2N relay.

- Discussion

Intra-gNB direct to multi-hop indirect path switching

In Rel-17/18 U2N relay, the following events are introduced to support service continuity.

- Event X1: Serving L2 U2N Relay UE becomes worse than absolute threshold1 AND NR Cell becomes better than another absolute threshold2;

- Event X2: Serving L2 U2N Relay UE becomes worse than absolute threshold;

- Event Y1: PCell becomes worse than absolute threshold1 AND candidate L2 U2N Relay UE becomes better than another absolute threshold2;

- Event Y2: Candidate L2 U2N Relay UE becomes better than absolute threshold;

- Event Z1: Serving L2 U2N Relay UE becomes worse than absolute threshold1 AND candidate L2 U2N Relay UE becomes better than another absolute threshold2

Regarding intra-gNB direct to multi-hop indirect path switching, it may happen when PCell becomes worse and candidate U2N relay becomes better. In this case, Event Y1/Y2 could be used. When Uu quality between (remote) UE and PCell becomes worse (than threshold1) and the PC5 quality between (remote) UE and candidate U2N relay is better (than threshold2), the (remote) UE may initial the measurement report procedure. Based on the measurement report, the gNB may select a target relay within the same gNB and send RRCReconfiguration message to the (remote) UE to trigger the path switching procedure.

Proposal 1: For intra-gNB direct to multi-hop indirect path switching, (remote) UE switched to candidate U2N relay within the same gNB is supported. The target U2N relay could be same as or different from the previous serving cell of (remote) UE.

Proposal 2: Event Y1/ Y2 could be used for intra-gNB direct to multi-hop indirect path switching. The PC5 quality between the remote UE and candidate U2N relay/candidate first relay is considered for evaluating Event Y1/ Y2.

Intra-gNB single-hop indirect to multi-hop indirect path switching

For intra-gNB single-hop indirect to multi-hop indirect path switching, taking two-hop switching to single-hop as an example, there may be the following cases.

- Case D-1: Remote-relay1-gNB(cell1) TO remote-relay3-relay4-gNB(cell1)

- Case D-2: Remote-relay1-gNB(cell1) TO remote-relay3-relay4-gNB(cell2)

- Case D-3: Remote-relay1-gNB(Cell1) TO remote-relay2-relay1-gNB (Cell1)

- Case D-4: Remote-relay1-gNB(Cell1) TO remote-relay2-relay1-gNB (Cell2)

- Case D-5: Remote-relay1-gNB(cell1) TO remote-relay1-relay2-gNB(cell1)

- Case D-6: Remote-relay1-gNB(cell1) TO remote-relay1-relay2-gNB(cell2)

For Case D-1/2, it may happen when the serving U2N relay becomes worse and candidate U2N relay becomes better. In this case, Event Z1/Y2 could be used. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event Z1. When the PC5 quality between remote UE and the first relay is worse than threshold1 and PC5 quality between remote UE and/or a candidate relay becomes better than threshold2, the remote UE may initial the measurement report procedure. Based on the measurement report, the gNB may select a target U2N relay within the same gNB and send RRCReconfiguration message to the remote UE to trigger the path switching procedure.

For Case D-3/4, they are just special cases of Case D-1/2, which may happen when the serving U2N relay1 becomes worse and candidate U2N relay2 becomes better. Since the selection of target relay is up to gNB, no extra spec impact is foreseen to support these cases.

Proposal 3: For Intra-gNB single-hop indirect to multi-hop indirect path switching, remote UE switched to different relays within the same gNB is supported. The serving cell of the new relays could be the same as or different from the previous serving cell of the remote UE.

Proposal 4: Event Z1/Y2 could be used for intra-gNB multi-hop indirect to direct path switching. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event Z1.

For Case D-5/6, it may happen when Uu quality between relay1 and NR cell becomes worse and PC5 quality between relay1 and candidate relay becomes better. If Event Y1 is configured for relay1, relay1 may initial the measurement report procedure. Based on the measurement report, the gNB may switch relay1 onto indirect path via relay2. If the PC5 quality between remote UE and relay1 is stable, whether to support service continuity for remote UE in these scenarios shall be further discussed. Generally, it is more like a kind of “group HO”, which is not supported in previous release. In last RAN2 meeting, similar issue was discussed for intra-gNB multi-hop indirect to single-hop indirect path switching. The following agreement was reached.

Agreements in RAN2#128 - For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure. This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE.

For Case D-5/6 in intra-gNB single-hop indirect to multi-hop indirect path switching scenario, we think similar principle could be followed.

Proposal 5: For Case D-5/6 in intra-gNB single-hop indirect to multi-hop indirect path switching scenario, RAN2 does not specify anything to support these cases by using a single procedure. These cases can be handled by network implementation, e.g. by sequential path switches for the remote UE and the target intermediate relay UE.

- Conclusion

In this contribution, we discuss general aspects on service continuity second priority scenarios in multi-hop U2N relay.

Proposal 1: For intra-gNB direct to multi-hop indirect path switching, (remote) UE switched to candidate U2N relay within the same gNB is supported. The target U2N relay could be same as or different from the previous serving cell of (remote) UE.

Proposal 2: Event Y1/ Y2 could be used for intra-gNB direct to multi-hop indirect path switching. The PC5 quality between the remote UE and candidate U2N relay/candidate first relay is considered for evaluating Event Y1/ Y2.

Proposal 3: For Intra-gNB single-hop indirect to multi-hop indirect path switching, remote UE switched to different relays within the same gNB is supported. The serving cell of the new relays could be the same as or different from the previous serving cell of the remote UE.

Proposal 4: Event Z1/ Y2 could be used for intra-gNB multi-hop indirect to direct path switching. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event Z1.

Proposal 5: For Case D-5/6 in intra-gNB single-hop indirect to multi-hop indirect path switching scenario, RAN2 does not specify anything to support these cases by using a single procedure. These cases can be handled by network implementation, e.g. by sequential path switches for the remote UE and the target intermediate relay UE.

- Reference

RP-241609, New WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet

3GPP TS 38.300 18.2.0: "NR; Radio Resource Control (RRC) protocol specification".