3GPP TSG-RAN WG2 Meeting #129	           R2-2500569

Athens, Greece, Feb 17th – Feb 21st, 2025

Agenda item:	8.1.2.3

Source:	China Telecom

Title:	LCM for UE-sided model for positioning use case

Document for:	Discussion

- Introduction

As indicated by the agenda document, we will focus on the LCM for UE-sided model for positioning use case with high priority in this paper. The functionality management and data collection for the UE-sided model training for positioning use case will be discussed.

- Discussion

2.1	Functionality management

In TR 38.843, functionality activation/deactivation is considered as a part of LCM [1]. RAN2 also sent an LS to RAN1 to ask some questions about LCM for the UE-sided model for beam management, including functionality activation/deactivation related question. According to the reply LS and the related discussion in RAN1, companies have strong concerns for introducing explicitly functionality management procedure such as new procedure or signaling dedicated for AI/ML functionality management. From our point of view, it is better to try to reuse current procedure or signaling to realize functionality activation/deactivation in an implicit method.

Proposal 1: RAN2 try to reuse current procedure or signaling to realize functionality activation/deactivation in an implicit method.

Regarding functionality deactivation, during RAN2#128 meeting, RAN2 has the following agreements for functionality deactivation for the UE-sided model for BM case [2]. UE is not allowed for autonomously functionality deactivation. From our point of view, the principle for functionality deactivation also applies to the AI/ML based positioning use case, i.e. the UE is not allowed for autonomously AI based positioning functionality deactivation. Furthermore, since the LMF other than the gNB is responsible for the control of traditional positioning function, it is straightforward that LMF deactivates the AI based positioning functionality.

Agreements

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

Regarding functionality activation, RAN2#127bis meeting agreed the following baseline procedure for LCM for the UE-sided model for positioning use case 1 [3]. In our understanding, the LMF implicitly ‘activates’ the AI based positioning function by step 5 after the UE reporting the applicable functionality to the LMF.

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message.

Proposal 2: For UE-sided model, the LMF activates/deactivates the AI positioning functionality.

2.2	Data collection for UE-sided model training

Generally, the following aspects need to be considered for configuring the UE for training UE-sided model for AI/ML based positioning use case:

-	The consistency of NW-side additional condition across training and inference for UE-sided model

-	Who is expected to initiate data collection for the UE-sided model training

-	How to enable or configure the UE to perform data collection for the UE-sided model training

-	When and how to stop data collection for the UE-sided model training

Since the consistency of NW-side additional condition across training and inference for UE-sided model is still under discussion in RAN1, we mainly discuss the last three bullets. The UE at least need RS resource configuration used for data collection for model training, e.g. PRS configuration. Similar to the principle that data collection for training is under the network’s control for AI/ML based beam management case, and LMF is responsible for the control of positioning function, LMF can configure the UE to perform data collection for training with or without UE’s request.

Proposal 3: LMF can configure the UE to perform data collection for training with or without UE’s request.

Regarding when and how to stop data collection for the UE-sided model training, we think LMF can decide whether to stop data collection for training or not based on the information from the UE or the OTT server. For example, if the UE-sided model training is completed, the data collection for model training is expected to stop. In this situation, LMF may decide to stop the data collection for training if informed by the OTT server of the completion of model training or informed by the UE of the completion of model training.

Another example is, if the UE can’t perform data collection for training anymore, the data collection for model training is also expected to stop. In this situation, it is possible that the UE can report to the LMF via LPP message and then the LMF can decide to stop data collection for training. In summary, LMF decide to stop data collection for UE-sided model training for positioning use case, other than the ‘UE autonomously decided’ method. The LPP procedure and signaling design details can be left to stage 3.

Proposal 4: LMF decide whether to stop data collection for the UE-sided model training for positioning use case.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: RAN2 try to reuse current procedure or signaling to realize functionality activation/deactivation in an implicit method.

Proposal 2: For UE-sided model, the LMF activates/deactivates the AI positioning functionality.

Proposal 3: LMF can configure the UE to perform data collection for training with or without UE’s request.

Proposal 4: LMF decide whether to stop data collection for the UE-sided model training for positioning use case.

4	References

- [1] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- [2] Report of 3GPP TSG RAN WG2 meeting #128, Orlando, US.

- [3] Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China.