3GPP TSG-RAN WG2 Meeting #129	           R2-2500571

Athens, Greece, Feb 17th – Feb 21st, 2025

Agenda item:	8.10.4

Source:	China Telecom

Title:	Discussion on SON of intra-NTN mobility

Document for:	Discussion

- Introduction

RAN3 have made some progress of SON/MDT for intra-NTN mobility scenarios, and send an LS to ask RAN2 some questions regarding MRO and MDT for intra-NTN mobility. In this paper, we mainly discuss MRO for intra-NTN mobility aspects.

- Discussion

RAN3 have the following agreements related to MRO for intra-NTN mobility:

RAN3#124: MRO for intra-NTN mobility shall consider the following trigger conditions: - Measurement-based trigger condition; - A time-based trigger condition; - A location-based trigger condition; - A time-based and Measurement-based combined trigger condition; - A location-based and Measurement-based combined trigger condition; Do not consider MRO for feeder link switch. MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover. RAN3#125: MRO for CHO with a time-based trigger condition: Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell. Distinguish if the RLF happens before or after T1 Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell. RAN3#125bis: For location based trigger condition: Too late CHO Execution: RLF happens in the source cell before CHO execution. The UE re-establishes in a cell other than the source cell. Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell. For earth fixed case, there is no need to report the measured distance from the UE. FFS on the earth moving case. For joint trigger condition: Case 1: Too Late CHO Execution: RLF happens in the source cell. The UE re-establishes in the cell other than the source cell. Case 2: Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens in the target cell short after successful HO. The UE re-establishes in the source cell or a third cell. UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not. RAN3#126: WA: There is no need to report any other info in the RLF report for the location based CHO.

Regarding MRO for CHO with a time-based trigger condition, RAN3 agreed that network need information to distinguish if the RLF happens before or after T1. RAN3 identified potential solutions in previous meetings as the following:

Alt 1: the RLF report includes for single time-based trigger condition:

- Absolute time (measured UTC time) when RLF occurred

Alt 2: a flag to indicate whether the RLF happens before T1 or after T1

Alt 3: based on existing timers in RLF report.

The existing RLF report includes timeSinceFailure, timeConnFailure and timeSinceCHO-Reconfig. As pointed out by companies, the issue with reusing these timers for MRO for intra-NTN mobility is that the t1-Threshold defined in TS 38.331 is in 10ms units while the granularity of the existing timers in RLF report is in seconds or 100ms. The field description in TS 38.331 is as the following:

t1-Threshold The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900).

timeConnFailure This field is used to indicate the time elapsed since the last HO execution until connection failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (radio link or handover) failure. Value in seconds. The maximum value 172800 means 172800s or longer. In the case of failure(s) (either at source or at target or at both) associated to DAPS handover, this field indicates the time elapsed since the latest connection (radio link or handover) failure.

timeSinceCHO-Reconfig In case of handover failure, this field is used to indicate the time elapsed between the initiation of the last handover execution towards the target cell and the reception of the latest conditional reconfiguration. In case of radio link failure, this field is used to indicate the time elapsed between the radio link failure and the reception of the latest conditional reconfiguration while connected to the source PCell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

If timeSinceCHO-Reconfig is used to distinguish if the RLF happens before or after t1-Threshold, it would result in a wrong conclusion of comparing t1-Threshold and (the time CHO config is received + timeSinceCHO-Reconfig) in the network, because the network can’t know the true time of timeSinceCHO-Reconfig in 10ms, i.e. the reported value is 200ms, but the true time may be 150ms, 160ms, 170ms, 180ms or 190ms. On the other hand, the network need deduce the time when CHO config is received by the UE based on the absolute time when the network sent CHO config and propagation delay, which introduce additional uncertainty. Thus we think only Alt 1 and Alt 2 are feasible.

Compared with Alt2, Alt 1 provide additional information, i.e. the reference for the adjustment amount of t1-Threshold if the failure is caused by improper T1 setting. Therefore, it is beneficial to have the absolute time or the relative time between t1-Threshold and the time RLF occurs in the RLF report. Furthermore, reporting the relative time between t1-Threshold and the time RLF occurs has less overhead than the absolute time. We suggest to introduce the relative time between t1-Threshold in the RLF report.

Proposal 1: Regarding MRO for CHO with a time-based trigger condition, include relative time between the time RLF occurs and t1-Threshold in the RLF report.

As for case 2 in case of CHO with a time-based trigger condition, if the UEs report the absolute time when CHO execution failed or relative time between CHO execution failure and t1-Threshold, the network can figure out the adjustment amount of t1-Threshold if the failure is caused by improper T1 setting. Considering the overhead, we suggest to introduce the relative time in the RLF report.

Proposal 2: Regarding MRO for CHO with a time-based trigger condition, include relative time between CHO execution failure and t1-Threshold in the RLF report.

According to the LS sent by RAN3, RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not [1]. The existing RLF report contains IE and parameters related to the fulfilled CHO trigger conditions. However, the existing IE and parameters are included for the fulfilled CHO trigger conditions of neighbour cells which were configured as candidate cells. See the following text is copied from chapter 5.3.10.5 in TS 38.331. While UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition” for the source cell or target cell, in order to help the network to deduce which CHO trigger condition of the source cell or target cell is set improperly. We think new IE or parameters is needed in RLF report.

2> for each neighbour cell, if any, included in measResultListNR in measResultNeighCells: 3> if the UE supports RLF-Report for conditional handover and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure: 4> set choConfig in MeasResult2NR to the execution condition for each measId within condTriggerConfig associated to the neighbour cell within the MCG VarConditionalReconfig; 4> if the first entry of choConfig corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure; or 4> if the second entry of choConfig, if available, corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure: 5> set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of choConfig or to the execution condition condSecondEvent corresponding to the second entry of choConfig, whichever execution condition was fulfilled first in time; 5> set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in choConfig that was fulfilled first in time, and the point in time of fulfilling the condition in choConfig that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the choConfig were fulfilled;

RLF-Report-r16 ::=                   CHOICE {

nr-RLF-Report-r16                    SEQUENCE {

measResultLastServCell-r16           MeasResultRLFNR-r16,

measResultNeighCells-r16             SEQUENCE {

measResultListNR-r16                 MeasResultList2NR-r16       OPTIONAL,

measResultListEUTRA-r16              MeasResultList2EUTRA-r16    OPTIONAL

}                                                OPTIONAL,

c-RNTI-r16                           RNTI-Value,

previousPCellId-r16                  CHOICE {

nrPreviousCell-r16                   CGI-Info-Logging-r16,

eutraPreviousCell-r16                CGI-InfoEUTRALogging

}                                                                    OPTIONAL,

MeasResultList2NR-r16 ::=            SEQUENCE(SIZE (1..maxFreq)) OF MeasResult2NR-r16

MeasResult2NR-r16 ::=                SEQUENCE {

ssbFrequency-r16                     ARFCN-ValueNR                                           OPTIONAL,

refFreqCSI-RS-r16                    ARFCN-ValueNR                                           OPTIONAL,

measResultList-r16                   MeasResultListNR

}

MeasResultListNR ::=                    SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR

MeasResultNR ::=                        SEQUENCE {

physCellId                              PhysCellId                                                            OPTIONAL,

measResult                              SEQUENCE {

cellResults                             SEQUENCE{

resultsSSB-Cell                         MeasQuantityResults                                           OPTIONAL,

resultsCSI-RS-Cell                      MeasQuantityResults                                           OPTIONAL

},

rsIndexResults                          SEQUENCE{

resultsSSB-Indexes                      ResultsPerSSB-IndexList                                       OPTIONAL,

resultsCSI-RS-Indexes                   ResultsPerCSI-RS-IndexList                                    OPTIONAL

}                                                                                                         OPTIONAL

},

...,

[[

cgi-Info                                CGI-InfoNR                                                            OPTIONAL

]],

[[

choCandidate-r17                        ENUMERATED {true}                                                     OPTIONAL,

choConfig-r17                           SEQUENCE (SIZE (1..2)) OF CondTriggerConfig-r16                       OPTIONAL,

triggeredEvent-r17                      SEQUENCE {

timeBetweenEvents-r17                   TimeBetweenEvent-r17                                              OPTIONAL,

firstTriggeredEvent-r17                 ENUMERATED {condFirstEvent, condSecondEvent}                      OPTIONAL

}                                                                                                         OPTIONAL

]],

[[

entering-r18                            ENUMERATED {true}                                                     OPTIONAL

]]

}

Proposal 3: design new IE/parameters to report the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition” in RLF report.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: Regarding MRO for CHO with a time-based trigger condition, include relative time between the time RLF occurs and t1-Threshold in the RLF report.

Proposal 2: Regarding MRO for CHO with a time-based trigger condition, include relative time between CHO execution failure and t1-Threshold in the RLF report.

Proposal 3: design new IE/parameters to report the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition” in RLF report.

4	References

- [1] R3-247908, LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT, RAN3.