3GPP TSG-RAN WG2 Meeting #129                                         R2-2500606

Athens, Greece, Feb 17th – 21th, 2025

Source:        ZTE Corporation

Title:           Discussion on random access procedure in SBFD

Agenda item:   8.11.2

Document for:  Discussion and Decision

- Introduction

The WID [1] of Rel-19 SBFD related to random access is given as below:

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

This contribution provides our view on random access procedures in SBFD technique.

- Discussion

SBFD RACH configuration in NUL carrier

Legacy RACH configuration can be provided within UL BWP configuration, where the UL BWP can be allocated on NUL carrier or SUL carrier. For legacy RACH configuration in NUL, ROs in DL slot/symbol is considered as invalid; however, SUL carrier can always work even in D slot/symbol, i.e., UE can perform RA in SUL regardless of cell-level TDD pattern.

For SBFD, UL subband should only be configured within DL carrier, and SBFD symbol should only be legacy D or F slot/symbol. If UE performs random access in SBFD symbols, the ROs should occupy or overlap with both SBFD symbol (time domain) and UL subband (frequency domain). As the fact that SUL usually has non-overlapping lower frequency than NUL DL carrier to provide larger UL coverage, in order to occupy or overlap with resources in the UL subband, ROs should be configured within NUL carrier, not SUL carrier.

Figure 1. Example relationship between UL subband, DL carrier, NUL carrier in a TDD system

Proposal 1: RAN2 confirms that SBFD RACH configuration option 1 and option 2 should only be configured in NUL carrier, no matter the SBFD RACH configuration is in SIB or in dedicated signaling.

CBRA in SBFD ROs

RAN2#127 has made the following agreements to discuss the RACH configuration in RRC signaling:

RAN2#127 Agreement RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS.

Wherein the SBFD RACH configuration in the SIB is for RRC_INACTIVE/IDLE UE to perform CBRA (or CFRA for SI request), and in dedicated RRC signaling is for RRC_CONNECTED UE to perform CFRA (or CBRA when fallback).

CBRA RACH resource is provided in SIB1 -> BWP-UplinkCommon for RRC_IDLE/INACTIVE UEs, or in RRCReconfiguration -> BWP-UplinkCommon for RRC_CONNECTED UEs. This section provides our thoughts on the RRC RACH configuration for SBFD-aware UEs in CBRA.

RACH configuration for option 2

In RAN1#118 meeting, the further agreement of RACH configuration option 2 is achieved as follows:

Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

This means, for option 2, only SBFD RO will be the valid RO type.

Observation 1: RACH configuration Option 2 can only provide valid SBFD ROs.

Separate RACH configuration provides independent basic RACH parameters compared with the legacy RACH configuration. So the valid RO of derived from separate RACH configuration and legacy RACH configuration can be totally in different pattern to increase the UL coverage, reduce the latency, etc.

According to the agreement in RAN2#127-bis, a new RACH configuration dedicated for SBFD-aware UE for option 2 will be needed:

Agreement in RAN2#127-bis For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. FFS dedicated RRC signalling detail.

We suggest to add the SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon, like the following figure:

Figure 2. Recommended CBRA RACH configuration option 2

This way is to configure option 2 using similar principle of introducing 2-step RACH configuration, i.e., the new SBFD RACH configuration is also contained in each element in AdditionalRACH-ConfigList.

Proposal 2: For SBFD RACH configuration option 2 in CBRA, add a SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon, and add the SBFD RACH configuration in each AdditionalRACH-Config.

RO type selection

RAN2#127-bis has agreed to select RO type based on some pre-defined rules:

Agreement Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

RAN2#128 has agreed the following to discuss the detailed selection criteria:

Agreement FFS on the following options: - Option 1: Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism: - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. - Option 2: UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

Option 1 is NW indication + other metrics including SSB RSRP, while option 2 is SSB RSRP + prioritize one type. Our view is that the SSB RSRP should not be a criteria for selecting SBFD RO or legacy RO, since if there is one SSB RSRP threshold configured for selecting RO type, for cell edge UE, the SSB RSRP should be lower than the configured threshold to select SBFD RO to increase coverage, but for cell center UE, the SSB RSRP should be larger than the configured threshold to select SBFD RO to resist interference. Furthermore, RAN2 has already agreed to only enable one type of SBFD RACH configuration (either option 1 or option 2), which means SBFD RACH for increasing coverage or resisting interference cannot work together at same time in a cell. So at one time, only cell center UE or cell edge UE can enjoy the benefit of using SBFD RO, rather than all SBFD-aware UEs in the cell.

Therefore, SSB RSRP should not be a criteria for selecting SBFD RO or legacy RO, and option 1 should be further considered.

Proposal 3: RAN2 to support option 1 for RO type selection, and the SSB RSRP should not be a criteria for selecting RO type.

Currently UE selects nearest RO after SSB/CSI-RS is selected, as specified in TS38.321 below:

5.1.2 Random Access Resource selection If the selected RA_TYPE is set to 4-stepRA, the MAC entity shall: <omitted> 1> else if an SSB is selected above: 2> if the set of Random Access resources associated with Msg1 repetition is selected for this Random Access procedure: 3> determine the next available set of PRACH occasions (as specified in TS 38.213 [6]) for the Msg1 repetition number applicable for this Random Access procedure corresponding to the selected SSB (the MAC entity shall select a set of PRACH occasions randomly with equal probability amongst sets of PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB and selected Msg1 repetition number for this Random Access procedure; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available set of PRACH occasions corresponding to the selected SSB). 2> else: 3> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured, or indicated by PDCCH, or indicated by the LTM Cell Switch Command MAC CE (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected SSB).

When a gNB configures SBFD resource and lets UE initiating RA on the SBFD resource, it aims to provide more ROs (more RA chances) to reduce the RA latency, so it is likely that the system has more SBFD ROs than non-SBFD ROs configured within one RACH configuration. If a SBFD-aware UE selects a nearest non-SBFD RO according to current spec for the first several Msg1/MsgA attempts, the whole RA time will very likely be longer than choosing SBFD ROs in the beginning, since the subsequent SBFD ROs will be pretty intense although SBFD RO is not nearest RO type. In addition, this is not good for load balancing, since the non-SBFD ROs may suffer from multiple UE’s access including SBFD-aware UEs. Therefore, always selecting the nearest RO may not be a good way when SBFD RO is introduced.

Observation 2: Always selecting the nearest RO may not be a good way when SBFD RO is introduced.

To solve the above problem and to balance the whole RA latency and total RA chances, UE can use a time window to determine whether to select SBFD RO or legacy RO. If during the time window UE finds the SBFD RO exists, no matter whether legacy RO is nearest or not, UE should choose the SBFD RO to insure more RA chances without losing too much latency; if during the time window UE finds there is no SBFD RO, UE should select the nearest RO based on legacy rule to insure latency, since in this case, even if the SBFD RO may be intense after the time window, UE choosing SBFD RO will lose RA latency.

Figure 3. Example of the time window and RO selection

Proposal 4: Introduce a time window to let UE determine whether to select SBFD RO or legacy RO, in order to balance the RA latency and RA opportunity:

- If UE finds the SBFD RO exists in the time window, no matter whether legacy RO is the nearest or not, UE should choose the nearest SBFD RO in the time window;

- If UE finds there is no SBFD RO in the time window, UE should select the nearest RO based on legacy rule.

MAC order of RO type selection

Based on the analysis above, for CBRA, UE can select RO type based on some criteria. It is RAN2’s responsibility to study the impact of such selection on the current MAC RA procedure order/flow.

The current RA procedure in MAC spec is as follows:

Figure 4. Current MAC order of initiating RA procedure

The RACH configuration selection/RO type selection can happen before resource set selection, or after resource set selection. The main difference is that, the content of RACH resource set will be different in these two ways. When before, the selected set of RACH resource is per RO type; when after, the selected set of RACH resource is per UE.

Current fallback mechanism is restricted within one RACH resource set, e.g., 2-step RACH falls back to 4-step RACH, even though 2-step RACH and 4-step RACH are two different RACH configurations, they belong to one set of RACH resources.

Since RAN2 already agreed on SBFD RO fallback to legacy RO, we think the similar principle should be applied here, i.e., fallback mechanism (from SBFD RO to non-SBFD RO) is restricted within one RACH resource set, in order to minimum the impact on the legacy procedure. Based on this principle:

- If RRC only configures option 1, RO type selection happens within one RACH configuration, so RO type selection should be after set selection to ensure one set containing two types of ROs;

- If RRC only configures option 2, RO type selection happens between legacy RACH configuration and additional SBFD RACH configuration, so RO type selection (i.e., RACH configuration selection) should be after set selection to ensure one set containing two types of ROs.

Figure 5. The example MAC order of RO type selection

Based on above, we propose:

Proposal 5: For both RRC configured option 1 and option 2, RO type selection should be after the set selection, to ensure that RACH resources of different RO type can be selected within one RACH resource set.

Fallback from legacy RO to SBFD RO

Last meeting there is a discussion on whether to support legacy RO fallback to SBFD RO. We do not observe the benefit for such scenario, since using SBFD RO usually faces larger interference.

Another issue is that, for a SBFD-aware UE supporting 2-step RACH configuration, it the UE initially selects 2-step RACH resource, after msgA-TransMax-r16 times of failure, UE will fall back to try 4-step RACH resource for the rest of times. If the number of preamble transmission failure before fallback from legacy RO to SBFD RO is also configurable, then it is unclear that whether the UE should fall back to 4-step RACH resource, or fall back to 4-step SBFD RACH resource. Additional spec impact will be needed to solve this issue.

Another spec impact would be that, if fallback from 2-step RACH resource -> 4-step RACH resource -> 4-step SBFD RACH resource is supported, the maximum preamble transmission number should also be increased, to ensure the same RA successful rate.

Figure 6. Fallback from legacy RO to SBFD RO when 2-step RACH is also configured

In summary, allowing fallback from legacy RO to SBFD RO will have larger spec impact. Therefore we propose:

Proposal 6: Do not support the fallback from legacy RO to SBFD RO.

Early identification of SBFD-aware UE

RAN2#127-bis meeting has agreed the following:

Agreement RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

That is to say Msg1 based early identification is not supported in legacy ROs. The question is whether to support Msg3 based early identification when legacy RO is selected.

Figure 7. Example of initial access timeline

Firstly, as the figure indicates, if early identification is achieved in Msg3, the benefit is only on Msg5 UL, NAS security UL and AS security UL (see green arrows), these UL transmission can be scheduled on UL subband. When gNB acquires UE capability, the support of SBFD feature will be directly known by gNB. However the NAS security and AS security set up procedure is only applicable in RRC_IDLE, i.e., even if UE resumes from RRC_INACTIVE, these two steps are not needed. So the scenario of scheduling UL transmission in UL subband becomes more limited.

Secondly, Msg3 based early identification will achieve less benefit than Msg1 based early identification. If companies thinks early identification is not an urgent feature to support for Msg1 based early identification, then Msg3 based early identification shouldn’t be supported at all due to the lower benefit.

Based on the above, we have the following proposal:

Proposal 7: Do NOT support Msg3 based early identification for SBFD-aware UE.

CFRA in SBFD ROs

RO type indication in CFRA

RAN2#128 has achieved the following agreement:

Agreement The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

The signaling is FFS. gNB configures different CFRA RACH resource for different CFRA scenarios, using RRC/MAC CE/DCI signaling. To be specific:

- For UE performing CFRA for BFR, the CFRA RACH configuration is provided in BWP-UplinkDedicated - BeamFailureRecoveryConfig;

- For UE performing CFRA for ReconfigurationWithSync, the CFRA RACH configuration is provided in ReconfigurationWithSync - rach-ConfigDedicated;

- For UE performing CFRA for LTM early sync, the CFRA RACH configuration is provided in RRCReconfiguration - LTM-Config -  LTM-Candidate - ltm-EarlyUL-SyncConfig - EarlyUL-SyncConfig;

- For UE performing CFRA for PDCCH order, the corresponding DCI can also be used to indicate some CFRA resource;

- For UE performing CFRA for LTM switch, a LTM cell switch commend MAC CE is introduced to indicate CFRA resource in LTM:

TS38.321-i30: - 6.1.3.75 LTM Cell Switch Command MAC CE Figure 6.1.3.75-1: LTM Cell Switch Command MAC CE

Therefore, the RO type indication can be given via RRC signaling, DL MAC-CE or DCI:

- In RRC signaling, each CFRA RACH configuration is associated with a RACH-ConfigGeneric, the RACH-ConfigGeneric can naturally contain SBFD RO and non-SBFD RO, since the RO pattern is configured periodical and has nothing to do with TDD pattern. So one simple way is to add an indication in each RRC-configured CFRA RACH configuration to say that, whether UE should only consider SBFD ROs provided by the corresponding RACH-ConfigGeneric to be valid, or UE should only consider non-SBFD ROs provided by the corresponding RACH-ConfigGeneric to be valid.

  - For the BFR CFRA resource, the RO type indication can be configured in BWP-UplinkDedicated – BeamFailureRecoveryConfig;

  - For the ReconfigurationwithSync CFRA resource, the RO type indication can be configured in ReconfigurationWithSync - rach-ConfigDedicated;

  - For the LTM early sync CFRA resource, the RO type indication can be configured in RRCReconfiguration - LTM-Config - LTM-Candidate - ltm-EarlyUL-SyncConfig - EarlyUL-SyncConfig.

- For CFRA resource provided via LTM cell switch MAC CE, the RO type indication be added in the MAC CE that, whether the RA triggered by the MAC CE can only be performed on SBFD RO;

- For PDCCH order, RAN1#118-bis meeting has the following agreement to explicit indicate the RO type in PDCCH order:

Agreement For SBFD-aware UEs and PRACH mask indication for CFRA triggered by PDCCH order, discuss whether/how to indicate/determine additional-RO only, legacy-RO only, or both is used.

RAN2 should focus on RRC and MAC spec impact, and leave DCI/PDCCH order to RAN1 decision. So we have the following proposal:

Proposal 8: For RO type indication in CFRA, the following RRC and MAC spec impact should be considered:

- Add an RO type indication in BeamFailureRecoveryConfig to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in rach-ConfigDedicated to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in EarlyUL-SyncConfig to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in LTM cell switch command MAC CE to indicate whether the RA triggered by the MAC CE can only be performed on SBFD RO.

RO type indication in SI request

Additionally for SI request, the legacy RACH configuration is provided in SIB1 - SI-SchedulingInfo - SI-RequestConfig/SI-RequestConfigRepetition. Some companies raised concern on the SI request in SBFD ROs due to the spec impact may be huge. However, SI request can be seen as a non-typical CFRA procedure, so gNB can directly indicate whether the RA for SI request should be performed on SBFD RO or legacy RO, e.g., 1 bit in SI-RequestConfig and SI-RequestConfigRepetition-r18 to indicate whether the SI request can be performed in SBFD RO or not, respectively. These two IEs cover all the SI request scenarios, i.e., normal UE SI/POS SI request, RedCap UE SI/POS SI request, normal UE/RedCap UE SI/POS SI request with Msg1 repetition.

Observation 3: The spec impact of supporting SI request in SBFD RO can be small, e.g., 1 bit in SI-RequestConfig and SI-RequestConfigRepetition-r18 to indicate whether the SI request can be performed in SBFD RO or not, respectively.

Therefore we propose that:

Proposal 9: For SI request, add an RO type indication in SI-RequestConfig and SI-RequestConfigRepetition to indicate whether the corresponding RACH-ConfigGeneric contains valid useable SBFD RO.

Fallback from CFRA to CBRA

In legacy RA mechanism, if CFRA resource is provided for the current RA event, UE should firstly try to use CFRA resource to initiate RA by comparing the measured RS-RSRP and configured power threshold. If the measured RS-RSRP is larger than the configured power threshold, UE should use the indicated CFRA resource; if not, UE should fall back to use the CBRA resource within the RACH resource set selected in advance. Each time when the current preamble transmission fails, UE should compare the measured RS-RSRP and configured power threshold to determine whether CFRA or CBRA should be used. The basic procedure flow is as following:

Figure 8. CFRA falls back to CBRA

Since there is no SBFD RO in legacy mechanism, UE keeps the same RO type when CFRA fallbacks to CBRA, i.e., in fallback CBRA, UE should select the same RO type as the RO type indicated by gNB for CFRA resource. We think the similar procedure should be applied here, i.e., if gNB indicates to use SBFD RO for the CFRA procedure, when CFRA falls back to CBRA, UE should also select SBFD RO for the CBRA procedure. That is to say, UE should NOT use the criteria to select between SBFD RO and legacy RO in CBRA, when the CBRA is a fallback from CFRA. The RO type selection criteria is only for pure CBRA case.

Proposal 10: When CFRA falls back to CBRA, if CFRA is indicated to perform on SBFD ROs, the fallback CBRA should also be performed on SBFD ROs.

Msg1 repetition in SBFD ROs

RAN1#118 has made the following agreement regarding Msg1 repetition in SBFD ROs:

Agreement For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-Ros (not across additional-Ros and legacy Ros) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-Ros and legacy Ros) as in legacy releases.

It can be seen that Msg1 repetition in SBFD ROs is supported in RAN1, so RAN2 should discuss solutions and specification impacts on this aspect.

Msg1 repetition availability/repetition number determination on SBFD ROs

The rsrp-ThresholdMsg1-RepetitionNum2/4/8 is configured only for CBRA to determine whether Msg1 repetition should be applied, and which Msg1 repetition number should be applied.

As specified in TS38.321 5.1.1b, the basic procedure of Msg1 repetition on legacy ROs are:

Step 1: UE determines the Msg1 repetition feature availability and available Msg1 repetition number(s) on legacy ROs, based on one or more configured RSRP threshold rsrp-ThresholdMsg1-RepetitionNumX-r18:

- If current measured DL pathloss RSRP < any of configured rsrp-ThresholdMsg1-RepetitionNumX-r18, Msg1 repetition is applicable and X is an applicable Msg1 repetition number for current RA;

- If current measured DL pathloss RSRP ≥all of configured rsrp-ThresholdMsg1-RepetitionNumX-r18, Msg1 repetition is not applicable for current RA.

Step 2: UE selects a RACH resource set, according to the determined Msg1 repetition feature availability and determined available Msg1 repetition number(s):

- If there is only one configured set that is associated with all features that UE wants to apply, UE selects this set;

- If there are multiple sets, with each associated with all features but with different available Msg1 repetition numbers, UE selects the set that is associated with the highest repetition number;

- If there is no set that is associate with all features, but there are configured sets associated with subset of all features, UE selects a set according to priority rule in clause 5.1.1d.

Step 3: UE selects 4-step RA and UE performs Msg1 repetition on the selected set (with the determined Msg1 repetition number) and on legacy ROs.

RAN1#119 has further made following agreement regarding Msg1 repetition in SBFD ROs:

Agreement For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.  Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

When a new rsrp-ThresholdMsg1-RepetitionNumX is introduced for SBFD ROs, the legacy step 1 will need to perform on legacy ROs and SBFD ROs separately/independently, i.e., UE determines the Msg1 repetition feature availability and Msg1 repetition number(s) on legacy ROs using existing rsrp-ThresholdMsg1-RepetitionNumX-r18, in the meanwhile, UE also determines the Msg1 repetition feature availability and Msg1 repetition number(s) on SBFD ROs using newly introduced rsrp-ThresholdMsg1-RepetitionNumX.

Proposal 11: In CBRA, for Msg1 repetition in SBFD ROs, the MAC order should be:

- Step 1: UE determines the Msg1 repetition feature availability and Msg1 repetition number(s) on legacy ROs and SBFD ROs separately (based on the separately configured RSRP threshold);

- Step 2: UE selects a set of RACH resource corresponds to the determined Msg1 repetition feature availability and Msg1 repetition number(s), wherein the set of RACH resource contains two types of RO;

- Step 3: UE selects a RO type;

- Step 4: UE performs Msg1 repetition using the determined Msg1 repetition number on the selected RO type.

Fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD RO

The other issue is whether/how to support Msg1 repetition number fallback in SBFD ROs in CBRA. In legacy mechanism, UE can fallback from lower repetition number to higher repetition number if several RA attempts fail, in order to increase the RA successful rate of the poor coverage UE. For the similar purpose, the UE should also be able to fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD symbols.

The spec also restricts that UE should select a fallback set that has the same associated feature combination:

TS38.321-i40, 5.1.4: 2> if the Random Access procedure is not completed: 3> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure: 4> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 4> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure;

To reduce the specification effort, the fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD symbols should also be associated the same feature combination, i.e., UE does not re-select RACH resource set when such fallback happens.

Proposal 12: Support fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD symbols, and such fallback should be associated with same feature combination.

- Conclusion

In this contribution, we propose the following observation and proposals:

SBFD RACH configuration in NUL carrier

Proposal 1: RAN2 confirms that SBFD RACH configuration option 1 and option 2 should only be configured in NUL carrier, no matter the SBFD RACH configuration is in SIB or in dedicated signaling.

CBRA in SBFD ROs

Observation 1: RACH configuration Option 2 can only provide valid SBFD ROs.

Observation 2: Always selecting the nearest RO may not be a good way when SBFD RO is introduced.

Proposal 2: For SBFD RACH configuration option 2 in CBRA, add a SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon, and add the SBFD RACH configuration in each AdditionalRACH-Config.

Proposal 3: RAN2 to support option 1 for RO type selection, and the SSB RSRP should not be a criteria for selecting RO type.

Proposal 4: Introduce a time window to let UE determine whether to select SBFD RO or legacy RO, in order to balance the RA latency and RA opportunity:

- If UE finds the SBFD RO exists in the time window, no matter whether legacy RO is the nearest or not, UE should choose the nearest SBFD RO in the time window;

- If UE finds there is no SBFD RO in the time window, UE should select the nearest RO based on legacy rule.

Proposal 5: For both RRC configured option 1 and option 2, RO type selection should be after the set selection, to ensure that RACH resources of different RO type can be selected within one RACH resource set.

Proposal 6: Do not support the fallback from legacy RO to SBFD RO.

Proposal 7: Do NOT support Msg3 based early identification for SBFD-aware UE.

CFRA in SBFD ROs

Observation 3: The spec impact of supporting SI request in SBFD RO can be small, e.g., 1 bit in SI-RequestConfig and SI-RequestConfigRepetition-r18 to indicate whether the SI request can be performed in SBFD RO or not, respectively.

Proposal 8: For RO type indication in CFRA, the following RRC and MAC spec impact should be considered:

- Add an RO type indication in BeamFailureRecoveryConfig to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in rach-ConfigDedicated to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in EarlyUL-SyncConfig to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in LTM cell switch command MAC CE to indicate whether the RA triggered by the MAC CE can only be performed on SBFD RO.

Proposal 9: For SI request, add an RO type indication in SI-RequestConfig and SI-RequestConfigRepetition to indicate whether the corresponding RACH-ConfigGeneric contains valid useable SBFD RO.

Proposal 10: When CFRA falls back to CBRA, if CFRA is indicated to perform on SBFD ROs, the fallback CBRA should also be performed on SBFD ROs.

Msg1 repetition in SBFD ROs

Proposal 11: In CBRA, for Msg1 repetition in SBFD ROs, the MAC order should be:

- Step 1: UE determines the Msg1 repetition feature availability and Msg1 repetition number(s) on legacy ROs and SBFD ROs separately (based on the separately configured RSRP threshold);

- Step 2: UE selects a set of RACH resource corresponds to the determined Msg1 repetition feature availability and Msg1 repetition number(s), wherein the set of RACH resource contains two types of RO;

- Step 3: UE selects a RO type;

- Step 4: UE performs Msg1 repetition using the determined Msg1 repetition number on the selected RO type.

Proposal 12: Support fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD symbols, and such fallback should be associated with same feature combination.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, 2024-06