3GPP TSG-RAN WG2 #129	R2-2500612

Athens, GR, Feb 17th – 21st, 2025

Agenda item:	   8.1.2.3

Source:		Lenovo

Title:	LCM for AIML based positioning with UE-sided model

Document for:		Discussion and Decision

# Introduction

In this paper we discuss the 1st priority use case for LCM for AIML based positioning with UE-sided model, i.e., case 1.

- (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

# Discussion

## 2.1	Training data collection

RAN1 Agreement For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB Conclusion For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

In the previous RAN1 discussion, RAN1 has considered using the following two options to provide necessary DL PRS configuration. And in our view, both can be supported based on legacy LPP Assistance Data Transfer procedure (RequestAssistanceData/ProvideAssistanceData) or LPP Assistance Data Delivery Procedure (ProvideAssistanceData)

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

For Option A, UE can send a RequestAssistanceData message to LMF requesting on-demand DL PRS configuration, including relevant information, e.g., time duration, PRS characteristics. Then LMF will provide the demanded DL PRS configuration to UE via ProvideAssistanceData message.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

For Option B, LMF could provide the DL PRS configuration to UE via ProvideAssistanceData message directly.

The legacy LPP Assistance Data Transfer procedure (via RequestAssistanceData/ ProvideAssistanceData messages) is taken as the baseline for UE to request DL PRS configuration from LMF for training data collection.

The legacy LPP Assistance Data Delivery procedure (via ProvideAssistanceData messages) is taken as the baseline for LMF to proactively provide DL PRS configuration to UE for training data collection.

## 2.2	Positioning accuracy monitoring

RAN1 Agreement Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

RAN1 also discussed the monitoring of AI-based positioning for case 1, and at least (Option A) target UE can perform the monitoring metric calculation and may transmit the outcome to the LMF. In our understanding, there are two possible scenarios:

- Option A initiated by UE: based on UE’s own demand, UE may perform monitoring by itself. UE may need LMF to provide related configuration or assistance information. UE will not send the monitoring outcome to LMF, instead UE may indicate to LMF if the AI-based positioning become non-applicable (e.g. via an Error cause)

  - This can be supported by LPP Request Assistance Data and Provide Assistance Data messages.

- Option A initiated by LMF: LMF configures UE to perform monitoring and send the monitoring outcome back to LMF.

  - This can be supported by LPP Request Location Information and Provide Location Information messages.

(Monitoring Option A initiated by UE) LPP RequestAssistanceData and ProvideAssistanceData messages are used for UE to request LMF to provide relevant configuration/data for monitoring metric calculation at UE without sending the outcome to LMF.

(Monitoring Option A initiated by LMF) LPP RequestLocationInformation and ProvideLocationInformation messages are used for LMF to configure the UE to calculate monitoring metric and send the outcome to LMF.

# 3	Conclusion

Based on the discussion above, we observe:

No table of figures entries found.

Based on the discussion above, we propose:

Proposal 1	The legacy LPP Assistance Data Transfer procedure (via RequestAssistanceData/ ProvideAssistanceData messages) is taken as the baseline for UE to request DL PRS configuration from LMF for training data collection.

Proposal 2	The legacy LPP Assistance Data Delivery procedure (via ProvideAssistanceData messages) is taken as the baseline for LMF to proactively provide DL PRS configuration to UE for training data collection.

Proposal 3	(Monitoring Option A initiated by UE) LPP RequestAssistanceData and ProvideAssistanceData messages are used for UE to request LMF to provide relevant configuration/data for monitoring metric calculation at UE without sending the outcome to LMF.

Proposal 4	(Monitoring Option A initiated by LMF) LPP RequestLocationInformation and ProvideLocationInformation messages are used for LMF to configure the UE to calculate monitoring metric and send the outcome to LMF.

# Reference