3GPP TSG-RAN WG2 #129	R2-2500613

Athens, GR, Feb 17th – 21st, 2025

Agenda item:	   8.1.3

Source:		Lenovo

Title:	Data collection for NW-sided model training

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss issues related to the training data collection for NW sided model for beam management and positioning use cases.

# Discussion

## 2.1	Data Collection for Beam Prediction

### 2.1.1	RAN3/SA5 related aspects

# RP-242399 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

Currently, RAN3 has started their normative phase discussion on AI for positioning topic. While in the current WID, RAN3/SA5 are not involved in the discussion related to training data collection, therefore we would rely on LS to coordinate with them to trigger any necessary discussion.

RAN3 started their normative phase discussion about AI based Positioning, however RAN3/SA5 are not formally involved in the discussion about NW side model training data collection for AI based beam management.

#### 2.1.1.1	MDT based framework for OAM-centric data collection

RAN2#126 Agreement Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

In the last RAN2 meeting, it was agreed that immediate MDT will be taken as the baseline framework for OAM centric data.

In the meanwhile, the legacy MDT framework, only supports the gNB being requested to collect L3 measurements. In order to request the gNB to collect L1 measurements, some enhancement on network interface signalling may be necessary, e.g., NG-AP interface signalling between AMF and gNB.

Legacy MDT framework only support gNB being requested to collect L3 measurement results. Enhancements to network interface signalling may be needed to support the request of L1 measurement results.

At the same time, there are two kinds of MDT framework, namely management-based MDT and signalling-based MDT. For management-based MDT, OAM configures the MDT configuration to gNB directly without pointing to a specific UE, and it is upon gNB to select the UE according to the configured criteria. Comparatively, for signalling-based MDT, the data collection is pointing to a specific UE via CN, and gNB configures the UE measurement report accordingly.

Comparing to signalling-based MDT, management-based MDT framework seems more suitable for training data collection among a group of UEs that fulfils certain criteria (e.g., cell or frequency or channel condition as discussed for the CSI/BM/Positioning use cases). In addition, management-based MDT is inherently anonymized meaning OAM does not need to understand the data is from which UE, as such it is user privacy friendly.

For management-based MDT, OAM configures the MDT configuration to gNB directly without pointing to a specific UE, and it is upon gNB to select the UE according to the configured criteria.

For signalling-based MDT, the data collection is pointing to a specific UE via CN, and gNB configures the UE measurement report accordingly.

Since management-based MDT and signaling-based MDT would have different implications and specification impacts to RAN3/SA5, it is suggested that RAN2 decides if management-based MDT or signaling-based MDT or both are assumed and inform RAN3/SA5 via LS.

In MDT based framework for OAM-centric data collection, RAN2 is suggested to check the possible impact on RAN3/ SA5 and inform RAN3/SA5 via LS to trigger their discussion.

#### 2.1.1.2	Logged data transition after HO/RLF/RRC state transfer

During the data collection phase, UE may log the L1 measurement results in its buffer and wait for gNB’s request/scheduling to report them. It is possible that before UE reports the logged data to gNB, UE may experience handover, or RLF, or entering RRC inactive/idle. In those cases, UE will not be able to transmit the logged data to the network anymore, and UE may either

- Option 1: Discard the remaining logged data that has not been reported yet

- Option 2: Keep the remaining logged data and wait for the network to fetch after connecting to the new cell or entering RRC connected again.

Upon handover, RLF, or entering RRC Inactive/Idle, RAN2 discusses the handling of the remaining logged data that has not been reported yet:

  - Option 1: Discard the remaining logged data

  - Option 2: Keep the remaining logged data and wait for the network to fetch after HO, RRC reestablishment or entering RRC connected again.

If Option 2 is supported, then the gNB, that fetches the remaining logged data from UE after HO/RLF/RRC state transition, needs to forward the received remaining logged data to either the old gNB or OAM that initiates the data collection at the first place.

If Option 2 is supported, RAN2 is suggested to inform RAN3 via LS to support the transfer of logged data over network interface.

### 2.1.2	Logging related aspects

Agreements 1 As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. 2 FFS on event based data collection/logging 3 Event-triggered data logging will be supported. At least radio condition based event triggered logging will be supported. FFS the details of radio condition based event. FFS if other events are supported.

Based on the agreement from last meeting, the UE can be configured to not initiate data collection and logging upon receiving the configuration. In this case, in our perspective the network can dynamically ask the UE to activate a particular configuration using L1/L2 indication. This enables the network to start/stop the data collection and logging flexibly depending on the respective functionality or specific scenario. For example,  the trigger for data collection and logging by the network may as well be dependent on some network-side conditions. Without any support for dynamic activation/deactivation, either the UE may be required to collect and log data for a long period than necessary and eventually lead of excess amount of logged data or the data collection and logging must be reconfigured/deconfigured often.

The initial state of data collection and logging configuration can be activated/deactivated. The configurations which are not activated upon configuration can be activated dynamically.

The active data collection and logging configurations can be deactivated by the network.

For AI/ML training, the data collection can vary based on factors such as network-side additional conditions or UE-side additional conditions. Measurements may need to be logged and reported differently; thus, a single data collection configuration may not fit for the different network-side/UE-side conditions. The network may want to activate or deactivate certain data collection configuration based on some dynamic conditions. In order to reduce the burden of de-configuring/re-configuring, network can provide multiple configurations to the UE, which can be dynamically activated/deactivated as and when required. This can be achieved byconfiguring the UE with one or more configurations which can be activated/switched upon an event being triggered such as the RSRP threshold.

UE can be configured with one or more data collection configurations whose initial state is configurable by the network.

### 2.1.3	Reporting related aspects

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- On-demand request from the network is supported.   FFS details on signalling

According to the last time RAN2 meeting, after logging the data, UE can report it to network based on the on-demand request, and FFS if event-based reporting and if data availability indication should be sent beforehand. In the following figure, we describe the overall data reporting procedure that would work efficiently in our view.

- Step 1: UE can be configured with event that would trigger the data report after UE starts logging the required measurement results. For example, some buffer size threshold (e.g., percentage of maximum buffer size) can be configured to trigger the report before the buffer is full.

- Step 2: UE will first send an availability indication to gNB, such that gNB can be prepared to reserve radio resources for scheduling the UL transmission of logged data. Comparing to let UE transmits the logged data straight away once triggered, this is considered a safer option to not interfere with other regular UL transmissions from the same or other UEs, gNB has the flexibility to fetch the logged data when the UL traffic in the cell is low. Since UE may have logged data for different data collection configurations, e.g. data logged for different UE/NW-side conditions, it is beneficial to provide gNB information on the data collection configurations for which UE has some logged data available for transmission, e.g. by signalling an ID identifying the data collection configuration. The availability indication should - similar to a buffer status report - indicate the amount of logged data (e.g. in bytes) for each data collection configuration (identified by an ID) in order to enable gNB to request some corresponding logged data and allocate UE resources accordingly.

  - It should be emphasized that the legacy BSR reporting procedure is not suitable for providing NW information on logged AI/ML data which is stored in a separate buffer, since BSR only reports data being avaiable for transmission at PDCP level and/or RLC level. Therefore, only RRC messages, e.g, UEInformationResponse message, containing logged data which are sumitted to PDCP layer would be accounted for in a BSR. Furthermore, the BSR would not distinguish between RRC messages being comprised of logged AI/ML data and other RRC messages pending for transmission in UE’s buffer. Therefore, we think some AI/ML specific availability indication is necessary.

- Step 3: Upon reception of the availability indication, gNB can request the UE to transmit some specific, e.g. for some specific data collection configuration(s), or all the logged data stored in its buffer via UEInformationRequest message

- Step 4: UE will generate and transmit UEInformationResponse message that contains the requested logged data stored in its buffer. If one UEInformationResponse is not enough to convey all logged data, UE should be able to generate and transmit multiple UEInformationResponse messages consecutively.

UE can be configured to trigger the data report upon a percentage of its maximum buffer size is reached.

Upon the trigger of the data report, UE will send an availability indication together with data size information for gNB to fetch the logged data with enough radio resource reserved.

UEInformationRequest and UEInformationResponse messages are used for gNB to fetch the logged data from UE.

If one UEInformationResponse is not enough to convey all logged data, UE should be able to generate and transmit multiple UEInformationResponse messages consecutively without waiting for another UEInformationRequest message.

### 2.1.4	UE consent

In immediate MDT framework, only UEs that have given MDT user consent can be configured to collect and report data for MDT purposes, which may not be enough to support the AIML driven data collection which can be initiated by OAM or gNB considering:

- MDT and AIML driven are considered as two kinds of data usage purposes. It may not be a good idea to directly reuse the MDT user consent (that UE may report its location) for the AIML driven data collection (UE will not report its location for AI based beam prediction use case).

- In case of gNB initiated training data collection, the MDT user consent may not be provided by the core network if OAM/CN is not prepared to perform MDT data collection.

Existing MDT user consent checking mechanism may not be enough to serve the purpose of data collection for beam prediction initiated by gNB or OAM.

In the meanwhile, in SA2 AIML related discussion, user consent for analytics (e.g., model inference) or ML model training is specified and stored in UDM, which is different than the MDT user consent. NWDAF will check the user consent before collecting or using the UE data for the AIML purpose.

We believe similar/same user consent for AIML purpose different than MDT user consent can be introduced and should be checked by the gNB before configuring the UE data collection either triggered by OAM or by gNB itself.

### TS 23.288 ### User consent for analytics Depending on local policy or regulations, to protect the privacy of user data, the data collection, ML model training and analytics generation for a SUPI or GPSI, Internal or External_Group_Id or "any UE" may be subject to user consent bound to a purpose, such as analytics or ML model training. The user consent is subscription information stored in the UDM, which includes: a) whether the user authorizes the collection and usage of its data for a particular purpose; b) the purpose for data collection, e.g. analytics or model training. The NWDAF retrieves the user consent to data collection and usage from UDM for a user, i.e. SUPI prior to collecting user data from an NF as described in clause 6.2.2 and from a DCCF as described in clause 6.2.6.

SA2 has defined user consent for analytics or model training, which will be checked by the NWDAF before collecting or using UE side data.

UE consent for AIML purpose (e.g., UE consent defined in SA2 for NWDAF triggered data collection) is needed before gNB configures the UE to perform data collection for gNB-side AIML model training.

## 2.2	Data collection for Positioning

### 2.2.1	Data collection for case 3a, 3b

(1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning.

RAN1#116bis Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. RAN3#125bis: Case 3a agreements: - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

RAN1 has agreed that for case 3a, the label and its related data can be generated by at least LMF. Since it’s an AIML model deployed at gNB-side, gNB can retrieve the training data from LMF as discussed in RAN3.

From RAN2 point of view, RAN2 can assume LPP procedure is used by LMF to collect the training data from UE/PRU. The possible LPP impact could be upon RAN1 discussion. For example, if label comprises LOS/NLOS indication, there may be no further LPP impacts, however if the label also includes PRU SRS Tx Location, then possible LPP impacts are expected to retrieve/update the PRU location. RAN2 is thus suggested to wait for RAN1 progress before discussing the possible LPP impact.

For (case 3a) NG-RAN node assisted positioning with gNB-side model, RAN2 supports LMF collecting training data from PRU or non-PRU UE via existing LPP procedures (e.g., RequestLocationInformation and ProvideLocationInformation). The detailed LPP impact is pending on RAN1 discussion.

(1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning. It is only for UL based direct positioning.

During the RAN1#116bis and RAN1#117 meetings, RAN1 assumes the training data (e.g., label and time stamp) for sub use case 3b can be generated by PRU or non-PRU UEs, and how the training data is transferred from PRU to NW is out of RAN1 scope.

From RAN2 point of view, since case 3b is related to LMF-sided AIML model, it seems natural for LMF to collect the required training data (e.g., label and time stamp) from PRU via LPP protocol.

RAN1#117 Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved.

For (case 3b) NG-RAN node assisted positioning with LMF-side model, RAN2 supports LMF collecting training data from PRU or non-PRU UE (e.g., with estimated SRS Tx transmission location) via existing LPP procedures (e.g., RequestLocationInformation and ProvideLocationInformation). The detailed LPP impact is pending on RAN1 discussion.

### 2.2.2	Data collection for case 2b

(2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning.

Although RAN2 has not yet discussed the case 2b, SA2 has considered case 2b in their study and asks RAN2’s feedback related to the procedure.

S2-2501133 To RAN 1 and RAN 2: For case 2b, SA2 is working on some procedures in corresponding R19 specifications, but has also captured the following Editor’s Notes on the interaction between the LMF and UE which depend on RAN progress: Editor's note: The input data used for AI/ML based positioning will be decided by RAN WGs. Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs. Editor's note: The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

RAN1 Working Assumption: Working Assumption For training data generation of AI/ML based positioning Case 2a and 2b, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE. Working Assumption For training data generation of AI/ML based positioning Case 2b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope.

For (case 2b) UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning, RAN1 has made assumptions in the RAN1#116bis and RAN1#117 meeting that the training data including channel measurements, label, and time stamp, which can be generated by PRU or non-PRU UE with estimated location. It seems also straight forward for the LMF to collect the required training data via existing LPP procedures, e.g., LPP RequestLocationInformation and ProvideLocationInformation messages.

For (case 2b) UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning, RAN2 supports LMF collecting training data (e.g., channel measurements, label and time stamp) from PRU and non-PRU UE via LPP procedure (e.g., RequestLocationInformation and ProvideLocationInformation). The detailed LPP impact is pending on RAN1 discussion.

### 2.2.3	Data collection via LPP protocol

In terms of the principles for reporting of the training data from PRU/non-PRU UE to LMF, we believe similar principle compared to training data collection in AIML based BM could apply.

For instance, the PRU/non-PRU UE could log multiple instances of channel measurements associated with a ground truth label and a time stamp, then UE could report multiple logged instances in one LPP message to LMF. The LPP message conveying the collected training data could be transmitted with lower priority than other LPP messages and can be triggered on-demand or event-based. W.r.t event-based logging, we believe area-based event can be considered. For example, a LMF may be interested to train an AIML model that is capable of determine UE location if UE is within an area of interest (e.g., in a campus, or in a factory). Therefore, UE can be configured to start/stop data collection if the UE moves into or outside the area of interest.

With respect to the related LPP procedure, in our view, the existing location information transfer procedure based on RequestLocationInformation and ProvideLocationInformation LPP messages can be enhanced to serve the purpose.

For AIML based positioning with LMF-side model, the legacy location information transfer procedure (i.e., RequestLocationInformation and ProvideLocationInformation LPP messages) is enhanced to support the data collection.

RAN2 supports the logging of measured training data (e.g., channel measurements, label, time stamp) at PRU/non-PRU UE, and the logged multiple instances of measured training data is reported to LMF via LPP message as configured by LMF.

The LPP message conveying the measured training data can be reported on-demand or event-based from PRU/non-PRU UE to LMF via lower priority SRB.

For AIML based positioning with LMF-side model, PRU/non-PRU UE may start/stop training data collection according to area-based event, e.g., the UE moves in or leaves an area.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	RAN3 started their normative phase discussion about AI based Positioning, however RAN3/SA5 are not formally involved in the discussion about NW side model training data collection for AI based beam management.

Observation 2	Legacy MDT framework only support gNB being requested to collect L3 measurement results. Enhancements to network interface signalling may be needed to support the request of L1 measurement results.

Observation 3	For management-based MDT, OAM configures the MDT configuration to gNB directly without pointing to a specific UE, and it is upon gNB to select the UE according to the configured criteria.

Observation 4	For signalling-based MDT, the data collection is pointing to a specific UE via CN, and gNB configures the UE measurement report accordingly.

Observation 5	Existing MDT user consent checking mechanism may not be enough to serve the purpose of data collection for beam prediction initiated by gNB or OAM.

Observation 6	SA2 has defined user consent for analytics or model training, which will be checked by the NWDAF before collecting or using UE side data.

Based on the discussion above, we propose:

Proposal 1	In MDT based framework for OAM-centric data collection, RAN2 is suggested to check the possible impact on RAN3/ SA5 and inform RAN3/SA5 via LS to trigger their discussion.

Proposal 2	Upon handover, RLF, or entering RRC Inactive/Idle, RAN2 discusses the handling of the remaining logged data that has not been reported yet:

a.	Option 1: Discard the remaining logged data

b.	Option 2: Keep the remaining logged data and wait for the network to fetch after HO, RRC reestablishment or entering RRC connected again.

Proposal 3	If Option 2 is supported, RAN2 is suggested to inform RAN3 via LS to support the transfer of logged data over network interface.

Proposal 4	The initial state of data collection and logging configuration can be activated/deactivated. The configurations which are not activated upon configuration can be activated dynamically.

Proposal 5	The active data collection and logging configurations can be deactivated by the network.

Proposal 6	UE can be configured with one or more data collection configurations whose initial state is configurable by the network.

Proposal 7	UE can be configured to trigger the data report upon a percentage of its maximum buffer size is reached.

Proposal 8	Upon the trigger of the data report, UE will send an availability indication together with data size information for gNB to fetch the logged data with enough radio resource reserved.

Proposal 9	UEInformationRequest and UEInformationResponse messages are used for gNB to fetch the logged data from UE.

Proposal 10	If one UEInformationResponse is not enough to convey all logged data, UE should be able to generate and transmit multiple UEInformationResponse messages consecutively without waiting for another UEInformationRequest message.

Proposal 11	UE consent for AIML purpose (e.g., UE consent defined in SA2 for NWDAF triggered data collection) is needed before gNB configures the UE to perform data collection for gNB-side AIML model training.

Proposal 12	For (case 3a) NG-RAN node assisted positioning with gNB-side model, RAN2 supports LMF collecting training data from PRU or non-PRU UE via existing LPP procedures (e.g., RequestLocationInformation and ProvideLocationInformation). The detailed LPP impact is pending on RAN1 discussion.

Proposal 13	For (case 3b) NG-RAN node assisted positioning with LMF-side model, RAN2 supports LMF collecting training data from PRU or non-PRU UE (e.g., with estimated SRS Tx transmission location) via existing LPP procedures (e.g., RequestLocationInformation and ProvideLocationInformation). The detailed LPP impact is pending on RAN1 discussion.

Proposal 14	For (case 2b) UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning, RAN2 supports LMF collecting training data (e.g., channel measurements, label and time stamp) from PRU and non-PRU UE via LPP procedure (e.g., RequestLocationInformation and ProvideLocationInformation). The detailed LPP impact is pending on RAN1 discussion.

Proposal 15	For AIML based positioning with LMF-side model, the legacy location information transfer procedure (i.e., RequestLocationInformation and ProvideLocationInformation LPP messages) is enhanced to support the data collection.

Proposal 16	RAN2 supports the logging of measured training data (e.g., channel measurements, label, time stamp) at PRU/non-PRU UE, and the logged multiple instances of measured training data is reported to LMF via LPP message as configured by LMF.

Proposal 17	The LPP message conveying the measured training data can be reported on-demand or event-based from PRU/non-PRU UE to LMF via lower priority SRB.

Proposal 18	For AIML based positioning with LMF-side model, PRU/non-PRU UE may start/stop training data collection according to area-based event, e.g., the UE moves in or leaves an area.

# Draft LS to RAN3/ SA5

# Title:	LS on NW side model training data collection for AI based beam management

# Response to:

# Release:	Rel-19

# Work Item:	NR_AIML_Air

Source:	RAN2

To:	RAN3, SA5

Cc:	RAN1, SA2

Contact Person:

#### Name:

#### E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

For the use case of AI based beam management, an AI model could be deployed at gNB to predict beam quality in either spatial domain (“BM-Case1”) or temporal domain (“BM-Case2”).

In order to collect required data for the training of such gNB-sided model, RAN2 agreed to support OAM-centric data collection based on immediate MDT framework. The training data may include L1-RSPR and beam-IDs which are considered new data type comparing to those already supported in the legacy MDT framework.

Therefore, enhancement to the legacy MDT framework is possibly needed and may impact RAN3 and SA5 specification.

2. Actions:

To WG RAN3, WG SA5

ACTION:

RAN2 kindly asks RAN3 and SA5 to take the above progress into account in their future work and provide further feedback if any.

3. Date of Next RAN2 Meetings:

RAN2#129		17th – 21st Feb 2025	Athens, GR

RAN2#129-bis		07th – 11th April 2025	TBD, CN