3GPP TSG-RAN WG2 Meeting #129	R2-2500616

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:		8.8.4

Source:		Lenovo

Title:		Further considerations on broadcast service area information in NTN

Document for:		Discussion

- Introduction

The latest RAN2#128 meeting made the following agreements regarding broadcast services in NTN:

Agreements: - The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. - The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. - The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed) - A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s). - RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. - Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way) - The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

In this contribution we discuss some further optimizations based on broadcast service area information (IntendedServiceArea) in NTN, including MCCH monitoring and service continuity.

- Discussion

For an NTN cell that could be much larger than an MBS broadcast service area, a UE may not be automatically authorized to receive an MBS broadcast service by the serving NTN cell it is in, i.e., not all UE within the same NTN cell is authorized to receive the same MBS broadcast service. For this reason, RAN2 have agreed to introduce broadcast service area information (IntendedServiceArea) in a new SIB for NTN, and further impacts and issues are expected to be resolved as well, including MCCH monitoring enhancements based on the area information and broadcast service continuity.

## MCCH monitoring based on broadcast service area information

For the initial MCCH monitoring, in legacy TN a UE starts to monitor upon entering the cell providing SIB20, and it is up to UE implementation to use the cell/tracking area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service. In an NTN cell which could be much larger than an MBS broadcast service area, such implementation relaying on the cell-level area information in USD may need to be further optimized based on the broadcast service area information introduced in the new SIB which are considered to be semi-static without frequent change. Although the previous RAN2#127 meeting agreed that UE may not need to (re)acquire up-to-date MCCH when the UE is not in any broadcast service area of its interested broadcast services, there are more details to be decided for MCCH monitoring skipping and resuming.

Proposal 1: UE may choose to skip MCCH monitoring based on the broadcast service area information provided in the new SIB (e.g., IntendedServiceArea) and its associated MBS session, e.g. when UE is not in any broadcast service area of its interested MBS sessions.

Meanwhile, MCCH uses a modification period and MCCH contents are only allowed to be modified at each modification period boundary. In NTN UE needs to ensure a valid UE location information and a valid ephemeris data (in SIB19) for uplink synchronization. UE re-acquiring SIB19 in NTN may possibly miss the modification period boundary. In such case, enhancements may also be considered to accelerate UE in awareness of MCCH change.

Proposal 2: UE may choose to monitor MCCH based on the broadcast service area information provided in the new SIB (e.g., IntendedServiceArea) and its associated MBS session when UE is in at least one broadcast service area of its interested MBS sessions, e.g. after re-acquiring SIB19 that overlaps with MCCH modification period boundary.

## Service continuity based on broadcast service area information

For IDLE/INACTIVE MBS broadcast service continuity in TN, mobility procedures for MBS reception allow the UE to start or continue receiving MBS service when changing cells. The gNB may indicate in SIB21 the mapping between the current and/or neighboring carrier frequencies and MBS Frequency Selection Area Identities (FSAI). The FSAI is derived from the serviceArea and radiofrequency elements received in the USD (User Service Description) via NAS signalling from network, wherein the serviceArea element declares one or more service areas designated by the SAI (Service Area Identity) which is mapped onto one or more cells.

If UE is receiving or interested to receive an MBS broadcast service and can only receive this MBS broadcast service by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session. The UE may also consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session.

This legacy mechanism works well under the assumption that an MBS broadcast service area is mapped to one or more TN cells. The UE can simply determine the frequency priority based on the receiving or interested MBS broadcast service and the frequency it associates to, and reselects to a cell that supports that service within its whole coverage. When considering that an NTN cell coverage could be larger than an MBS broadcast service area, which cannot be mapped to the cell’s whole coverage, UE cannot simply determine the frequency priority only based on the association between service and frequency, and service area information of neighbour cell may be needed.

Although RAN2#128 agreed that a signalled broadcast service area information may include geographic areas across the current serving cell and overlapping neighbor cell(s), a further question is how to let UE know this and how to use it for service continuity guaranteeing. A straight-forward way is to have a pointer to the corresponding broadcast service area in SIB21 so that UE can associate the area with a neighbor cell or its frequency. Noticing that it was still FFS in RAN2#128 whether and how to have a pointer for a broadcast service area in the new SIB, service continuity could be a good reason.

Proposal 3: A pointer (broadcast service area index or IDs) can be included in SIB21 to associate the neighbor cell frequency with the broadcast service area information for service continuity.

Based on the information above, UE can better guarantee service continuity by frequency priority handling.

Proposal 4: UE can prioritize a frequency for cell reselection if UE is in the broadcast service area associated to that frequency as in SIB21.

For CONNECTED MBS broadcast service continuity in TN, the UE in RRC_CONNECTED state may send MBS Interest Indication to the gNB, including the list of MBS frequencies and interested services, as well as frequency priorities. The gNB may use this indication when providing handover configuration to allow the UE to receive the MBS services the UE is interested in. Due to the same assumption that an MBS broadcast service area is mapped to one or more TN cells, UE does not need to provide its current or expected MBS broadcast service area information in the MBS Interest Indication. When considering that an NTN cell coverage could be larger than an MBS broadcast service area, UE’s current/expected MBS broadcast service area information is necessary at gNB for further handover decisions e.g., to judge whether UE is staying/moving into or moving out of an MBS broadcast service area and configure an appropriate target cell.

Proposal 5: UE may report its current broadcast service area information or corresponding pointer to gNB for service continuity in RRC_CONNECTED.

- Conclusion

In this contribution we discuss the further issues of broadcast service area indication and service continuity in NTN. It is proposed that:

Proposal 1: UE may choose to skip MCCH monitoring based on the broadcast service area information provided in the new SIB (e.g., IntendedServiceArea) and its associated MBS session, e.g. when UE is not in any broadcast service area of its interested MBS sessions.

Proposal 2: UE may choose to monitor MCCH based on the broadcast service area information provided in the new SIB (e.g., IntendedServiceArea) and its associated MBS session when UE is in at least one broadcast service area of its interested MBS sessions, e.g. after re-acquiring SIB19 that overlaps with MCCH modification period boundary.

Proposal 3: A pointer (broadcast service area index or IDs) can be included in SIB21 to associate the neighbor cell frequency with the broadcast service area information for service continuity.

Proposal 4: UE can prioritize a frequency for cell reselection if UE is in the broadcast service area associated to that frequency as in SIB21.

Proposal 5: UE may report its current broadcast service area information or corresponding pointer to gNB for service continuity in RRC_CONNECTED.