3GPP TSG-RAN WG2 Meeting #129	R2-2500619

Athens, Greece, Feb. 17th – 21st, 2025                       Revision of R2-2410271

Agenda item:		8.9.3

Source:		Lenovo

Title:		EDT for uplink capacity enhancement in NTN

Document for:		Discussion

- Introduction

The latest RAN2#128 meeting made the following agreement regarding uplink capacity enhancements in Rel-19 IoT NTN:

RAN2#128 agreements: - Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) - Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels - The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels) - There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) - The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific - There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) - For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time - For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs) - The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource. - RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

In this contribution we discuss the further issues and solutions for EDT enhancements in IoT NTN.

- Discussion

Considering the need of supporting massive capacity in terms of number and types of UE, there is a need to identify methods to reduce the necessary uplink and downlink signaling. From this perspective there are two directions that are to be studied: Msg3 transmission without Msg1/2 and reduced overhead reduction for Msg4 delivery.

## Msg3 transmission without Msg1/2 for EDT

In legacy EDT, the UE relies on transmission of EDT-dedicated preamble to request for EDT, and eNB can identify a request for EDT based on that an EDT-dedicated preamble is used. If Msg1(preamble) transmission is omitted for EDT, new schemes are needed for UE to request for EDT operation and for eNB to identify a request for EDT operation. A following issue is that, the UE relies on reception of Msg2(RAR) to acquire the UL resource for the followed Msg3 transmission in legacy EDT operation, as there is no resource pre-configuration for the UE. If Msg2(RAR) reception is omitted for EDT, how to configure UE with the corresponding resource for Msg3 transmission needs to be resolved.

For Rel-19 IoT NTN aiming to achieve Msg3 transmission without Msg1(preamble)/Msg2(RAR), RAN2#127bis and RAN2#128 meetings agreed that system information is used to provide cell-specific CB-Msg3 PUSCH resources per CE level, while some further details are still FFS.

As the existing CE level selection procedure was agreed to be used for CB-Msg3, the first issue is whether the same thresholds can be reused and how many levels can be supported. In our understanding as RAN2 have agreed to provide cell-specific CB-Msg3 PUSCH resources per CE level, it is also reasonable to have the same thresholds as well as level differentiation.

Proposal 1: The RSRP thresholds for CE level determination is reused to determine whether UE can use the cell-specific CB-Msg3 PUSCH resources associated to the corresponding CE level.

For the initiation of an EDT operation, legacy EDT scheme considers one metric of data size while the channel quality (e.g., signal strength) can be guaranteed by random access mechanisms (e.g., power ramping). This mechanism was agreed to be reused while there is a further issue about the maximum TBS for different CE levels. In our understanding there is no strong motivation to have a separate TBS size limitation for each CE level.

Proposal 2: The maximum TBS size to initiate CB-Msg3 is the same for all CE levels.

For SA, after the CB-Msg3 transmission the UE starts a PDCCH monitoring window for response reception. While for DSA, a following issue is whether to have one or multiple PDCCH monitoring window(s) per each replica. Noticing that RAN2#128 have agreed that UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity, we think it is better to keep it simple by having only one monitoring window.

Proposal 3: Only one PDCCH monitoring window is started for all CB-Msg3 replicas in DSA case.

There are still other remaining issues including the common or separate RNTI(s) for replicas, but it is obvious that CB-Msg4 format shall be decide to continue such discussions.

## Efficient delivery of Msg4 for EDT

In legacy EDT the delivery of Msg4 (e.g., RRCEarlyDataComplete for CP-EDT) is unicast via dedicated RRC signalling, and its included fields (e.g., dedicatedInfoNAS, extendedWaitTime, idleModeMobilityControlInfo, redirectedCarrierInfo) are specified as optional. From network perspective eNB can reduce overhead simply by omitting some unnecessary fields or restricting the optional fields in Msg4.

Moreover, these fields of Msg4 are not all UE-specific e.g., extendedWaitTime, idleModeMobilityControlInfo, and redirectedCarrierInfo could be common for multiple or even all UEs in the cell, and dedicatedInfoNAS could also be common when it is empty or zero (i.e., no DL data for EDT from core network). Therefore it is not necessary to always unicast all these fields with duplicated signalling overhead. Based on the above analysis, at least the following two directions can be considered for efficient Msg4 delivery:

- Case A: At least part of Msg4 is common for multiple UEs initiated EDT in the cell. In this case the common part of Msg4 may be delivered in a non-unicast manner.

- Case B: No DL data or other configuration is to be delivered to UE (i.e., empty Msg4). In this case an alternative indication other than Msg4 via RRC can be considered e.g., the Layer 1 or MAC CE acknowledgement used for PUR.

Therefore the following directions can be further discussed:

Proposal 4: RAN2 to discuss the following enhancements for Msg4 delivery in EDT:

- Msg4 delivery in non-unicast ways (e.g., multicast);

- Msg4 delivery using lower layer indication (e.g., Layer 1 indication or MAC CE as in PUR).

- Conclusion

In this contribution we discuss the potential issues and solutions for EDT enhancements in IoT NTN. The following proposals are given:

Proposal 1: The RSRP thresholds for CE level determination is reused to determine whether UE can use the cell-specific CB-Msg3 PUSCH resources associated to the corresponding CE level.

Proposal 2: The maximum TBS size to initiate CB-Msg3 is the same for all CE levels.

Proposal 3: Only one PDCCH monitoring window is started for all CB-Msg3 replicas in DSA case.

Proposal 4: RAN2 to discuss the following enhancements for Msg4 delivery in EDT:

- Msg4 delivery in non-unicast ways (e.g., multicast);

- Msg4 delivery using lower layer indication (e.g., Layer 1 indication or MAC CE as in PUR).