3GPP TSG-RAN WG2 Meeting#129	R2-2500622

Athens, Greece, Feb.17–21, 2025

Agenda item:	 8.10.2.1

Source:		Lenovo

Title:		Discussion on MRO enhancements for LTM

Document for:		Discussion and Decision

1	Introduction

RAN2#128 meeting agreed that:

- No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues.

- Reuse the existing ra-InformationCommon for the RA-based LTM failure.

- If RAN3 does not address this meaning that we need a RAN2 solution, add a list indicating which LTM candidates the UE had at RLF.

In this paper, we further discuss MRO enhancements for LTM.

2	Discussion

To reduce interruption time during PSCell change procedure, SCG LTM is introduced in R18 with same priority as MCG LTM. SCG LTM failure case and even near failure successful SCG LTM case should be avoided to improve network performance. In this paper, we focus on MCG LTM to mainly discuss RLF report, SHR and RA report, and MRO for SCG LTM can be further discussed when the discussion of MRO for MCG LTM is stable.

  - 2.1 MRO for connection failure in a LTM procedure

RAN2#127bis meeting agreed that unless RAN3 defines a NW-based solution, the UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). Since it is network that configures the UE to perform UE based TA measurement or PDCCH ordered early TA acquisition, network can identify how the UE got the TA value based on the stored UE context, it is not needed for the UE to log and report whether and how the UE got the TA value used for a failed LTM cell switch.

Proposal 1: It is not needed for the UE to log and report whether and how the UE got the TA value used for a failed LTM cell switch.

  - 2.2 MRO for near failure in a LTM procedure

RAN2#127 meeting agreed to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit. Considering the case that network does not have UE context related with LTM configurations (e.g. the list of LTM candidate cells) when receiving the SHR, and to align with RAN2#127bis meeting’s agreement for RLF report (i.e. Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell), an explicit indication concerning whether a neighbour cell is an LTM candidate cell or not may be included in the SHR.

Proposal 2: Include an explicit indication concerning whether a neighbour cell is an LTM candidate cell or not in the SHR, unless RAN3 defines a NW-based solution.

  - 2.3 RA report for a LTM procedure

In a LTM procedure, depending on the availability of a valid TA value, the UE performs either a RACH-less LTM or RACH-based LTM cell switch:

- the UE performs RACH-less LTM cell switch upon receiving the cell switch command:

  - if the TA value is provided in the cell switch command, the UE applies the TA value as instructed by the network. For this case, before cell switch command, early TA acquisition/UL synchronization procedure is triggered by PDCCH order to an LTM candidate cell, or the UE performs RACH for early UL synchronization or RACH for early TA acquisition;

  - if UE-based TA measurement is configured, but no TA value is provided in the cell switch command, the UE applies the TA value by itself if available.

- if no valid TA value is available, the UE performs RACH-based LTM cell switch upon receiving the cell switch command:

  - the UE performs CFRA to the target cell, if contention-free Random Access Resources are provided in LTM Cell Switch Command MAC CE;

  - or, the UE performs CFRA to the target cell, if no contention-free Random Access Resource is provided in LTM Cell Switch Command MAC CE, but contention-free Random Access Resources are provided in the RRC Reconfiguration message for the LTM procedure;

  - or, the UE performs CBRA to the target cell, if no contention-free Random Access Resource is provided.

To optimize RA resources for LTM procedure, the UE may report L1 measurement results (of source cell, or LTM candidate cell, or target cell) during LTM procedure in the RA report. Additionally, for optimization of early UL synchronization or early TA acquisition, a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early UL synchronization or early TA acquisition may be included in the RA report.

Proposal 3: The UE can report L1 measurement results during LTM procedure, and a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early TA acquisition, in the RA report.

3	Conclusion

In this paper, MRO enhancements for LTM are discussed. We have the following proposals:

Proposal 1: It is not needed for the UE to log and report whether and how the UE got the TA value used for a failed LTM cell switch.

Proposal 2: Include an explicit indication concerning whether a neighbour cell is an LTM candidate cell or not in the SHR, unless RAN3 defines a NW-based solution.

Proposal 3: The UE can report L1 measurement results during LTM procedure, and a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early TA acquisition, in the RA report.