3GPP TSG-RAN WG2 Meeting#129	R2-2500623

Athens, Greece, Feb.17–21, 2025

Agenda item:	 8.10.2.2

Source:		Lenovo

Title:		Discussion on MRO enhancements for CHO with candidate SCGs

Document for:		Discussion and Decision

1	Introduction

In this paper, we would further discuss MRO enhancements for CHO with candidate SCG(s).

2	Discussion

  - 2.1 MRO for connection failure case

Until RAN3#126 meeting, RAN3 achieved several agreements and sent an LS to RAN2 [1]

RAN3 concluded that the following information can be reported by the UE in RLF Report for failure reason detection and optimization:

- The type of the first fulfilled execution condition (e.g. CPAC or CHO). Fulfilment means all events of the type are met.

- Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

- The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

Additionally, RAN2#128 meeting agreed that:

- RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements.

- Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

RAN3 summarized the failure cases as below table in [2]:

| Cases | Figure: CHO with candidate SCG(s) is configured | Brief Description |
| --- | --- | --- |
| Case 1 |  | Failure happens before CHO and/or CPAC execution. 1a) RLF@srcPCell 1b) SCG failure 1c) MCG failure+SCG failure |
| Case 2 |  | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. 2a) RLF@srcPCell 2b) SCG failure 2c) MCG failure + SCG failure |
| Case 3 |  | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. 3a) RLF@srcPCell 3b) SCG failure 3c) MCG failure + SCG failure |
| Case 4 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. CHO execution to a candidate PCell fails. |
| Case 5 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. RLF shortly after CHO execution success |
| Case 6 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. SCG failure shortly after CHO execution success |
| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| Case 8 |  | Failure happens shortly after CHO and CPAC execution success 8a) RLF@targetPCell 8b) SCG failure 8c) MCG failure + SCG failure |
| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |

It was agreed in RAN3#125 meeting that MRO for case 4/5/6 is in the scope and it was FFS whether there is any specification impact for case 4/5/6. In case of 4/5/6, configuration of CHO only is configured to the UE together with configuration of CHO with candidate SCG(s) configuration, the UE performs CHO only execution due to CHO with candidate SCG(s) can’t be triggered (e.g. CHO execution condition of CHO with candidate SCG(s) is fulfilled but CPAC execution condition of CHO with candidate SCG(s) is not fulfilled, CPAC execution condition of CHO with candidate SCG(s) is fulfilled but CHO execution condition of CHO with candidate SCG(s) is not fulfilled, neither CHO execution condition nor CPAC execution condition of CHO with candidate SCG(s) is fulfilled). To improve mobility performance, it is beneficial for network to know the reason about why CHO with candidate SCG(s) together with CHO only is configured but CHO only is performed instead of CHO with candidate SCG(s), e.g. only CHO execution condition or only CPAC execution condition of CHO with candidate SCG(s) is fulfilled, or neither CHO execution condition nor CPAC execution condition of CHO with candidate SCG(s) is fulfilled. RAN3#125 meeting agreed the RLF report includes the type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met. This agreement applies to the case 4/5, for case 6, SCG failure information message needs to include the type of the first fulfilled execution condition of CHO with candidate SCG(s) if any e.g. CPAC or CHO.

Proposal 1: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, can be stored and reported by the UE in SCG failure information message.

RAN3#125bis meeting further agreed to include the identifier of candidate PCell(s) and/or PSCell(s) that fulfilled execution conditions when/before the RLF is encountered in RLF report. For case 6, it is needed to include identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the failure is encountered in the SCG failure information message.

Also, for case 2b, it is needed to include identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the failure is encountered in the SCG failure information message; for case 3b, it is needed to include identifier of candidate PCell(s) which met the configured CHO execution conditions before the failure is encountered in the SCG failure information message.

Proposal 2: Identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before connection failure is encountered can be stored and reported by the UE in SCG failure information message.

RAN2#127bis meeting agreed that the UE reports the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Further, RAN2#128 meeting agreed that the UE shall log (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). For case 7b and case 8b, SCG failure happens after CHO with candidate SCG(s) is triggered. To follow the agreement that “the time information (as agreed for RLF) are included in SCGFailureInformation”, it is beneficial to store and report the elapsed time between the point in time of the first fulfilled execution condition and SCG failure in the SCG failure information message.

Proposal 3: Time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), as well as the elapsed time between the point in time of the first fulfilled execution condition and SCG failure, can be stored and reported by the UE in SCG failure information message.

  - 2.2 MRO for near failure successful case

When CHO with candidate SCG(s) is executed, a near-failure successful PCell change and/or a near-failure successful PSCell addition or PSCell change may happen.

For legacy SHR or SPR, T310/T312/T304 related trigger threshold is configured by network to trigger SHR or SPR. It seems that there is no issue to reuse legacy SHR or SPR trigger threshold for the CHO with candidate SCG(s) procedure.

When CHO with candidate SCG(s) is performed, both CHO execution condition of the target PCell and corresponding CPAC execution condition of the associated target PSCell are fulfilled, there is a possibility that the time duration between the fulfilment of CHO execution condition and the fulfilment of CPAC execution condition is higher than a threshold, which may cause a near-failure successful CHO with candidate SCG(s). Therefore, besides legacy T310/T312/T304 related trigger threshold, RAN2 can consider new trigger condition(s) for SHR or SPR for the CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell. When the time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell is higher than the threshold, SHR and SPR may be generated by the UE.

Proposal 4: Introduce new trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell.

No matter what is the trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), for sub-optimal successful CHO execution, to help network know how to modify the configuration of the CHO with candidate SCG(s) procedure, R17 SHR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) can be stored and reported by the UE in SHR. Also, to follow the agreement that “the time information (as agreed for RLF) are included in SHR”, the time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell) can be stored and reported by the UE in SHR.

Proposal 5: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), can be stored and reported by the UE in SHR.

Similarly, for any sub-optimal successful CPAC, R18 SPR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell) can be stored and reported by the UE in SPR.

Proposal 6: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), can be stored and reported by the UE in SPR.

3	Conclusion

In this paper, MRO enhancements for CHO with candidate SCG(s) are discussed. We have the following proposals:

Proposal 1: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, can be stored and reported by the UE in SCG failure information message.

Proposal 2: Identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before connection failure is encountered can be stored and reported by the UE in SCG failure information message.

Proposal 3: Time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), as well as the elapsed time between the point in time of the first fulfilled execution condition and SCG failure, can be stored and reported by the UE in SCG failure information message.

Proposal 4: Introduce new trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell.

Proposal 5: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), can be stored and reported by the UE in SHR.

Proposal 6: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), can be stored and reported by the UE in SPR.

Reference

[1] R3-247908, LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT, Samsung

[2] R3-244223, Failure scenarios on MRO for CHO with candidate SCGs, Samsung, Cybercore, ZTE