3GPP TSG-RAN WG2 Meeting#129	R2-2500624

Athens, Greece, Feb.17–21, 2025

Agenda item:	 8.10.4

Source:		Lenovo

Title:		Discussion on MRO for intra-NTN mobility

Document for:		Discussion and Decision

1	Introduction

RAN3#126 meeting achieved several agreements and sent an LS to RAN2 [1]

On MRO for NTN, RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not.

In this paper, we would discuss MRO for intra-NTN mobility.

2	Discussion

RAN3#124 meeting agreed MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover, and the following trigger conditions for MRO for intra-NTN mobility shall be considered:

- Measurement-based trigger condition;

- A time-based trigger condition;

- A location-based trigger condition;

- A time-based and Measurement-based combined trigger condition;

- A location-based and Measurement-based combined trigger condition;

For time-based and measurement-based combined trigger condition:

RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not.

As we can find, currently the firstTriggeredEvent-r17 IE is included in the RLF report, if the firstTriggeredEvent-r17 IE is marked as condEventT1-r17, it means the fulfilled CHO trigger conditions before RLF occurs is time based trigger condition, if the firstTriggeredEvent-r17 IE is marked as condEventA3-r16 or condEventA5-r16, it means the fulfilled CHO trigger conditions before RLF occurs is measurement based trigger condition, if the firstTriggeredEvent-r17 IE is not present, it means none execution condition is fulfilled. Therefore, the existing firstTriggeredEvent-r17 IE can be reused to indicate the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition”.

Proposal 1: The existing firstTriggeredEvent-r17 IE can be reused to indicate the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition”.

For CHO configured with time-based and measurement-based combined trigger condition, there may be a case that an RLF happens due to the absolute time value t1-Threshold + duration arrives, but measurement based CHO execution condition is not fulfilled. Since signal strength/quality difference between NTN cell center and cell edge is small, time-based trigger condition has bigger impacts to mobility performance compared with measurement-based trigger condition in CHO NTN, it is beneficial to enable network know that measurement based CHO execution condition is setting inappropriately to avoid such a failure case, therefore, measurement results of source cell, target cell and candidate cell(s) at the absolute time value t1-Threshold + duration, may be stored or reported by the UE in RLF report.

Proposal 2: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) at the absolute time value t1-Threshold + duration.

For location-based and measurement-based combined trigger condition:

RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not.

Similar as the case of “time and measurement based trigger condition”, if the firstTriggeredEvent-r17 IE is marked as condEventD1-r17, it means the fulfilled CHO trigger conditions before RLF occurs is location based trigger condition, if the firstTriggeredEvent-r17 IE is marked as condEventA3-r16 or condEventA5-r16, it means the fulfilled CHO trigger conditions before RLF occurs is measurement based trigger condition, if the firstTriggeredEvent-r17 IE is not present, it means none execution condition is fulfilled. Therefore, the existing firstTriggeredEvent-r17 IE can be reused to indicate the fulfilled CHO trigger conditions before RLF occurs in case of “location and measurement based trigger condition”.

Proposal 3: The existing firstTriggeredEvent-r17 IE can be reused to indicate the fulfilled CHO trigger conditions before RLF occurs in case of “location and measurement based trigger condition”.

For both earth fixed case and earth moving case, for CHO configured with location-based and measurement-based combined trigger condition, there may be a case that location based CHO execution condition is fulfilled but measurement based CHO execution condition is not fulfilled before RLF happens, since signal strength/quality difference between NTN cell center and cell edge is small, location-based trigger condition has bigger impacts to mobility performance compared with measurement-based trigger condition in CHO NTN, to enable network know that measurement based CHO execution condition is setting strictly, measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled before RLF happens, may be stored or reported by the UE in RLF report.

Proposal 4: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled before RLF happens.

Additionally, for the joint trigger condition, there may be another case that measurement based CHO execution condition is fulfilled but location based CHO execution condition is not fulfilled before RLF happens, to enable network know that location based CHO execution condition is not setting properly, it is beneficial to enable network know the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled before RLF happens, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled before RLF happens, we can see that network can’t derive them because it does not know the time and UE location when only measurement based CHO execution condition is fulfilled. Therefore, the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled before RLF happens, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled before RLF happens, may be stored or reported by the UE in RLF report.

Proposal 5: RLF report can be enhanced to include the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled before RLF happens, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled before RLF happens.

3	Conclusion

In this paper, MRO enhancements for intra-NTN mobility are discussed. We have the following proposals:

Proposal 1: The existing firstTriggeredEvent-r17 IE can be reused to indicate the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition”.

Proposal 2: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) at the absolute time value t1-Threshold + duration.

Proposal 3: The existing firstTriggeredEvent-r17 IE can be reused to indicate the fulfilled CHO trigger conditions before RLF occurs in case of “location and measurement based trigger condition”.

Proposal 4: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled before RLF happens.

Proposal 5: RLF report can be enhanced to include the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled before RLF happens, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled before RLF happens.

4 Reference

[1] R3-247908, LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT, Samsung