3GPP TSG-RAN WG2 Meeting#129	R2-2500625

Athens, Greece, Feb.17–21, 2025

Agenda item:	 8.10.5.1

Source:		Lenovo

Title:		Discussion on RACH optimization for SDT

Document for:		Discussion and Decision

1	Introduction

Until now RAN2 have achieved agreements for RACH optimization for SDT including:

For the purpose of SON enhancements for SDT, include RSRP/data volume related information

Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report.

Do not add logging of sdt-RSRP-Threshold, since already agreed by RAN3 to not support it.

UE logs and reports the failure cause for SDT to the network. FFS the details, e.g. if we down select some of the failure causes.

For failed SDT case, UE includes the DL RSRP and UL data volume at the time of SDT evaluation in SON report. For successful SDT procedure, the UE does not log.

RAN2 understands for SON/MDT R19 the SDT enhancements only relate to the RA-SDT procedure. This does not rule out the case when the UE falls back from RA-SDT.

The UE logs the DL RSRP and UL data volume for the failed to initiate of RA-SDT in RA report.

The following failure causes can be logged for failed SDT in RA report:

⁻	upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing;

⁻	upon random access problem indication is received from MCG MAC while SDT procedure is ongoing;

⁻	upon T319a expires;

⁻	upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing;

⁻	FFS: unsuccessfully completed upon cell re-selection.

In this paper, we would further discuss RACH optimization for RA-SDT.

2	Discussion

SDT is a procedure allowing data transmission while remaining in RRC_INACTIVE without transitioning to RRC_CONNECTED state. SDT is enabled on a radio bearer basis and is initiated by the UE only if less than a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, measured RSRP in the cell is above a configured threshold, and a valid resource for SDT transmission is available.

Last RAN2#128 meeting agreed the UE logs the DL RSRP and UL data volume for the failed to initiate of RA-SDT in RA report. Even RA-SDT can be initiated, the ongoing RA-SDT procedure may fail due to a low threshold of radio link quality or due to a high threshold of data volume, for example, the UE triggers RA-SDT but data transmission fails because of bad radio link quality or heavy data volume. When RA-SDT fails, the UE may re-trigger SDT transmission or enter RRC connected for data transmission.

To improve RA-SDT performance, it is beneficial to store and report the RSRP when the triggered RA-SDT fails, then network can analyse the failure issue e.g. whether the RSRP threshold used to decide whether to trigger RA-SDT is set properly, for example, in the case that the RSRP threshold and RSRP when RA-SDT fails are close, it means that the RSRP threshold is set too low, network may modify with a higher RSRP threshold. Similarly, it is beneficial to store and report the data volume when the triggered RA-SDT fails, network can analyse whether the data volume threshold used to decide whether to trigger RA-SDT is set properly, e.g., in the case that the data volume threshold and data volume when RA-SDT fails are close, it means that the data volume threshold is set too high, network may modify with a lower data volume threshold.

Proposal 1: The UE reports RSRP and data volume when triggered RA-SDT fails in the RA report for SDT optimization.

Additionally, RAN3#126 meeting achieved agreements and sent an LS to RAN2 [1]:

On SON for RA-SDT, RAN3 sees benefits in including in RA Report the time from the start of RA-SDT until the reporting of the RA report. The network can use it to determine the configuration that was applied at the time of the start of RA-SDT.

Based on the LS [1] from RAN3, the time from the start of RA-SDT until the reporting of the RA report is beneficial for UE context identification at the node when receiving the RA report, therefore, the time from the start of RA-SDT until the reporting of the RA report should be included in the RA Report.

Proposal 2: The time from the start of RA-SDT until the reporting of the RA report should be included in the RA Report.

3	Conclusion

In this paper, RACH optimization for RA-SDT are discussed. We have the following proposals:

Proposal 1: The UE reports RSRP and data volume when triggered RA-SDT fails in the RA report for SDT optimization.

Proposal 2: The time from the start of RA-SDT until the reporting of the RA report should be included in the RA Report.

4 Reference

[1] R3-247908, LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT, Samsung