3GPP TSG-RAN WG2 Meeting #129	R2-2500630

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:		8.6.2

Source:		Lenovo

Title:		Some issues for Inter-CU LTM

Document for:		Discussion and Decision

- Introduction

This contribution aims to discuss the further potential issues for Inter-CU LTM.

- Discussion

Rel-19 ID transferred to target gNB

RAN2 agreed that Rel-19 ID is configured for a candidate configuration. If the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update. For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18. Source cell is responsible for Rel-19 ID assignment. Once the candidate cell is changed as the new serving cell due to cell switching, the new serving cell will be responsible for Rel-19 ID assignment. Therefore, the new serving cell should be aware of the assigned Rel-19 ID list.

Beside Inter-CU MCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3 in inter-CU SCG LTM.

Proposal 1: The assigned Rel-19 ID list for all the serving cell and candidate cells should be transferred to target gNB (target MN for Inter-CU MCG LTM or target SN for Inter-CU SCG LTM). LS is needed for RAN3.

Coexistence between Inter-CU MCG LTM and Subsequent CPAC

In Rel-18 LTM, simultaneous configuration for both LTM and CPAC is allowed. After Inter-MN PCell change, the configuration for subsequent CPAC should be released by network via explicit indication. We can follow the same behavior that the configuration for subsequent CPAC should be released by network via explicit indication after Inter-CU MCG LTM.

- RAN2#121 In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case).

Proposal 2: Subsequent CPAC can be configured while Inter-CU MCG LTM or SCG LTM is configured.

Proposal 3: The configuration for subsequent CPAC should be released by network via explicit indication after Inter-CU MCG LTM as legacy.

Inter-CU SCG LTM and MCG failure recovery

In Rel-18 Intra-CU SCG LTM, Intra-CU SCG LTM switch is allowed while MCG failure recovery procedure is ongoing. More specifically, UE will trigger fast MCG link recovery upon RLF on MCG. After UE transmits MCGFailureInformation message to MCG via SCG, UE starts T316 and waits for the response e.g handover command. Intra-CU SCG LTM could be triggered while T316 is running. That means the response will be pending in SCG until UE completes SCG change triggered LTM.

Regarding Inter-CU SCG LTM, if Inter-CU SCG LTM cell switch is allowed while MCG failure recovery procedure is ongoing. The response from MCG to UE should be transmitted to target SN. Therefore, once source SN activates Inter-CU LTM switching, source SN indicates it to MN. Then, MN transmits the response e.g reconfiguration message for PCell change to target SN. Specifically, the SN-DU inform the SN-CU about the initiation of the cell switch command to the UE via DU-CU Cell Switch Notification. Then, SN-CU informs it to MN-CU.

Proposal 4: Inter-CU SCG LTM switch is allowed to be triggered while MCG failure recovery procedure is ongoing as legacy Intra-CU SCG LTM.

Proposal 5: Once source SN activates Inter-CU LTM switching while MCG failure recovery procedure is ongoing, source SN indicates it to MN. Then, MN transmits the response of MCGFailureInforamtion to target SN.

Coexistence between Inter-CU MCG LTM and CHO/CPAC

It was agreed that L3 HO and CHO can be configured to UE, while the inter-CU LTM is configured without DC configuration. When UE performs the L3 mobility (traditional HO or CHO), the UE does not autonomously release inter-CU LTM configurations as Rel-18, unless these are explicitly released by the network. The RRCReconfiguration message to execute an L3 mobility (traditional HO or CHO) procedure may reconfigure inter-CU LTM configurations. In Rel-16, once UE performs handover, UE stops evaluating the execution condition(s) once a handover is executed. Similarly, UE can stop evaluating the CHO candidate cell once Inter-CU LTM is executed as legacy.

TS38.300 9.2.3.4.1 General A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE starts evaluating the execution condition(s) upon receiving the CHO configuration, and stops evaluating the execution condition(s) once a handover is executed.

Proposal 6: UE stops evaluating the CHO candidate cell once Inter-CU LTM is executed as legacy. No specification change.

In Rel-18 LTM, after MCG LTM cell switch execution, the UE shall remove all CHO/CPAC configurations except for the configuration of subsequent CPAC. That means UE will remove all CHO and CPAC configuration after MCG LTM cell switch is completed. It is natural that the UE shall remove all CHO/CPAC configurations after Inter-CU MCG LTM cell switch is completed.

Proposal 7: The UE shall remove all CHO/CPAC configurations after Inter-CU MCG LTM cell switch is completed as legacy. No specification change.

Security in Inter-CU mobility

RAN2 has sent an LS to SA3 including some potential on high level and sought their opinion. Further to the LS, for which reply from SA3 has responded in S3-244316.

SA WG3 thanks RAN WG2 for the LS on the security handling for inter-CU LTM in non-DC cases. SA WG3 has been considering the security mechanisms proposed for the inter-CU LTM feature. For this feature, SA WG3 want to preserve the principle that when there is a PDCP anchor change, then it is necessary to perform a key change (i.e. for solutions or handovers that do not require a PDCP anchor change, there is no need to provide rekeying). The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure. SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like, - Security threat of the unprotected MAC CE to deliver the NCC value. - Sending NCC value per candidate cell via RRC message during LTM preparation phase. SA WG3 will keep RAN WG2 and RAN WG3 informed as we make further progress on this topic

In addition to SA3 response, it is important for UE to distinguish if no-key derivation is applied, or a horizontal or vertical key derivation is to be applied. As SA3 only envisages to “after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling”, a subsequent mobility without a RRC signaling is not possible and moreover if a new NH, NCC pair becomes available AFTER this RRC signalling how can this be taken into account? So, it is best to do the following:

- Configure a set of NCC value to be used in sequence for each appearance of the UE in a given candidate cell

- If a new NH, NCC pair would become available at any time (i.e., due to a new NAS AKA run), then the network should transmit keySetChangeIndicator to the UE.

Proposal 8: Regarding inter-CU security change, signalling should enable UE to distinguish if no-key derivation is applied, or a horizontal or vertical key derivation is to be applied. If a vertical key derivation is to be used, then a fresh RRC signalling including keySetChangeIndicator is sent to the UE.

Proposal 9: If a NAS AKA is run (and NH, NCC pair becomes available), then a fresh RRC signalling including keySetChangeIndicator is sent to the UE.

Finally, we also asked SA3 question on security Algorithm change. Since this should be relatively rare, implementing the key change using RRC should be acceptable.

Proposal 10: When the security Algorithm change is required, it is performed using layer 3 handover.

UE-based TA measurement

UE-based TA measurement is supported in Rel-18 based on RAN4 LS (R4-2314455). In UE-based TA measurement, UE acquires the TA value(s) of the candidate cell(s) by measurement. According to LS, whether UE based TA measurement can be supported depends on TAE, DL timing estimation error (of both serving and target cell), serving cell TA resolution error and TA adjustment error. Therefore, network e.g source CU can decide whether UE based TA measurement can be supported in Inter-CU LTM. In Rel-18 LTM, source CU will transmit UE based TA measurement ID for serving cell and candidate cell to source DU which will helpful for source DU to decide whether to trigger RACH based early TA procedure. Also, source CU will transmit UE based TA measurement ID for serving cell and candidate cell to candidate DU because candidate DU should be aware of whether the triggered LTM is RACHless or not. Similarly, if UE based TA measurement can be supported in Inter-CU LTM, source CU needs to transmit UE based TA measurement ID for serving cell and candidate cell to source DU and candidate DU.

Observation 1: Based on LS (R4-2314455), whether UE based TA measurement can be supported depends on TAE, DL timing estimation error (of both serving and target cell), serving cell TA resolution error and TA adjustment error.

Proposal 11: UE based TA measurement can be supported for Inter-CU LTM.

Proposal 12: If UE based TA measurement can be supported in Inter-CU LTM, source CU can decide whether to configure UE based TA measurement for a candidate cell.

Conclusion

In this contribution, the following proposals are given based on the discussion:

Rel-19 ID transferred to target gNB

Proposal 1: The assigned Rel-19 ID list for all the serving cell and candidate cells should be transferred to target gNB (target MN for Inter-CU MCG LTM or target SN for Inter-CU SCG LTM). LS is needed for RAN3.

Coexistence between Inter-CU MCG LTM and Subsequent CPAC

Proposal 2: Subsequent CPAC can be configured while Inter-CU MCG LTM or SCG LTM is configured.

Proposal 3: The configuration for subsequent CPAC should be released by network via explicit indication after Inter-CU MCG LTM as legacy.

Inter-CU SCG LTM and MCG failure recovery

Proposal 4: Inter-CU SCG LTM switch is allowed to be triggered while MCG failure recovery procedure is ongoing as legacy Intra-CU SCG LTM.

Proposal 5: Once source SN activates Inter-CU LTM switching while MCG failure recovery procedure is ongoing, source SN indicates it to MN. Then, MN transmits the response of MCGFailureInforamtion to target SN.

Coexistence between Inter-CU MCG LTM and CHO/CPAC

Proposal 6: UE stops evaluating the CHO candidate cell once Inter-CU LTM is executed as legacy. No specification change.

Proposal 7: The UE shall remove all CHO/CPAC configurations after Inter-CU MCG LTM cell switch is completed as legacy. No specification change.

Security in Inter-CU mobility

Proposal 8: Regarding inter-CU security change, signalling should enable UE to distinguish if no-key derivation is applied, or a horizontal or vertical key derivation is to be applied. If a vertical key derivation is to be used, then a fresh RRC signalling including keySetChangeIndicator is sent to the UE.

Proposal 9: If a NAS AKA is run (and NH, NCC pair becomes available), then a fresh RRC signalling including keySetChangeIndicator is sent to the UE.

Proposal 10: When the security Algorithm change is required, it is performed using layer 3 handover.

UE-based TA measurement

Proposal 11: UE based TA measurement can be supported for Inter-CU LTM.

Proposal 12: If UE based TA measurement can be supported in Inter-CU LTM, source CU can decide whether to configure UE based TA measurement for a candidate cell.

References

[1] Chair notes for RAN2#128