3GPP TSG-RAN WG2 Meeting #129                                  R2-2500639

Athens, Greece, Feb. 17th – 21st , 2025

Agenda item:		8.3.2.1

Source:				Samsung

Title:					Discussion on simulation results for RRM measurement prediction

Document for:		Discussion & Decision

1	Introduction

In RAN2#126, RAN2 agreed the following table on the prioritization between different evaluation scenarios/cases for RRM measurement prediction.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we show our evaluation results for the cases with ‘High’ priority and discuss some observations based on the results.

2	Discussion

2.1	Evaluation results for RRM measurement prediction

In this section, we would like to show our evaluation results for the high priority cases and share some observation based on the results. We focus on the evaluation for L3 cell-level RSRP prediction and beam-level spatial domain prediction is deprioritized since it is already studied in R18 AI/ML for Air SI. Also, our system level simulator adopts all the agreed mandatory simulation assumption that should be used by all the companies as summarized in Annex part.

2.1.1 General evaluation parameters/assumptions

As a result of the offline e-mail discussion [POST126][031], some simulation parameters/assumptions are agreed to be reported by the companies with their simulation results [1]. The following Table 1 shows the set of parameters commonly used for all the cases in our evaluation. The other parameters specific to each case are reported in the following sections for each case, separately.

Table 1

| Report parameters |  | Samsung |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 3 |
|  | UE trajectory boundary processing option | boundary-terminated model |
|  | UE speed (30,60,90,120 Km/h) | 30, 60, 90, 120 Km/h (for temporal domain prediction) 120Km/h (for frequency domain prediction) |
|  | Whether LOSsoft is modeled or not | Yes |
|  | Spatial consistency option (A or B in TR 38.901) | B |
|  | Number of TX beams | 1 (for FR1) / 32 (for FR2) |
|  | Number of RX beams | 1 (for FR1) / 4 (for FR2) |
| Any other parameters | L1/L3-filtering option (sliding or non-sliding) | sliding |
|  | L1-filtering window length | 10 samples |
|  | Target evaluation cells | 7 nearest cells (serving + 1st-tier neighbour cells) |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM autoencoder 2 LSTM layers for encoder 2 LSTM layers + 3 dense layers for decoder |
|  | Model complexity in a number of parameters(M) | 4.5K |
|  | Model complexity in model size (e.g. Mbyte) | ~200 Kbyte |
|  | Computational complexity [FLOPs] | ~20K |

For L1/L3-filtering option, we assume the sliding case where the L1/L3 measurement results are produced every sampling period (i.e., 40msec for FR1 and 80msec for FR2) as described in Figure 1 in [2]. From our understanding, the measurement period in RAN4 specification (e.g., 200msec) just provides minimum time requirements for RRM measurement and thus it does not mean that L1/L3 measurement results cannot be generated every sampling period. In practice, for measurement event triggering, the UE should continuously check if an entry condition is fulfilled for the measurement results during TTT (e.g., 0ms, 40 ms, 64 ms, …). Thus, we think it is reasonable that the UE can have L1/L3 measurement results with the agreed sampling period rather than the measurement period.

For evaluation of the prediction accuracy, we use the data sets from UE’s current serving cell and the 1st-tier neighbour cells (i.e., not consider the 2nd/3rd-tier neighbour cells for prediction target cells). In our view, there is no need for UE to perform RRM measurement prediction for the 2nd/3rd-tier neighbour cells since they are unlikely to be the serving cell/HO target cells for the UE. Evaluating the prediction accuracy including the cells that cannot be the prediction target cells in practice may distort the evaluation results. Therefore, we limit the target evaluation cells to the UE’s current serving and 1st-tier neighbour cells.

2.1.2 FR2 to FR2 intra-frequency temporal domain case A (Case 4)

The following Table 2 shows our simulation assumption and results for the Case 4 (i.e., temporal domain prediction Case A in FR2).

Table 2

| Report parameters |  | Samsung |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | OW | 80/400/800/1600 msec (1/5/10/20 sampling periods) |  |  |  |  |
|  | PW | 400 msec (5 sampling periods) |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | 31000/3500 samples* |  |  |  |  |
|  | Testing | 9000 samples* |  |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) | ~200 Kbyte |  |  |  |  |
|  | Computational complexity [FLOPs] | 20K |  |  |  |  |
| AI/ML model input/output | Model input | Historical L3 cell-level RSRP within OW |  |  |  |  |
|  | Model output | Future L3 cell-level RSRP within PW |  |  |  |  |
| Metrics | Average L3 cell level RSRP difference (dB) | OW | Average RSRP diff. (dB) |  |  |  |
|  |  |  | 30km/h | 60km/h | 90km/h | 120km/h |
|  |  | 80 msec | 1.389 | - | - | - |
|  |  | 400 msec | 0.864 | - | - | - |
|  |  | 800 msec | 0.817 | - | - | - |
|  |  | 1600 msec | 0.802 | 1.175 | 1.287 | 1.445 |

*Note: Each sample consists of input RSRP values and output RSRP values.

<The impact of OW length>

For Case A prediction, the model is trained to predict the future L3 cell-level RSRP within the prediction window based on the historical L3 cell-lever RSRP within the observation window. Here, we fix the prediction window as the agreed initial value (i.e., 400 msec for FR2) and evaluate the prediction accuracy with different observation window size. The results for the UE speed 30 km/h show that the average L3 RSRP difference can be 1.38/0.864/0.817/0.802dB for distinct observation window lengths of 1/5/10/20 samples, respectively.

From the evaluation results, it is observed that the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved) as we use the longer observation window size. This makes sense because a longer observation window provides more information for the AI/ML model, which can be used to improve the prediction accuracy. However, Figure 1 below shows that the accuracy improvement becomes marginal when the observation window length is over 5 samples.

Figure 1. L3 RSRP difference with different observation window sizes

Observation 1: As we use a longer observation window, the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved). However, the accuracy improvement becomes marginal when the observation window length is over 5 samples.

On the other hand, the longer observation window can increase the signalling/memory overhead in the model training/inference procedure. For example, if we assume the use of gNB-side model for RRM measurement prediction, the UE is required to report the input data (i.e., historical L3 RSRP measurement within the observation window) to the gNB for inference. Therefore, in this case, the use of longer observation window will increase the signalling overhead between UE and gNB and also require larger memory size at the UE side. The same goes for the UE-side model as well since the UE needs to provide the label data (i.e., set of input and output data) to the training entity (e.g., UE-side OTT server).

Observation 2: For RRM measurement prediction, the use of a longer observation window can increase the signalling/memory overhead in LCM procedure (especially for model training/inference).

Based on the observations above, there exists a trade-off between prediction accuracy and signalling/memory overhead depending on the length of observation window. Therefore, we cannot continuously extend the observation window to improve the prediction accuracy but instead should determine a suitable value for it. In our evaluation,’20 samples’ is chosen as a value for the observation window length, and this value is commonly used in the other cases (i.e., Case 2/3) as well.

Observation 3: There is a trade-off between prediction accuracy and signaling/memory overhead depending on the length of observation window. A reasonable value for the length of observation window should be determined considering the trade-off relation.

Proposal 1: For RRM measurement prediction, RAN2 to capture the following observations in TR 38.744 to explain the effect of observation window length on prediction accuracy and signaling/memory overhead.

- As we use a longer observation window, the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved).

- The use of a longer observation window can increase the signalling/memory overhead in LCM procedure (especially for model training/inference).

- There is a trade-off between prediction accuracy and signaling/memory overhead depending on the length of observation window.

- A reasonable value for the length of observation window should be determined considering the trade-off relation.

<Use of L1 beam-level input>

For cell level RRM measurement prediction, RAN2 agreed to study the following sub-cases at RAN2#125bis.

- Sub-case 1: To predict beam level results, then generate cell level results based on the predicted beam results.
(Input: L1 beam-level RSRP, Output: L1 beam-level RSRP  L3 cell-level RSRP after consolidation)

- Sub-case 2: To directly predict cell level results based on cell level results.
(Input: L3 cell-level RSRP, Output: L3 cell-level RSRP)

- Sub-case 3: To directly predict cell level results based on beam level results
(Input: L1 beam-level RSRP, Output: L3 cell-level RSRP)

To check the performance of the sub-cases 1/2/3, we evaluate the accuracy of RRM measurement prediction (i.e., L3 RSRP difference) for each of the sub-cases with the same parameters in Table 1&2, except the model input/output. The following Table 3 shows the evaluation results.

Table 3

|  | Sub1 (L1 beam L1 beam) | Sub2 (L3 cell L3 cell) | Sub3 (L1 beam L3 cell) |
| --- | --- | --- | --- |
| L3 RSRP difference | 2.019 dB | 0.802 dB | 1.915 dB |

In our view, the important point here is whether the use of L1 beam-level RSRP input really helps improve the prediction accuracy and the answer from the result is ‘No’. We can see that the subcase 2 with L3 cell-level input shows much better performance compared to the subcase 1&3 with L1 beam-level input in prediction accuracy. On the other hand, it is obvious that the use of L1 beam-level RSRP will result in an increase in signalling/memory overhead for model training/inference. The input data size will increase by up to 64 times when we assume the use of L1 beam-level RSRP. From our understanding, if we provide L1 beam-level RSRP as an input data for AI/ML model for the subcase 1&3, the model should become much more complex to learn the meaning of input data in the extended dimension. Moreover, for the sub case 3, the model should additionally learn the logic of the beam consolidation mechanism, which does not need to be part of AI/ML model inference. It can be performed by the legacy rule-based algorithm without the unnecessary overhead. In summary, the use of L1 beam-level input can increase the signalling/memory overhead without much gain in RRM measurement prediction.

Observation 4: Sub-case 2 with L3 cell-level input shows much better prediction accuracy compared to Sub-case 1&3 with L1 beam-level input.

Observation 5: The use of L1 beam-level input can increase signalling/memory overhead without gain in RRM measurement prediction.

<AI based approach vs. Non-AI based approach>

For Non-AI approach, Sample-and-Hold method is adopted for the comparison with our AI approach in temporal domain Case A prediction. The following Table 4 shows the prediction accuracy (i.e., RSRP difference) of each approach with different UE speeds.

Table 4

| Case | UE speed (km/h) | Prediction accuracy (RSRP difference) |  | Gain (RSRP difference gap) |
| --- | --- | --- | --- | --- |
|  |  | AI | Non-AI (S&H) |  |
| Case A prediction in FR2 | 30 | 0.802 dB | 1.377 dB | 0.575 dB |
|  | 60 | 1.175 dB | 1.746 dB | 0.571 dB |
|  | 90 | 1.287 dB | 1.933 dB | 0.646 dB |
|  | 120 | 1.445 dB | 2.094 dB | 0.649 dB |

Based on the simulation results, we can observe that AI shows better prediction accuracy (i.e., lower RSRP difference) than non-AI approach in all cases across different UE speeds. Also, the gain of AI approach compared to non-AI increases as UE speed increases. The results seem reasonable to us because the performance of Non-AI of maintaining the last measurement value inevitably deteriorates as the channel variation of UE intensifies.

Observation 6: For Case A temporal domain prediction, AI can provide gain (i.e., lower L3 cell-level RSRP difference) compared to Non-AI (i.e., Sample and Hold). The gain increases as UE speed increases.

2.1.3 FR1 to FR1 intra-frequency temporal domain case B (Case 2)

The following Table 5 shows our simulation assumption for the Case 2 (i.e., temporal domain prediction Case B in FR1).

Table 5

| Report parameters |  | Samsung |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | Measurement reduction rate (R) | 50% | 66% |  | 80% |  |
|  | OW | 800 ms | 800 ms |  | 800 ms |  |
|  | PW | 40 ms | 80 ms |  | 160 ms |  |
|  | Filtering options (1/2/3) in [4] | Filtering option 3: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T1 (i.e. the old L3 filtered cell result from last actual measurement) |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | 59000/6500 samples* |  |  |  |  |
|  | Testing | 9500 samples* |  |  |  |  |
| AI/ML model input/output | Model input | Historical L3 cell level RSRP in OW with the interval (= sampling period * 1/(1-R) ) |  |  |  |  |
|  | Model output | Future L3 cell level RSRP in PW with the interval (= sampling period) |  |  |  |  |
| Metrics | Average L3 cell level RSRP difference (dB) | Measurement reduction rate (MRRT) | UE speed |  |  |  |
|  |  |  | 30km/h | 60km/h | 90km/h | 120km/h |
|  |  | 50% | 0.264 | 0.311 | 0.449 | 0.47 |
|  |  | 66.7% | 0.376 | 0.496 | 0.707 | 0.745 |
|  |  | 80% | 0.656 | 0.871 | 1.102 | 1.303 |

*Note: Each sample consists of input RSRP values and output RSRP values.

For Case B prediction, we assume the example 2 described in Figure 2.1-2 in the summary of [POST127][030] [3]. The model is trained to predict the future L3 cell-level RSRP with the interval of ‘sampling period (i.e., 40msec)’ in PW based on the historical L3 cell-level RSRP with the interval of ‘sampling period *1/(1-R)’ in OW. For example, let’s assume the case with measurement reduction rate (= 2/3) as in Figure 2 below. In this scenario, at the specific time instance ‘Tn’, the model inference can be performed to predict the future RSRP values for 2 distinct time instances between the current (Tn) and the subsequent (Tn+1) measurement time instance. With the assumption, the number of predicted time instances can be 1, 2, 4 for the distinct measurement reduction rates 1/2(=50%), 2/3(=66.7%), 4/5(=80%), respectively.

Figure 2. Example of Case B prediction with the measurement reduction rate 2/3(=66%)

During simulation the input L3 filtered RSRP result is achieved based on L1 beam results of dataset. In Case B where some measurements are skipped, it is unclear how to perform L3 filtering (at T3 in Figure 3) in the case that previous L3 filtered result is from prediction (at T2 in Figure 3). The following three options are provided in [4].

Figure 3. Temporal domain Case B example 2

- Filtering option 1: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T2;

- Filtering option 2: L3 cell result at T3 is filtered based on the L1 filtered result at T3; in other words, not consider old L3 filtered result.

- Filtering option 3: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T1 (i.e. the old L3 filtered cell result from last actual measurement);

For option 1, the output from the previous prediction would be used as part of input for the following prediction. I.e., the predicted value at T2 is used to calculate the L3 filtered value at T3 which is provided as input data for the prediction at T4. In this case, the prediction error would be propagated/accumulated over time. Also, this can increase our simulation time too much because all the prediction should be performed in order, which doesn’t allows parallel computing. The option 2 means no L3 filtering in practice. The option 3 increases the interval between two samples (L3 filtered value at T1 and T3) for L3 filtering, which can make the actual channel variation applied to L3 filtered RSRP more slowly compared to the option 1.

In our view, the option 1 is not aligned with the legacy L3 filtering process since it is not only based on actual measurement but also based on predicted results, which also makes the prediction error accumulated over time. The option 2 does not make sense since it actually means no L3 filtering. Thus, in our simulation, we adopt the option 3 as our L3 filtering option.

<Impact of measurement reduction rate (MRRT)>

Table 5 shows the evaluation results that the average L3 cell level RSRP difference for the UE speed 30km/h can be 0.264, 0.376, 0.656 dB for each of the different measurement reduction rates 1/2(=50%), 2/3(=66.7%), 4/5(=80%), respectively. From the results, we can also find the tradeoff between measurement reduction rate and prediction accuracy. As the higher measurement reduction rate is used for Case B prediction, the measurement overhead at the UE side decreases, but the RSRP difference increases, which can result in HO KPI degradation. From UE’s perspective, it can prefer a higher measurement reduction rate to reduce its measurement overhead, but NW may be conservative about reducing the measurement overhead too much to maintain overall HO performance. Thus, we need to study how UE/gNB can determine or negotiate a suitable value of measurement reduction rate for Case B prediction.

Observation 7: For Case B prediction, there is a tradeoff between measurement reduction rate and prediction accuracy (i.e., L3 RSRP difference).

Based on the observations above, we would like to propose the following.

Proposal 2: For Case B temporal domain prediction, RAN2 to capture the tradeoff between measurement reduction rate and prediction accuracy in TR 38.744.

Proposal 3: RAN2 to study how UE/gNB can determine a suitable value of measurement reduction rate for Case B prediction.

<AI based approach vs. Non-AI based approach>

For Non-AI approach, Sample-and-Hold method is adopted for the comparison with our AI approach as in temporal domain Case A prediction. The following Table 6 shows the prediction accuracy (i.e., RSRP difference) of each approach with different MRRT and UE speed.

Table 6

| Case | UE speed (km/h) | MRRT (%) | Prediction accuracy (RSRP difference) |  | Gain (RSRP difference gap) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | AI | Non-AI (S&H) |  |
| Case B prediction in FR1 | 30 | 50 | 0.264 dB | 0.379 dB | 0.115 dB |
|  |  | 80 | 0.656 dB | 0.98 dB | 0.324 dB |
|  | 60 | 50 | 0.311 dB | 0.51 dB | 0.199 dB |
|  |  | 80 | 0.871 dB | 1.346 dB | 0.475 dB |
|  | 90 | 50 | 0.449 dB | 0.629 dB | 0.18 dB |
|  |  | 80 | 1.102 dB | 1.707 dB | 0.605 dB |
|  | 120 | 50 | 0.47 dB | 0.741 dB | 0.271 dB |
|  |  | 80 | 1.303 dB | 2 dB | 0.666 dB |

Based on the simulation results, we can observe that AI shows better prediction accuracy (i.e., lower RSRP difference) than non-AI approach in all cases with different MRRT and UE speeds. The average gain of AI approach is 0.22, 0.337, 0.393, 0.469 dB for each of UE speeds 30, 60, 90, 120 km/h, respectively. As in the Case A prediction, the gain of AI approach compared to non-AI increases as UE speed increases also in the Case B prediction. Meanwhile, we can also observe that the gain of AI approach over non-AI increases as the measurement reduction rate (MRRT) increases. In our simulation, we assume the baseline skipping pattern (i.e., 1:1 for MRRT 50%, 1:4 for MRRT 80%) in Figure 2 where a higher MRRT means a longer prediction window length. Thus, the performance of Non-AI of maintaining the last measurement value inevitably deteriorates more and more when we predict farther future with longer prediction window.

Observation 8: For Case B temporal domain prediction, AI can provide gain (i.e., lower L3 cell-level RSRP difference) compared to Non-AI (i.e., Sample and Hold). The gain increases as MRRT increases.

Proposal 4: For Case B temporal domain prediction, RAN2 to capture that AI can provide gain (i.e., lower L3 cell-level RSRP difference) compared to Non-AI (i.e., Sample and Hold) and the gain increases as MRRT increases.

2.1.4 FR1 to FR1 inter-frequency (frequency domain, Case 3)

The following Table 7 shows our simulation assumption/results for the Case 3 (i.e., inter-frequency prediction in FR1).

Table 7

| Report parameters |  | Samsung |  |  |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | Inter-frequency correlation assumption in general (yes or no) | Yes |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no) | Full |  |  |
| Data Size (Number of Samples) | Training/validity | 59000/6500 samples* |  |  |
|  | Testing | 9500 samples* |  |  |
| AI/ML model input/output | Model input | L3 cell level RSRP at the baseline freq. (4GHz) |  |  |
|  | Model output | L3 cell level RSRP at another freq. (2GHz) |  |  |
| Metrics | Average L3 cell level RSRP difference (dB) | UE speed |  |  |
|  |  | 30km/h | 60km/h | 120km/h |
|  |  | 4.144 | 3.907 | 3.61 |
|  | Channel correlation coefficient between two frequency layers | 0.899 | 0.902 | 0.918 |

*Note: Each sample consists of input RSRP values and output RSRP values.

For inter-freq. prediction, the model is trained to predict L3 cell-level RSRP of the cell in the inter-frequency based on L3 cell-level RSRP of the cell in the serving frequency. Specifically, we use 20 samples of the measured L3 RSRP in 4GHz to predict the 20 samples of the L3 RSRP at the same time instances in 2GHz. For the inter-freq. correlation, we assume the case where the cell to be measured and the cell to be predicted are co-located (i.e., located in the same sector/site with the same antenna configuration) and adopt the inter-frequency shadow fading model in TR 38.901.

<Impact of correlation coefficient between two frequencies>

Figure 4. L3 RSRP of the two cells in different frequency

Figure 4 depicts the true L3 RSRP in 4GHz, the true L3 RSRP in 2GHz, and the predicted L3 RSRP in 2GHz. In general, the results show quite a high correlation between the L3 RSRP of the two cells in different frequency. The correlation coefficient between the L3 RSRP of the two cells is observed as 0.899, 0.902, 0.918 for each of UE speed 30, 60, 120 km/h, respectively.

In our simulation, sequence of RSRP values for the same cell are correlated in time domain based on the spatial consistency modelling. Meanwhile, RSRP values for the two co-located cells are correlated in frequency domain based on the inter-frequency correlation modelling. Therefore, RSRP value at a certain time instance for a cell are affected by both spatial and frequency domain correlation modelling. From our understanding, the higher UE speed results in lower correlation between RSRP values in time domain, which relatively increase the effect of frequency domain correlation. Therefore, we observe higher correlation coefficient between L3 RSRP of the two co-located cells with higher UE speed in our simulation. However, we understand companies can have different observation on the relation between UE speed and the correlation coefficient depending on their assumption on spatial consistency/inter-frequency channel modelling.

With higher correlation coefficient, we can expect higher prediction accuracy (i.e., low RSRP difference) for the inter-frequency prediction. In our simulation, the average L3 RSRP difference is observed as 4.144, 3.907, 3.61 dB with the correlation coefficient 0.899, 0.902, 0.918, respectively. The results show that the prediction accuracy increases as the correlation coefficient increases.

Observation 9: For the inter-frequency prediction, the evaluation results show that higher correlation coefficient between two frequency layers results in higher prediction accuracy.

<AI based approach vs. Non-AI based approach>

For Non-AI approach, pathloss offset method is adopted for the comparison with our AI approach. Specifically, in our simulation environment, the average L3 RSRP offset between the two frequencies (i.e., 2 GHz and 4GHz) is observed about 7dB. Therefore, in non-AI approach, we set the pathloss offset between 4GHz and 2GHz to about 7dB (each case with different UE speed may have slightly different offset values depending on the training data set). For AI approach, we evaluate the prediction accuracy for cell-based approach (i.e., one cell for input) and for cluster-based approach (i.e., 21 cells for input), respectively. Please, find more details on the cluster approach in Section 2.3 below. The following Table 8 shows the prediction accuracy (i.e., RSRP difference) of each approach with different UE speed.

Table 8

| Case | UE speed (km/h) | Prediction accuracy (RSRP difference) |  |  | Gain (RSRP difference gap between AI-Cluster and Non-AI) |
| --- | --- | --- | --- | --- | --- |
|  |  | AI-Cell | AI-Cluster | Non-AI (pathloss) |  |
| Case B prediction in FR1 | 30 | 4.144 dB | 3.108 dB | 4.22 dB | 1.112 dB |
|  | 60 | 3.907 dB | 3.222 dB | 4.326 dB | 1.104 dB |
|  | 120 | 3.61 dB | 2.94 dB | 4.131 dB | 1.191 dB |

Based on the simulation results, we can observe that both AI-Cell and AI-Cluster show better prediction accuracy (i.e., lower RSRP difference) than non-AI approach in all cases across different UE speeds. Since AI-Cluster shows better prediction accuracy compared to AI-Cell, we calculate the gain of AI approach based on the results of AI-Cluster. The gain of AI is 1.112, 1.104, 1.191 dB for each of UE speeds 30, 60, 120 km/h, respectively.

Observation 10: For the inter-frequency prediction, AI can provide gain (i.e., lower L3 cell-level RSRP difference) compared to Non-AI (i.e., pathloss offset).

2.2	Impact of L1 filtering window length on prediction accuracy

In this section, we would like to discuss the impact of L1 filtering window length on RRM measurement prediction. For evaluation, RAN2 agreed to use fast fading channel model for RRM measurement prediction. With the fast fading channel model, the L1 raw RSRP of a certain beam/cell can randomly fluctuate even though it has time domain correlation according to the spatial consistency option A/B in TR 38.901. In our simulation, we adopt the option B and observe that L1 raw RSRP randomly fluctuates with an amplitude of about 6 dB.

Observation 11: With the agreed channel model based on TR 38.901, L1 raw RSRP of a certain beam/cell randomly fluctuates with an amplitude of about 6 dB.

In our view, the random fluctuation in L1 raw RSRP is reasonable considering the effect of fast fading and noise. However, in practice, it is impossible for the AI/ML model to predict that kind of randomness in L1 raw RSRP and thus we need to use the L1/L3-filtered value for RRM measurement prediction as in the legacy RRM operation.

For L3 filtering, RAN2 already agreed the value of L3 filtering parameter (i.e., FilterCoefficient = 4) for evaluation. However, for L1 filtering, it is up to companies what L1 filtering method to use for simulation. At the last meeting, RAN2 discussed two different L1 filtering options (i.e., sliding vs. non-sliding in [5]) and agreed to keep both on the table. In our evaluation, we adopt the option with sliding and assume the simplest filtering method of averaging the latest L1 raw RSRP values within the L1 filtering window.

To figure out the proper length of L1 filtering window, we evaluate the prediction accuracy of Case A prediction in FR2 with different length of L1 filtering window. Figure 4 below depicts the average L3 cell level RSRP difference according to prediction step with the different L1 filtering window length (i.e., 10, 20, 40, 80). Here, prediction step 1/2/3/4/5 means the prediction on the upcoming 1st/2nd/3rd/4th/5th sample periods from the inference time.

Figure 5. L3 RSRP difference with different L1 filtering window length

Figure 5 shows a trend the RSRP difference increases as the prediction step count increases. I.e., the accuracy of predication decreases when the RSRP prediction is for further future. The results indicate that the RSRP difference can be 0.846/0.551/0.383/0.283 dB for varying L1 filtering window lengths of 10/20/40/80 samples, respectively. Based on the results, it is observed that using a longer L1 filtering window can bring significant improvements in prediction accuracy.

Observation 12: The prediction accuracy can be improved as we use a longer L1 filtering window length.

Figure 6 below explains how the L1 filtering window length affects the RSRP difference of RRM measurement prediction. In the sub-figure (a), we can see that L3 RSRP randomly fluctuates a lot when there is no L1 filtering (i.e., L1 filtering window length = 1), which demonstrates the ineffectiveness of L3 filtering and highlights the necessity of L1 filtering. From the sub-figure (b)~(d), it is observed that the volatility caused by fast fading/noise in L3 RSRP decreases as the L1 filtering window length increases from 10 to 40. From our understanding, since it is impossible for AI/ML models to predict the random variation in L3 RSRP, the accuracy of prediction can be improved as the randomness decreases by using longer L1 filtering length.

Figure 6. L3 cell-level RSRP with varying L1 filtering window length

At first glance, one might think that using a longer L1 filtering length is always better to reduce the RSRP difference. However, if the L1 filtering length becomes too long, it will cause a delay in reflecting the real time channel variation to the L3 RSRP, which can lead to degradation of HO KPI.

Figure 7 below plots the L3 RSRP with the different L1 filtering window length (i.e., 1, 10, 20), which shows that longer filtering window length leads to the longer delay. It is observed that L3 RSRP graph is gradually shifted (delayed) in time domain as the L1 filtering window length increases.

Observation 13: The use of longer L1 filtering window length increases the delay of reflecting the real time channel variation to L3 RSRP.

Figure 7. Impact of L1 filtering window length on the channel variation delay

To check the impact of L1 filtering window on HO KPI, we also evaluate HO failure rate with different L1 filtering window length 10 and 20. The results show that HO failure rate increases about 6 % as the L1 filtering window length is increased from 10 to 20.

Observation 14: The use of longer L1 filtering window length can lead to HO KPI degradation.

Therefore, for RRM measurement prediction, we need to set a reasonable value for the L1 filtering window length considering its impact on both prediction accuracy and HO KPI.

Observation 15: There is a trade-off between prediction accuracy and HO performance depending on L1 filtering window length.

Based on the observations above, we would like to propose the following.

Proposal 5: For RRM measurement prediction, RAN2 to capture the following observations in TR 38.744 to explain the effect of L1 filtering window length on prediction accuracy and HO KPI.

- As L1 filtering window length increases, the prediction accuracy can be improved.

- The use of longer L1 filtering window length increases the delay of reflecting the real time channel variation to L3 RSRP, which can lead to HO KPI degradation.

- There is a trade-off between prediction accuracy and HO KPI on the length of L1 filtering window length.

- A reasonable value for the length of L1 filtering window should be determined considering the trade-off relation.

2.3	Cluster-based approach

For the RRM measurement result prediction, we can consider two different approaches (i.e., cell-specific approach and cluster approach). The cell-specific approach refers to “use measurement results of one cell to predict the measurement of one cell” while the cluster approach refers to “use measurement results of multiple cells to predict the measurement of one or more cells”. In our view, for RRM measurement prediction, the cluster approach can have some benefits compared to the cell-specific approach in the following aspects:

- Location (spatial) information

  - In the cluster approach, the measurement results for multiple cells are used as input of the model, which can implicitly provide the information on UE’s location (like footprint) to the prediction model. The location information can be useful for RRM measurement prediction due to the correlation between the channel condition and UE’s location.

- Diversity gain from multiple cell measurement

  - In the cell-specific approach, the measurement results for one cell may not be enough for stable prediction. For example, when the measurement results for an input cell suddenly changes due to a random blocking obstacle, it can lead to sudden change in the prediction results for an output cell even though the event does not actually affect the channel condition for the output cell. With the cluster approach, we can avoid that kind of unstable prediction by having diversity gain from multiple cell measurement.

- # of models needed for prediction

  - For the cell-specific approach, AI/ML model can be cell-specific and a separate model is required for all potential input/output cell combinations. In this case, UE should store/manage multiple models for RRM measurement prediction and may need to download a new set of models upon HO. On the other hand, for the cluster approach, one area-specific model can be used for all potential input/output cell combinations and the UE can continuously use that model within a certain area.

To check the benefit of the cluster approach over the cell-specific, we evaluate the prediction accuracy of the two approaches for both temporal domain prediction Case A in FR2 and frequency domain prediction in FR1. For the cell-specific approach, we train 21 separate models, where each model is trained by the measurement results for the corresponding cell. On the other hand, for the cluster approach, only one area-specific model is trained by the measurement results of all 21 cells in the simulation area. Other simulation assumptions are the same as in Table 2 and Table 7 for temporal domain prediction Case A and frequency domain prediction, separately. The following table shows the evaluation results.

Table 9

| Use case | Approach | AI input/output | Avg. L3 cell-level RSRP diff. (dB) | Model size |
| --- | --- | --- | --- | --- |
| Temporal domain prediction (Case A, FR2, 30km/h) | Cell-specific | Historical/future L3 cell-level RSRP for a cell | 0.863 | 200Kbyte*21 = ¦4.2 Mbyte |
|  | Cluster | Historical/future L3 cell-level RSRP for 21 cells | 2.145 | 3.5 Mbyte |
| Frequency domain prediction (FR1, 30km/h) | Cell-specific | L3 cell-level RSRP for a cell at the frequency 4GHz/2GHz | 3.611 | 200Kbyte*21 = ¦4.2 Mbyte |
|  | Cluster | L3 cell-level RSRP for 21 cells at the frequency 4GHz/2GHz | 2.657 | 3.5 Mbyte |

The evaluation results show that the cluster approach outperforms the cell-specific approach not in temporal domain prediction but in frequency domain prediction. From our understanding, in temporal domain prediction, the measurement results of the target cell has the most directly correlated information. Therefore, the measurement results from other cells may not be that useful and can be regarded as a noise if the model is too simple to handle the measurement results from multiple cells or the amount of training data set is not enough to train the complex model. On the other hand, in frequency domain prediction, it is inter-cell prediction and thus measurement results for the target cell are not given as input data. In this case, the measurement results from the multiple cells can be relatively more useful than in the time domain prediction.

Observation 16: The cluster approach outperforms the cell-specific approach in frequency domain prediction.

Observation 17: In temporal domain prediction (intra-cell prediction), the measurement results of the target cell has the most directly correlated information and thus the cluster approach using measurement results from other cells may not be that useful.

Observation 18: In frequency domain prediction (inter-cell prediction), there is no measurement results for the target cell in input data and thus the cluster approach using measurement results from other cells can be relatively more useful.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

FR2 to FR2 intra-frequency temporal domain case A (Case 4):

<The impact of OW length>

Observation 1: As we use a longer observation window, the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved). However, the accuracy improvement becomes marginal when the observation window length is over 5 samples.

Observation 2: For RRM measurement prediction, the use of a longer observation window can increase the signalling/memory overhead in LCM procedure (especially for model training/inference).

Observation 3: There is a trade-off between prediction accuracy and signaling/memory overhead depending on the length of observation window. A reasonable value for the length of observation window should be determined considering the trade-off relation.

Proposal 1: For RRM measurement prediction, RAN2 to capture the following observations in TR 38.744 to explain the effect of observation window length on prediction accuracy and signaling/memory overhead.

- As we use a longer observation window, the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved).

- The use of a longer observation window can increase the signalling/memory overhead in LCM procedure (especially for model training/inference).

- There is a trade-off between prediction accuracy and signaling/memory overhead depending on the length of observation window.

- A reasonable value for the length of observation window should be determined considering the trade-off relation.

<Use of L1 beam-level input>

Observation 4: Sub-case 2 with L3 cell-level input shows much better prediction accuracy compared to Sub-case 1&3 with L1 beam-level input.

Observation 5: The use of L1 beam-level input can increase signalling/memory overhead without gain in RRM measurement prediction.

<AI based approach vs. Non-AI based approach>

Observation 6: For Case A temporal domain prediction, AI can provide gain (i.e., lower L3 cell-level RSRP difference) compared to Non-AI (i.e., Sample and Hold). The gain increases as UE speed increases.

FR2 to FR2 intra-frequency temporal domain case B (Case 2):

<Impact of measurement reduction rate (MRRT)>

Observation 7: For Case B prediction, there is a tradeoff between measurement reduction rate and prediction accuracy (i.e., L3 RSRP difference).

Proposal 2: For Case B temporal domain prediction, RAN2 to capture the tradeoff between measurement reduction rate and prediction accuracy in TR 38.744.

Proposal 3: RAN2 to study how UE/gNB can determine a suitable value of measurement reduction rate for Case B prediction.

<AI based approach vs. Non-AI based approach>

Observation 8: For Case B temporal domain prediction, AI can provide gain (i.e., lower L3 cell-level RSRP difference) compared to Non-AI (i.e., Sample and Hold). The gain increases as MRRT increases.

Proposal 4: For Case B temporal domain prediction, RAN2 to capture that AI can provide gain (i.e., lower L3 cell-level RSRP difference) compared to Non-AI (i.e., Sample and Hold) and the gain increases as MRRT increases.

FR1 to FR1 inter-frequency (frequency domain, Case 3):

<Impact of correlation coefficient between two frequencies>

Observation 9: For the inter-frequency prediction, the evaluation results show that higher correlation coefficient between two frequency layers results in higher prediction accuracy.

<AI based approach vs. Non-AI based approach>

Observation 10: For the inter-frequency prediction, AI can provide gain (i.e., lower L3 cell-level RSRP difference) compared to Non-AI (i.e., pathloss offset).

Impact of L1 filtering window length on prediction accuracy:

Observation 11: With the agreed channel model based on TR 38.901, L1 raw RSRP of a certain beam/cell randomly fluctuates with an amplitude of about 6 dB.

Observation 12: The prediction accuracy can be improved as we use a longer L1 filtering window length. The L1 filtering window length has impact on the RSRP difference.

Observation 13: The use of longer L1 filtering window length increases the delay of reflecting the real time channel variation to L3 RSRP.

Observation 14: The use of longer L1 filtering window length can lead to HO KPI degradation.

Observation 15: There is a trade-off between prediction accuracy and HO performance depending on L1 filtering window length.

Proposal 5: For RRM measurement prediction, RAN2 to capture the following observations in TR 38.744 to explain the effect of L1 filtering window length on prediction accuracy and HO KPI.

- As L1 filtering window length increases, the prediction accuracy can be improved.

- The use of longer L1 filtering window length increases the delay of reflecting the real time channel variation to L3 RSRP, which can lead to HO KPI degradation.

- There is a trade-off between prediction accuracy and HO KPI on the length of L1 filtering window length.

- A reasonable value for the length of L1 filtering window should be determined considering the trade-off relation.

Cluster-based approach :

Observation 16: The cluster approach outperforms the cell-specific approach in frequency domain prediction.

Observation 17: In temporal domain prediction (intra-cell prediction), the measurement results of the target cell has the most directly correlated information and thus the cluster approach using measurement results from other cells may not be that useful.

Observation 18: In frequency domain prediction (inter-cell prediction), there is no measurement results for the target cell in input data and thus the cluster approach using measurement results from other cells can be relatively more useful.

4	Reference

- R2-2406095, “Summary of [POST126][031][AIMob] Simulations (OPPO)”, OPPO

- R2-2407849, “Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions (OPPO)”, OPPO

- R2-2407885, “Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions-phase 2(OPPO)”, OPPO

- R2-2409207	“Evaluation on RRM measurement prediction	“, ZTE Corporation

- R2-2410186 “Text proposal of 38.744”, OPPO

Annex: Simulation assumptions

Table 5.1-1 Simulation assumptions of FR1 and FR2

| Parameters | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; Inter-frequency correlation model is optional. without UErotation,Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; without UErotation,Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction 60,90,120 km/h for study targeting HO performance improvement | 30,60,90 km/h for study targeting measurement reduction 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Other assumptions are not precluded. |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Other assumptions are not precluded |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm (baseline) Other values (e.g., 34 dBm) not precluded |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | companies report one of the spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 | companies report one of the spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 |
| UE trajectory model | 3 options in 38.843 section 6.3.1 | 3 options in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | Companies report which of the following models they used: wrap-around model, circle-bouncing model, boundary-terminated model | Companies report which of the following models they used: wrap-around model, circle-bouncing model, boundary-terminated model |
| Sampling period | 40ms | 80ms |