3GPP TSG-RAN WG2 Meeting #129                                 R2-2500640

Athens, Greece, Feb. 17th – 21st , 2025

Agenda item:		8.3.3

Source:				Samsung

Title:					Simulation results for measurement event prediction and system level performance

Document for:		Discussion & Decision

1	Introduction

In RAN2#127bis, RAN2 agreed the following for evaluation of measurement event prediction.

Agreements:

	Measurement event prediction simulations will at least focus on intra-frequency FR2, case A, and second study goal (i.e. HO KPI improvement).   FFS what is KPI.

	Companies can bring simulation results for intra-frequency measurement reduction for FR1 and report what they are doing.  Focus on temporal case B.

	Companies will prioritize simulations on indirect method.   Companies can bring simulations on direct method and should report what method is being used.

	Measurement event prediction results are expected in RAN2#129

For measurement event prediction, according to the agreement above, we evaluate the prediction accuracy with the Case A in FR2 using the indirect method.

Meanwhile, the agreement below was made for system level performance (e.g., HO KPI) evaluation.

Agreements - The System level performance (e.g. HO performance) evaluation is optional (i.e. companies can bring results if they chose). - System level performance for measurement event prediction can be prioritized by companies if they chose to do it. - RAN2 will prioritize discussions on intermediate KPI discussion before discussing system level performance for the corresponding use case.

Even though the system level performance (e.g., HO KPI) evaluation is deprioritized, we carry out the system level simulation with high priority. In our view, the HO KPI evaluation is essential to finish this SI since it should be verified whether the use of AI/ML is really beneficial to mobility support in the end. Specifically, for Case A prediction in FR2, we need to check how much HO KPI can be improved by AI-based future event prediction. Also, for Case B prediction in FR1, it should be figured out whether the UE’s measurement overhead can be reduced with neglectable HO KPI degradation.

In this contribution, we show our initial evaluation results for both measurement event prediction and system level performance (e.g., HO KPI). Also, some observations are discussed based on the simulation results.

2	Discussion

2.1	Evaluation results for measurement event prediction

In this section, we would like to show our initial evaluation results for the measurement event prediction and share some observation based on the results. At this time, we focus on the evaluation for the event prediction with Case A in FR2 using the indirect method and the event prediction with Case B in FR1 is deprioritized according to the RAN2 agreement above.

2.1.1 General evaluation parameters/assumptions

<Simulation parameters>

For the evaluation, we adopt the agreed baseline values for Case A and set other values up to companies as below.

Table 1

| Parameters | Values | Note |
| --- | --- | --- |
| A3 event offset (db) | 2 | Open for 3db |
| TTT (ms) | 320 | Open for one shorter value |
| UE speed (km/h) | 30, 60, 90, 120 | Open for 60 and 120km/h |
| OW length (ms) | 800 | Up to implementation |
| PW length (ms) | 400 | Open for more values |
| Max ETD (ms) | 80 | Open for more values |

In RAN2#128, the definition of indirect measurement event prediction for Case A is agreed as below.

In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model.

Based on the agreement above, for the future measurement results, we reuse the output of RRM measurement prediction from Case A in FR2. Then, with the predicted future measurement results, it can be checked whether Event A3 reporting would be triggered or not within the prediction window. For the report triggering criteria, we follow the legacy entry/leaving conditions of Event A3 specified in TS 38.331 as below.

Inequality A3-1 (Entering condition) Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off Inequality A3-2 (Leaving condition) Mn + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off

In the entering/leaving condition, Mn and Mp is the (predicted) measurement results of the neighbor cells and the SpCell, separately. Off is A3 event offset that is agreed as 2dB for evaluation as in Table 1. Meanwhile, there is no agreed baseline values for MO/Cell specific offset (Ofn, Ocn, Ofp, Ocp) and the hysteresis (Hys). For the simplicity, we set those values as 0 in our simulation and would like to confirm it.

Proposal 1. For indirect Event A3 prediction, MO & Cell specific offset (Ofn, Ocn, Ofp, Ocp) and Hysteresis (Hys) values are set to ‘0’ in the entering/leaving condition.

<Simulation assumption>

The following figure shows how indirect Event A3 works in our simulation.

Figure 1. Indirect measurement event prediction

In Figure 1, at time T1 (current), with Case A prediction model, the UE can predict the future L3 RSRP in PW (T1-T4) based on the historical L3 RSRP measurement results in OW (T0-T1) for serving and neighboring cells. Then, we check whether Event A3 reporting would be triggered or not based on the predicted L3 RSRP within PW. In the example above, Event A3 entering condition is predicted to be first met at T2 and maintained during TTT (T2-T3). Thus, the output of Event A3 prediction is ‘predicted at T3’. If there is no predicted A3 event within PW, the output of Event A3 prediction is ‘not predicted’.

Meanwhile, we find some issue on prediction period that need further discussion and decision. For indirect prediction, we use the predicted L3 RSRP from RRM measurement prediction Case A in FR2. Since the agreed sample rate in FR2 is 80msec, the event prediction can be performed every 80 msec with the updated prediction results. However, in this case, the prediction windows are overlapped and the prediction is performed multiple times for the same time instance (e.g., T1) as in the option 1 in Figure 2 below. Therefore, one might think that the prediction period should be the PW length (i.e., 400 msec) as in the option 2. With the option 2, we can avoid the duplicated prediction and make the process more efficient, but the risk of missing the event increases since the number of chances to predict the event is reduced. In our evaluation, the option 1 is adopted to predict the event with higher accuracy. Nevertheless, we think it’s up to companies which option to select and each company should report the option they choose.

Figure 2. Options for prediction period

Proposal 2. For the period of indirect measurement event prediction, companies can select one of the following two options and report the option used for their evaluation.

- Option 1 (sliding): The prediction period should be set equal to the sample period.

- Option 2 (non-sliding): The prediction period should be set equal to the PW length.

2.1.2 Event prediction accuracy

For a metric of event prediction accuracy, we use F1 score, precision, and recall based on the definitions agreed in RAN2#128 as below.

F1 score = 2*Precision*Recall/(Precision + Recall) Precision = n3/(n1+n3) Recall =n3/(n2+n3) Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD]

According to the definition above, we evaluate the F1, Precision, Recall for the indirect measurement event prediction in FR2. Table 2 and Figure 3 below summarize our simulation results.

Table 2

| Metric | UE speed |  |  |  |
| --- | --- | --- | --- | --- |
|  | 30 km/h | 60 km/h | 90 km/h | 120 km/h |
| F1 score | 0.7979 | 0.8026 | 0.817 | 0.8319 |
| Precision | 0.9302 | 0.9188 | 0.9082 | 0.9111 |
| Recall | 0.6995 | 0.7134 | 0.7434 | 0.7663 |

Figure 3. Prediction accuracy of Event A3 prediction with indirect method

The results show that F1 score increases as UE speed increases. Considering that in RRM measurement prediction the prediction accuracy decreases (i.e., RSRP difference increases) as UE speed increases, this observation is quite interesting. From our understanding, for measurement event prediction, it is more important how well the AI/ML model predicts the tendency of RSRP change rather than how accurately it predicts the absolute value of RSRP. When UE speed increases, the measured RSRP values change faster and bigger, which makes it easier for the AI/ML model to predict the tendency of RSRP change. In detail, as UE speed increases, the fraction of true detections among positive detections (i.e., Precision) slightly decreases while the fraction of predicted events among actual events (i.e., Recall) increases quite a lot. Since the improvement in Recall is more distinct than the degradation in Precision, the overall F1 score can increase with UE speed.

Proposal 3. For indirect measurement event prediction, capture that the prediction accuracy (i.e., F1 score) increases with UE speed.

2.2	Evaluation results for HO KPI

In this section, we would like to show our initial evaluation results for the system level simulation (i.e., HO KPI) and share some observation based on the results. Through the HO KPI evaluation, the benefit of using AI/ML for mobility should be verified. Specifically, for Case A prediction in FR2, we need to check how much HO KPI can be improved by the measurement event prediction. Also, for Case B prediction in FR1, it should be figured out whether the UE’s measurement overhead can be reduced by RRM measurement prediction while maintaining HO KPI.

2.2.1 General evaluation parameters/assumptions

The following agreements was made on system level simulation (SLS) in RAN2#128.

Agreements on SLS : 1 As for handover model it is proposed to agree for both temporal domain case A and case B: Network start with handover preparation once a predicted measurement event is received. A handover command will be transmitted at least after preparation is completed. After that one fixed execution time is assumed. 2 To reuse HO failure model and corresponding metrics i.e. HO failure rate, total number of HO attempts per UE per second from 36.839 3 The agreed parameters in section 2.1.3 for measurement event prediction can be reused for SLS 4 The agreed parameters in table 2.2.2-1 i.e. all but last 3 parameters can be reused for both FR2 temporal domain case A and FR1 temporal domain case B in SLS 5 Interference model in section 2.2.3 is reused for SLS

Based on the yellow part, we set the parameters for SLS as summarized in Table 3 below.

Table 3

| Category | Parameters | Samsung |
| --- | --- | --- |
| Event prediction | A3 event offset | 2 dB |
|  | TTT | 320 ms |
|  | OW length | 800 ms |
|  | PW length | 400 ms (for FR2), 40/80/160 ms (for MRRT 50/66.7/80 % in FR1) |
|  | MRRT | 50%, 66.7%, 80% (only for FR1) |
|  | Prediction period | 80 ms (for FR2), 40/80/160 ms (for MRRT 50/66.7/80 % in FR1) |
| HOF/RLF | Qin threshold | -6 dB |
|  | Qout threshold | -8 dB |
|  | Sample rate (TIndication_interval) | 20 ms (FR2)/40ms(FR1) |
|  | Qin evaluation period | 100ms |
|  | Qout evaluation period | 200ms |
|  | T310 | 1000ms |
|  | N310 | 1 |
|  | N311 | 1 |

For the prediction period, we assume that the UE performs the measurement event prediction with every sample period (80 msec) in FR2. However, in case of FR1, since the UE skips real measurement with different MRRT, the different prediction period 40, 80, 160 ms are assumed for different MRRT 50, 66.7, 80 %, respectively.

2.2.2 HO KPI with Case A prediction in FR2

<Simulation assumption>

In RAN2#128, RAN2 discussed the assumption on how to improve HO KPI by measurement event prediction and the following two options are agreed as baseline HO operation for Case A prediction in FR2.

2 For measurement event prediction for temporal domain case A, company focus on option 2 or option 3. Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1)) Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

For the option 2, at current time t0, Event A3 is predicted at some point of time in future. Based on that prediction, NW can start HO preparation in advance, but it will not transmit handover command until a real measurement event is reported for the same neighbouring cell. When the UE reports the actual Event A3 at t2, NW can transmit HO command right away without the delay from HO preparation. In this way, the main benefit of this option is to save HO preparation time. Also, since NW makes HO command based on the actual measurement report from UE as in legacy, there is no risk of HO KPI degradation in this case.

For the option 3, at current time t0, Event A3 is predicted at some point of time in future. Based on that prediction, NW can start HO preparation in advance, but it will not transmit handover command until Event A3 entering condition is met based on the actual measurement results. When the entering condition is met based on the actual measurement results and the event is predicted to be met for TTT for the same neighbouring cell, the UE can report the actual Event A3 entering and NW can immediately transmit HO command. In this way, not only handover preparation time could be saved but also handover can be executed earlier than the legacy HO case. However, since NW makes HO command before the actual TTT expiry, there can be the risk of HO KPI degradation (due to too early HO) in this case.

Meanwhile, we find some issue on how to select target cell when there are multiple candidate target cells at the HO command timing. For instance, in the option 2, at the HO command timing (t2), there can be multiple cells for which the TTT expires simultaneously. Then, for the target cell selection, we can consider the following two options.

- Option A (based on real measurement results): NW can select the best cell based on the latest real RSRP measurement values as in legacy.

- Option B (based on predicted measurement results): NW can select the best cell based on the predicted future RSRP values.

From our understanding, for HO KPI evaluation in FR2, now RAN2 is focusing on how to avoid too late HO by advancing HO command timing, with the measurement event prediction. In that sense, we prefer the option A rather than option B not to mix another use case of AI prediction (i.e., HO target cell selection based on predicted RSRP). Also, when we consider the option B, there are multiple predicted RSRP values for multiple time instances within the prediction window and it is unclear which time instance should be the reference time for best cell selection. Based on the discussion above, we would like to propose the following.

Proposal 4. For SLS with Case A prediction in FR2, when there are multiple candidate cells at the HO command timing, NW selects the best cell based on the latest real RSRP measurement (i.e., not on the future RSRP prediction).

<Evaluation results>

Figure 4 below summarizes our HO KPI evaluation results for Case A prediction in FR2.

Figure 4. Evaluation results for HO KPI in FR2

For HOF rate, it is observed that AI with both option 2 and option 3 can reduce the HOF rate by 6~13% compared to the legacy baseline HO operation. Also, among the two AI options, the option 3 shows lower HOF rate than the option 2. Based on this, one may say that the option 3 outperforms the option 2. However, with the graph (b) for # of HO attempts per sec, we can realize that the option 3 reduce the HOF rate more than the option 2 by increasing the number of HO attempts. With the option 2, since NW transmits HO command based on the actual Event A3 triggering as in legacy, it shows almost same # of HO attempts with the baseline. On the other hand, in case of the option 3, NW can send the HO command in advance before the actual event triggering, which can lead to unnecessary increase in # of HO attempts.

Observation 1. For SLS with Case A in FR2, AI with both the option 2 and 3 can reduce the HOF rate compared to the legacy baseline HO. The option 3 shows lower HOF rate than the option 2.

Observation 2. For SLS with Case A in FR2, the option 2 shows almost the same # of HO attempts with the baseline, while the option 3 increases # of HO attempts by 22~62% compared to the baseline.

Based on the observations above, the benefit of the option 2 over the baseline seems clear since it reduces HOF rate while maintaining the # of HO attempts. However, for the option 3, the HO KPI metrics (HOF rate and # of HO attempts) are not enough to conclude whether the option 3 is better than the legacy baseline. For additional metric, considering that the data interruption from HO failure is most critical to UE’s data service experience, we would like to adopt # of HOF per sec as another metric for HO KPI.

Observation 3. The currently agreed metrics (i.e., HOF rate, # of HO attempts per sec) are not enough to validate the benefit of using AI through system level simulation (i.e., HO KPI evaluation).

Proposal 5. RAN2 to adopt ‘# of HOF per sec’ as additional metric for system level simulation (i.e., HO KPI evaluation) to consider the data interruption due to HOF that is most critical to UE’s service experience.

The graph (c) & (d) in Figure 4 shows our evaluation results for # of HOF per sec. From the results, it is observed that the option 2 can reduce the # of HOF compared to the baseline and the gain (i.e., decrease in the failure count) increases with UE speed. This implies that with the option 2, we can avoid ‘too late HO’ by saving HO preparation time and the number of opportunities of avoiding ‘too late HO’ with the option 2 increases with UE speed. On the other hand, the option 3 rather increases # of HOF compared to the baseline and the loss (i.e., increase in the failure count) increases with UE speed. From our understanding, the option 3 can reduce # of ‘too late HO’ by early HO, but it can also lead to increase in # of ‘too early HO’ since NW can trigger HO command before the actual Event A3 triggering. Therefore, the option 3 can increase the overall # of HOF compared to the baseline.

Observation 4. For SLS with Case A in FR2, the option 2 reduces # of HOF compared to the baseline. The gain (i.e., decrease in HOF count) increases with UE speed.

Observation 5. For SLS with Case A in FR2, the option 3 increases # of HOF compared to the baseline. The loss (i.e., increase in HOF count) increases with UE speed.

Based on the observation above, we would like to propose the following.

Proposal 6. For SLS with Case A in FR2, RAN2 to capture the following observations in TR.

- With both option 2 and 3, AI can reduce HOF rate compared to the baseline (i.e., legacy HO without AI)
- With the option 2, AI shows the almost same # of HO attempts with the baseline, while with the option 3, AI increases # of HO attempts compared to the baseline.
- With the option 2, AI reduces # of HOF compared to the baseline, while with the option 3, AI increases # of HOF compared to the baseline. 
- For the option 2, the benefit of using AI is verified through SLS.

2.2.3 HO KPI with Case B prediction in FR1

<Simulation assumption>

In RAN2#128, RAN2 discussed the assumption on how to use the actual/predicted measurement results for HO for Case B prediction in FR1 and the following is agreed as baseline HO operation.

1 For measurement event prediction for temporal domain case B: UE reports when A3 event is satisfied with actual measurements and predicted results. And handover command will be received after handover preparation. Remove T0 from the picture

With the assumption above, the HO operation becomes exactly same as in the legacy. When the entering condition of Event A3 is met for TTT, the UE sends the measurement report for Event A3 at t1 and NW can transmit HO command at t2 after finishing HO preparation. The only difference from the legacy is that the UE check the entering condition based on both real and predicted measurement results.

For the target cell selections, when there are multiple candidate cells at the HO command timing, there is the similar issue on how to select the target cell as in the Case A in FR2. However, in Case B prediction in FR1, there is no predicted future RSRP at the HO command timing and it seems reasonable for NW to select the best cell based on the latest predicted/real measurement results. Therefore, we would like to clarify the following.

Proposal 7. For SLS with Case B prediction in FR1, when there are multiple candidate cells at the HO command timing, NW selects the best cell based on the latest real/predicted RSRP measurement.

<Evaluation results>

For Case B prediction in FR1, we evaluate how much HOF rate increases with measurement reduction rate (MRRT). The following table summarizes the results.

| MRRT (Measurement reduction rate) | HOF rate |  |  |  |
| --- | --- | --- | --- | --- |
|  | UE speed |  |  | Average |
|  | 30 km/h | 60km/h | 90km/h |  |
| 0 (baseline) | 0.507 | 0.583 | 0.675 | 0.589 |
| 30% | 0.520 | 0.601 | 0.693 | 0.605 |
| 66.7% | 0.543 | 0.649 | 0.706 | 0.632 |
| 80% | 0.556 | 0.662 | 0.718 | 0.645 |

For the Case B prediction, we observe that HOF rate increases with the measurement reduction rate (MRRT). The observation seems reasonable since the prediction accuracy decreases as the UE skips the measurement more frequently. Compared to the baseline, the HOF rate increases by 2.78/7.64/9.78 % with different MRRT 30/66/80%, respectively. Based on the observation, we need to discuss how to determine the acceptable level of HOF rate increase.

Observation 6. For Case B prediction in FR1, HOF rate increases with the measurement reduction rate (MRRT).

Observation 7. For Case B prediction in FR1, compared to the baseline (no reduction), HOF rate increases by 2.78/7.64/9.78 % with different MRRT 30/66/80%, respectively.

Proposal 8. For Case B prediction in FR1, RAN2 to discuss how to determine the acceptable level of HOF rate increases based on the observation below.
- HOF rate increases with the measurement reduction rate (MRRT)
- Compared to the baseline (no reduction), HOF rate increases by 2.78/7.64/9.78 % with different MRRT 30/66/80%, respectively.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Evaluation results for measurement event prediction:

<Simulation parameters>

Proposal 1. For indirect Event A3 prediction, MO & Cell specific offset (Ofn, Ocn, Ofp, Ocp) and Hysteresis (Hys) values are set to ‘0’ in the entering/leaving condition.

<Simulation assumption>

Proposal 2. For the period of indirect measurement event prediction, companies can select one of the following two options and report the option used for their evaluation.

- Option 1 (sliding): The prediction period should be set equal to the sample period.

- Option 2 (non-sliding): The prediction period should be set equal to the PW length.

<Event prediction accuracy>

Proposal 3. For indirect measurement event prediction, capture that the prediction accuracy (i.e., F1 score) increases with UE speed.

HO KPI with Case A prediction in FR2:

<Simulation assumption>

Proposal 4. For SLS with Case A prediction in FR2, when there are multiple candidate cells at the HO command timing, NW selects the best cell based on the latest real RSRP measurement (i.e., not on the future RSRP prediction).

<Evaluation results>

Observation 1. For SLS with Case A in FR2, AI with both the option 2 and 3 can reduce the HOF rate compared to the legacy baseline HO. The option 3 shows lower HOF rate than the option 2.

Observation 2. For SLS with Case A in FR2, the option 2 shows almost the same # of HO attempts with the baseline, while the option 3 increases # of HO attempts by 22~62% compared to the baseline.

Observation 3. The currently agreed metrics (i.e., HOF rate, # of HO attempts per sec) are not enough to validate the benefit of using AI through system level simulation (i.e., HO KPI evaluation).

Proposal 5. RAN2 to adopt ‘# of HOF per sec’ as additional metric for system level simulation (i.e., HO KPI evaluation) to consider the data interruption due to HOF that is most critical to UE’s service experience.

Observation 4. For SLS with Case A in FR2, the option 2 reduces # of HOF compared to the baseline. The gain (i.e., decrease in HOF count) increases with UE speed.

Observation 5. For SLS with Case A in FR2, the option 3 increases # of HOF compared to the baseline. The loss (i.e., increase in HOF count) increases with UE speed.

Proposal 6. For SLS with Case A in FR2, RAN2 to capture the following observations in TR.

- With both option 2 and 3, AI can reduce HOF rate compared to the baseline (i.e., legacy HO without AI)
- With the option 2, AI shows the almost same # of HO attempts with the baseline, while with the option 3, AI increases # of HO attempts compared to the baseline.
- With the option 2, AI reduces # of HOF compared to the baseline, while with the option 3, AI increases # of HOF compared to the baseline. 
- For the option 2, the benefit of using AI is verified through SLS.

2 For measurement event prediction for temporal domain case A, company focus on option 2 or option 3. Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1)) Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

HO KPI with Case B prediction in FR1:

<Simulation assumption>

Proposal 7. For SLS with Case B prediction in FR1, when there are multiple candidate cells at the HO command timing, NW selects the best cell based on the latest real/predicted RSRP measurement.

<Evaluation results>

Observation 6. For Case B prediction in FR1, HOF rate increases with the measurement reduction rate (MRRT).

Observation 7. For Case B prediction in FR1, compared to the baseline (no reduction), HOF rate increases by 2.78/7.64/9.78 % with different MRRT 30/66/80%, respectively.

Proposal 8. For Case B prediction in FR1, RAN2 to discuss how to determine the acceptable level of HOF rate increases based on the observation below.
- HOF rate increases as the measurement reduction rate (MRRT) increases.
- Compared to the baseline (no reduction), HOF rate increases by 2.78/7.64/9.78 % with different MRRT 30/66/80%, respectively.