3GPP TSG-RAN WG2#129											      R2-2500641

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.8.2

Source: 	NEC

Title:	        Considering on PWS Support in NB-IoT

Document for:	 Discussion and Decision

- Introduction

In RAN2 #128 [1], regarding PWS, the following agreements were reached.

Agreements: 1. We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. 2. We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed 3. Add the following PWS indication in Paging-NB: - etws-Indication; - cmas-Indication.

In this contribution, we will discuss whether SIB1-NB acquisition is needed and provide our views.

- Discussion

In the last meeting, extending the existing ETWS/CMAS notification RRC procedure for eMTC to NB-IoT was agreed. According to the current procedure, when etws-Indication/cmas-Indication is included in paging message or direct indication information, the UE shall re-acquire SIB1-NB without waiting until the next system information modification period boundary. If the schedulingInfoList in SIB1-NB indicates that ETWS/CMAS notification is present, the UE shall acquire ETWS/CMAS notification. For NB-IoT, SIB1-NB is transmitted at a fixed schedule with a periodicity of 2560ms. Besides, the minimum periodicity value of other SIs is 640ms. Hence, when PWS arrives, the UE receives paging message, SIB1-NB and SIB10-NB/SIB11-NB/SIB12-NB in turn, which can consume too much time. Then it is also possible that PWS requirement may not be met when a small DRX cycle is configured.

Observation 1: PWS requirement may not be met even if a smaller DRX cycle is configured.

In order to reduce the latency of receiving ETWS/CMAS notification, some optimizations should be considered. In the last meeting, whether SIB1-NB acquisition is needed is FFS. If SIB1-NB acquisition procedure is removed, some latency may be reduced. But how to inform ETWS/CMAS notification needs to be considered. In our understanding, the following options may be considered:

- Option 1: ETWS primary notification, ETWS secondary notification and CMAS notification are transmitted to the UE by paging message.

  - When PWS notification is not large, it may be transmitted to the UE by paging message.

- Option 2: ETWS primary notification and scheduling information list for other SIBs can be transmitted to the UE by paging message

  - Considering ETWS primary notification is small and has higher priority, it may be firstly transmitted to the UE by paging message. Then the UE receives ETWS secondary notification and CMAS notification based on the scheduling information list in paging message.

- Option 3: scheduling information list for SIBs can be transmitted to the UE by paging message.

  - In this case, the UE receives ETWS primary notification, ETWS secondary notification and CMAS notification based on the indicated scheduling information list in paging message.

- Option 4: once etws-Indication/cmas-Indication is received, the UE starts to receive SIB10-NB/SIB11-NB/SIB12-NB at the configured NPDCCH occasions for system information.

In the following, we shall analyze the pros and cons of the above four options as shown in Table 1 below.

Table 1 Pros and Cons for the four options above

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | Latency of receiving PWS notification can be reduced | Paging message can be very large |
| Option 2 | Latency of receiving PWS notification can be reduced | Additional information can be transmitted by paging message, such as ETWS primary notification and scheduling information |
| Option 3 | Latency of receiving PWS notification can be reduced | Additional scheduling information can be included in paging message |
| Option 4 | Latency of receiving PWS notification can be reduced. No additional information in paging message | UE power consumption can increase |

Based on the analysis above, we can see that each option has its Pros and Cons. RAN2 to discuss the above options and select one option to receive PWS notification.

Proposal 1: In order to reduce the latency of receiving ETWS/CMAS notification, SIB1-NB acquisition is not needed.

Proposal 2: RAN2 to select one of the following options to receive ETWS/CMAS notification:

- Option 1: ETWS/CMAS notification can be transmitted to the UE by paging message.

- Option 2: ETWS primary notification and scheduling information for other SIBs can be transmitted to the UE by paging message.

- Option 3: Scheduling information list for SIBs can be included in paging message.

- Option 4: Upon receiving etws-Indication/cmas-Indication indication, the UE starts to receive ETWS/CMAS notification at the NPDCCH occasions.

- Conclusion

This contribution provides our analysis and views for the PWS support, the following observation and proposal are listed below:

Observation 1: PWS requirement may not be met even if a smaller DRX cycle is configured.

Proposal 1: In order to reduce the latency of receiving ETWS/CMAS notification, SIB1-NB acquisition is not needed.

Proposal 2: RAN2 to select one of the following options to receive ETWS/CMAS notification:

- Option 1: ETWS/CMAS notification can be transmitted to the UE by paging message.

- Option 2: ETWS primary notification and scheduling information for other SIBs can be transmitted to the UE by paging message.

- Option 3: Scheduling information list for SIBs can be included in paging message.

- Option 4: Upon receiving etws-Indication/cmas-Indication indication, the UE starts to receive ETWS/CMAS notification at the NPDCCH occasions.

- References

[1] Draft_RAN2_128_Meeting_Report_v1.