3GPP RAN WG2 Meeting #129	R2-2500642

# Athens, Greece, Feb. 17th – 21st,2025

Agenda Item:	8.2.2

Source:	HONOR

Title:	Discussion on A-IoT Paging

Document for:	Discussion and Decision

1. Introduction

In RAN, the following WID for paging was agreed [1]:

- A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. - Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

In this contribution, we focus on the issues related to A-IoT paging.

2 Discussion

2.1 Subsequent A-IoT Paging

The triggering of subsequent A-IoT paging

For the same service request from CN, the reader can page multiple times to detect as many devices as possible, which avoids devices not being paged due to insufficient power or random-access failure. If the number of target devices is provided to the reader, the reader can perform subsequent paging based on the number of received device IDs. Otherwise, the reader needs to determine whether to send subsequent paging message based on the detection of random-access failure. In addition, after receiving the device IDs from the reader, the CN may instruct the reader to page again if the number of acquired device IDs is not enough.

Proposal 1: The subsequent paging is initiated by the reader or the CN.

The ID information for service request

In the last meeting, RAN2 agreed to consider the scenario that different readers may send A-IoT paging messages to one specific device. Generally, the same service request from CN is sent to different readers. And for paging one or more devices, the CN may not send the same request to different readers if there is a relation between devices and reader. Therefore, it is needed to consider the ID information for same service request in different scenarios.

For the scenario that only one reader sends A-IoT paging messages to the same device, duplicate replies from the device can be avoided regardless of whether the ID is assigned by the CN or the reader. Since the ID assigned by reader will be simpler and shorter, reader can assign the ID in this scenario.

SA2 has concluded a correlation identifier used for the AIOTF to correlate the inventory responses to the request and sends it to RAN[2], while the ID size is still not determined by SA2. For the scenario of different readers may page the same device for same service request, the correlation identifier assigned by the CN can avoid duplicate responses from the device, as the device can identify the same request based on the ID regardless of the same or different readers. If the reader assigns a shorter ID, a simple option was proposed to enable CN to delete duplicated responses from different readers to the same service without further enhancing the A-IoT paging message. However, there is a case where the device mistakenly identifies the same ID associated different request from different readers are the same request. The device may skip the response to one of the service requests. Therefore, a reader ID needs to be included in paging message or considered when designing the ID for service request in paging message.

In summary, for well work in subsequent paging of above two scenarios, the detailed ID information can be:

- paging ID assigned by the reader;

- correlation identifier related the responses of request assigned by CN;

- paging ID assigned by reader with reader ID.

Since the correlation identifier size and reader ID size are not determined until now, the decision can be made when there is progress on SA2.

Proposal 2: The detailed information of ID for service request in paging message can be as following:

- Correlation identifier related the responses of request assigned by CN;

- Paging ID assigned by the reader with reader ID.

- Paging ID assigned by the reader without reader ID.

For the case of paging group IDs or without IDs, the reader cannot page the specific devices to respond. In this case, it is necessary to include an ID for the same service request in the paging message to avoid duplicate response. After receiving the paging message, if the same ID for the service request has not been received in the previous paging message, the device will respond to this paging. If the same ID has been received in the previous paging message and RA for responding the pervious paging message was unsuccessful, the device will respond to this paging. Otherwise, the device will not respond to.

Proposal 3: If the same ID for service request has not been received in the pervious paging message, the device will respond to the received paging message.

Proposal 4: If the same ID for service request has been received in the previous paging message, the device will respond to the paging message if the RA for responding the pervious paging message was unsuccessful.

2.2 The content in paging message

The scope of the WID confirms the case of a paging message contains one identifier, one identifier for a single device in the paging message may be one use case. In this case, the CFRA will be used, and subsequent paging is not needed. The situation is the same for multiple device IDs with CFRA. ID for service request used to avoiding duplicated responses is not needed for these cases, which may reduce the size of paging message or reduce the complexity and power consumption by avoiding storing ID information obtained in the paging message. Thus, the ID for service request is not mandatory for paging message.

Proposal 5: The ID for service request in the paging message is optional.

The following cases are captured in TR:

- The A-IoT paging message containing an identifier of a single A-IoT device - The A-IoT paging message containing a group ID that maps to multiple A-IoT devices - The A-IoT paging message that does not contain any identifier, i.e., indicating all A-IoT devices that can receive the A-IoT paging message need to respond - The A-IoT paging message containing multiple identifiers of A-IoT devices. The need for this use case is still to be confirmed/dependent according to the conclusion in [151]. From RAN2 perspective, it is feasible to support paging multiple identifiers of A-IoT devices, pending on TB size and multiplexing design of A-IoT paging message.

The WID restricts a paging message contains one identifier, and a paging message contains no identifier.  Since SA2 has concluded to provide the filter information which can be used to associate with multiple A-IoT devices. The one identifier can be the device ID or the ID information for a group. Therefore, one indication can be included in paging message to inform the ID type.

Proposal 6: The paging message contains the indication for the ID type of devices, e.g. device ID, filter information for multiple devices and without ID.

There is a note in WID “RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes”. Therefore, the information to identify whether the multiple identifiers are included in paging message should be introduced.

For the detailed information to identify whether the multiple device IDs is included, the following options can be considered:

- Option 1: an indication to inform whether a further device ID is included;

- Option 2: the number of device IDs;

- Option 3: the device ID field is included in the last part of paging message.

For the option 1, each included device ID can correspond to an indication. Based on this indication, the device could derive whether is a following device ID in the paging message. For the option 2, the device can get the number of device IDs in the paging message. For the option 3, the device can decode the device ID based the length of the device ID and length of the paging message PDU. In summary, all the above methods work well, RAN2 can discuss which option can be supported.

Proposal 7：RAN2 considers the following options to identify whether the multiple identifiers are included in paging message:

- Option 1: an indication to inform whether a further device ID is included;

- Option 2: the number of device IDs;

- Option 3: the device ID field is included in the last part of paging message.

Another aspect is whether the resource information should be included in paging message. The following information for D2R scheduling is captured in TR38.769 v19.0.0:

#### 6.1.2.8 Scheduling of D2R For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via the corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions - Midamble (if supported) related information For each information, it is for further study whether higher-layer signalling and/or L1 R2D control signalling is used - see Clause 6.1.1.10.

The Clause 6.1.1.10 provides many observations about whether the D2R scheduling information should be included in higher-layer signalling, which have been discussed in RAN1. The paging message is to initiate the RA procedure; therefore, it needs at least to include the resource information for the device to send the Msg1. However, whether the paging message or the L1 control signalling contains the resource information have been discussed in RAN1, RAN2 can wait the conclusions of RAN1.

Proposal 8: The PRDCH containing paging message includes the scheduling information, whether it is included in L1 control signalling or the paging message depends on RAN1.

3 Conclusions

Proposal 1: The subsequent paging is initiated by the reader or the CN.

Proposal 2: The detailed information of ID for service request in paging message can be as following:

- Correlation identifier related the responses of request assigned by CN;

- Paging ID assigned by reader with reader ID.

- Paging ID assigned by reader without reader ID.

Proposal 3: If the ID has not been received in the pervious paging message, the device will respond to the received paging message.

Proposal 4: If the ID has been received in the previous paging message, the device will respond to the paging message if the RA for responding the pervious paging message was unsuccessful.

Proposal 5: The ID for service request in the paging message is optional.

Proposal 6: The paging message contains the indication for the ID type of devices, e.g. device ID, filter information for multiple devices and without ID.

Proposal 7：RAN2 considers the following options to identify whether the multiple identifiers are included in the paging message:

- Option 1: an indication to inform whether a further device ID is included;

- Option 2: the number of device IDs;

- Option 3: the device ID field is included in the last part of paging message.

Proposal 8: The PRDCH containing paging message includes the scheduling information, whether it is included in L1 control signalling or the paging message depends on RAN1.

4. References

[1]	RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[2]	TR 23.700-13-121 Study on Architecture support of Ambient power-enabled Internet of Things