3GPP TSG-RAN WG2 Meeting #129	R2-2500643

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.2.3

Source:	HONOR

Title: 	Discussion on A-IoT random access

Document for:	Discussion and decision

1	Introduction

During RAN2#128 meeting, RAN2 discussed the A-IoT random access and made several agreements as:

Agreements for Random Access

- For 2 step and 3step CBRA, capture the unified solution in the TR and capture advantage and disadvantage of the solutions. This requires that devices would support both forms.

- FFS recommendation on whether we have 3step only or we have unified solution for 2step and 3step CBRA.

- Capture in TR as one option for unification for CBRA and CFRA. For this option random ID is always included in msg1. Capture disadvantage of reader doesn’t know whether the intended device is responding on the given resources.

- RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID

- If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported).

- The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose

- Capture the following option in TR: The devices can be triggered by R2D message to re-access before subsequent paging retransmissions.

The RAN2 finished the SI for Ambient IoT and approved the TR 38.848 as the outcome of the SI. Several potential issues related to random access are listed in the TR[1]. As agreed in RAN#106 plenary meeting, the WID[2]  of Rel-19 AIoT is agreed and includes some issues related to the random access as following:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this contribution, we analyze the remaining issues of the A-IoT random access based on TR 38.876 and provide proposals related to A-IoT random access procedure and the failure handling.

- Discussion

Ambient IoT, which is introduced as a new 3GPP IoT technology, aims to rely on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications and address some representative use cases. A-IoT random access is not the same as the NR random access or RFID access procedure since the overall device assumption and the design target are different based on the previous discussions in 3GPP working groups.

As shown in the TR, the A-IoT random access procedure consists of Random access type and access occasion/resource determination, (optional) Contention resolution of contention-based random access, and the upper layer data transmission with the reader. Furthermore, the failure handling for the random access is also important. Below, we analyse some issues found in the random access and provide the WF accordingly.

2.1 Failure detection

For 3-step CBRA, the device considers the contention resolution as successful only when the reader ack the same ID received in Msg1. In another word, the device considers the contention resolution as failed, if the Msg2 including the same ID in Msg1 is not received before the next occasion/slot. This device would join the subsequent paging procedure since it has not completed the RA for the previous paging with the same paging ID.

Observation 1: The device would detect the D2R transmission as failure if it has not received the mandatory corresponding R2D transmission before the following slot.

For 2 step random access, it is up to the reader implementation whether A-IoT Msg2 is sent in random access as shown in the TR. The device waits for the optional R2D transmission until the reception the Msg2 with its own RN or the next slot. What’s more, the device would not perform the re-access even if it has not received the Msg2 with its RN in Msg1. This issue is more severe for the 2-step CBRA resulted from the contention than CFRA. It is the same situation for the Msg3 in 3-step CBRA since the R2D failure/success feedback indication is optional. The device will not perform re-access in the following rounds if it is not aware of whether the Msg1/Msg3 is not well-received by the reader. That is, it is hard for the device to detect the failure without an implicit indication for Msg3 in 3-step CBRA and 2-step RA.

Observation 2: It is hard for the device to detect D2R data transmission failure if it has not received explicit R2D failure/success feedback indication since the subsequent R2D transmission is optional based on the reader implementation.

However, since RAN1 has studied A-IoT processing time aspects in terms of the following timing relationships:

TR2D_min: Minimum time between a R2D transmission and the corresponding D2R transmission following it. TD2R_min: Minimum time between a D2R transmission and the corresponding R2D transmission following it. TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions. TR2D_R2D_min: Minimum time between two different consecutive R2D transmissions to the same A-IoT device. TD2R_D2R_min: Minimum time between two different consecutive D2R transmissions from the same A-IoT device. For the time interval between a R2D transmission and the corresponding D2R transmission following it, there are two options studied: Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D ≥ TR2D_min.

As shown in the chart, the device expects the R2D transmission corresponding to the D2R transmission ahead would be sent before the TD2R_max.

Observation 3: Failure could be detected from perspective of device based on the TD2R_max (maximum time between the D2R transmission and the corresponding R2D transmission following it) defined by RAN1.

For example, Msg2 following the Msg1 is mandatory in the 3-step CBRA. Considering the failure occurs shortly before, it is possible that the explicit indication may be missed by the device. Furthermore, the reader could detection the contention without identifying the specific device(s) in this case. Based on the observation before, from the perspective of the device, it could regard the corresponding D2R transmission as failure if the R2D transmission is not received within the certain time from the corresponding D2R transmission. Thus, even though the explicit indication is missed by the device, it could determine the failure based on the gap between the D2R transmission and its following R2D transmission. For example, the TD2R_max could be used to determine the failure when there supposes to be a corresponding R2D transmission after the D2R transmission. The A-IoT processing time TD2R_max seems to be the pre-defined timer based on the UE/NW capability. In addition, the maximum time between the D2R transmission and the corresponding R2D transmission following it could be pre-configured by the reader based on its scheduling strategy or load condition. Thus, based on pre-configured/pre-defined timer, the failure detection from the device side could be the supplementary solution for the agreed explicit indication under the control of the network.

Proposal 1: Support the failure detection based on a timer which could be pre-configured time in the R2D trigger or the pre-defined time in RAN1.

Proposal 2: Support failure handling from perspective of the device, e.g., D2R transmission repetition upon failure detection.

2.2 Failure handling

The subsequent R2D transmission after Msg3 could be used to inform the Msg3 transmission failure and provide some performance improvement. For example, the R2D transmission after Msg3 could confirm the failure reception of the Msg3 and the absent of it indicates the success reception of Msg3. That is, the device would perform the re-access based on the failure indication and could skip the re-access as long as it is not requested explicitly.

Proposal 3: Present of the subsequent R2D transmission is to confirm the failure reception of the Msg3 while the absent of it before end of its occasion indicates the UE could skip the re-access as long as it is not requested explicitly.

In addition to the data transmission without contention, there may be contention resolution failure from perspectives of both reader and device for the following two cases: 1) more than one device selects the same access occasion; 2) the decoding failure of the received D2R message for one device. Unlike the D2R transmission failure, it is hard to locate only one device for the contention resolution failure for the first case. The default operation is already captured in the TP as that the device only determines the success of the contention resolution failure if it receives one Msg2 contains the same random number sent in its Msg1. And the re-access is not autonomously performed if the Msg2 is not received. However, it is also agreed that it is up to the reader implementation whether A-IoT Msg2 is sent in random access for 2-step CBRA. In this case, reader could skip the Msg2 even it already receives the Msg1 from the device and the device is not aware of the success of contention resolution. It requires more work to solve this problem. Thus, we propose the failure could be indicated to the device or devices in the same slot as the contention resolution failure and the re-access occasion could be assigned along with it.

Proposal 4: Support explicit R2D failure/success feedback indication for contention resolution failure.

In case of the D2R transmission after contention resolution or the failure reception of one Msg1, the reader could identify one specific device in this access occasion. It is possible for the reader to provide access occasion to one device and this device could re-send the failed D2R transmission (e.g., Msg1/Msg3). Otherwise, the reader needs to trigger another paging procedure to find the failed device(s) which results in more signalling overhead and longer latency to complete the service request and may cause possible contention in the subsequent paging. In addition, if the failure is informed timely, the failure handling could be performed with less latency. When the reader detects the D2R transmission failure, it is beneficial to perform the re-access in the same paging procedure.

Proposal 5: R2D messages between A-IoT paging messages is used by the reader to provide access occasion(s), which can be used for re-access purpose.

For both D2R transmission failure and contention resolution failure, the failure may be caused by the unavailability due to energy harvesting, coverage weakness and so on. The potential way to handle this kind of failure is to send the paging whenever the device stores enough energy and starts monitoring. Based on our understanding, higher layer repetition or multiple/subsequent paging could be used to collect all the targeted devices during one round or multiple rounds.

Proposal 6: Higher layer repetition of paging or multiple/subsequent paging could be used to handle the failure caused by un-availability.

2.3 Unified solution (i.e., solution 3)

The 3-step CBRA and 2-step CBRA have both pros and cons. 3-step CBRA reduces the negative impacts brought by contention by the short Msg1. Even contention occurs, since the Msg1 in 3-step CBRA only brings the RN16 (i.e., w/o data), the resource waste caused by collision of Msg1 is less than 2-step CBRA (i.e., Msg1with data). The 2-step CBRA provides more efficiency if there is limited collision. For example, with the assistance information (the amount of the target devices) from the CN, reader could assign a reasonable Q to reduce the contention. Thus, 3-step CBRA and 2-step CBRA could be both supported as stated in solution 3.

Proposal 7: To copy with different cases, RAN2 to support the Unified solution (i.e., solution 3) as the baseline.

2.4 Access occasion/Resource determination for Msg3

Access occasion/resource determination is the first step for the random access. The device needs to at least determine the access occasion for Msg1 based on the information in the A-IoT paging message.  Although the discussion of the access occasion is related to the detailed design of physical layer. RAN2 could still discuss the options of the resource determination for the following D2R messages like Msg3. Based on our understanding, there are two options for the resource determination of Msg3 as following:

Option 1:

The device could send the Msg3 after the confirmation of Msg2 by reusing the frequency resource and the subsequent timing resource as the Msg1. In this way, the reader could only provide/assign the resource in the paging message which reduces the signalling burden and the complexity.

Table 1 Frequency/Time resource determination for the D2R transmission for option 1

| Frequency/time Resource | Access Occasion 1 |  |  | Access Occasion 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | occasion 1 | occasion 2 | occasion 3 | occasion 1 | Occasion 2 | occasion3 |
| Freq 1 | RN1 | DL RN1, RN3 | Device ID 1 |  | DL RN2, RN4 |  |
| Freq 2 | RN2 |  |  | RN2 |  | Device ID 2 |
| Freq 3 | RN3 |  | Device ID 3 |  |  |  |
| Freq 4 | RN4 |  |  | RN4 |  | Device ID 4 |

Option 2:

The reader could re-assign or update the resource for the Msg3 by Msg2. Considering the different sizes of the Msg1 (RN16) and Msg3(device ID and/or higher layer data), reusing the frequency resource could occupy more timing resource which may cause larger latency or the waste of resources. The re-assigned or updated resource for the Msg3 provides more flexibility for the reader and improve the resource efficiency.

Table 2 Frequency/Time resource determination for the D2R transmission for option 2

| Frequency/time Resource | Access Occasion 1 |  |  | Access Occasion 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | occasion 1 | occasion 2 | occasion 3 | occasion 1 | occasion2 | occasion3 |
| Freq 1 | RN1 | DL RN1, F1+2；RN3, F3+4 | Device ID 1 |  | DL RN2, F1+2； RN4, F3+4 | Device ID 2 |
| Freq 2 |  |  |  | RN2 |  |  |
| Freq 3 | RN3 |  | Device ID 3 |  |  | Device ID 4 |
| Freq 4 |  |  |  | RN4 |  |  |

Proposal 8: RAN2 to discuss the access occasion/resource determination of Msg3: (1) assigned by the paging message (i.e., subsequent resource same as Msg1); (2) re-assigned or updated by Msg2.

3	Conclusions

In this contribution, we discussed the issues related to the random access procedure and failure handling aspects on Ambient IoT. Based on the  discussion, the following observation and proposals are concluded:

Observation 1: The device would detect the D2R transmission as failure if it has not received the mandatory corresponding R2D transmission before the following slot.

Observation 2: It is hard for the device to detect D2R data transmission failure if it has not received explicit R2D failure/success feedback indication since the subsequent R2D transmission is optional based on the reader implementation.

Observation 3: Failure could be detected from perspective of device based on the TD2R_max (maximum time between the D2R transmission and the corresponding R2D transmission following it) defined by RAN1.

Proposal 1: Support the failure detection based on a timer which could be pre-configured time in the R2D trigger or the pre-defined time in RAN1.

Proposal 2: Support failure handling from perspective of the device, e.g., D2R transmission repetition upon failure detection.

Proposal 3: Present of the subsequent R2D transmission is to confirm the failure reception of the Msg3 while the absent of it before end of its occasion indicates the UE could skip the re-access as long as it is not requested explicitly.

Proposal 4: Support explicit R2D failure/success feedback indication for contention resolution failure.

Proposal 5: R2D messages between A-IoT paging messages is used by the reader to provide access occasion(s), which can be used for re-access purpose.

Proposal 6: Higher layer repetition of paging or multiple/subsequent paging could be used to handle the failure caused by un-availability.

Proposal 7: To copy with different cases, RAN2 to support the Unified solution (i.e., solution 3) as the baseline.

Proposal 8: RAN2 to discuss the access occasion/resource determination of Msg3: (1) assigned by the paging message (i.e., subsequent resource same as Msg1); (2) re-assigned or updated by Msg2.

4	References

- TR 38.876 Study on solutions for ambient IoT (Internet of Things) (Release 19).

- RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR