3GPP RAN WG2 Meeting #129                                                     	R2-2500644

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.2.4

Source:	HONOR

Title:	Discussion on Data Transmission for Ambient IoT

Document for:	Discussion and Decision

1. Introduction

In the RAN#106 meeting, a new Work Item for Ambient IoT(A-IoT) has been approved in RP-243326 including the following RAN2 objectives on A-IoT operation [1]:

- RAN2 scope:

- Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

- A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

- A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

- A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

- Only MAC layer is included

In this contribution, we provide our considerations on the data transmission procedure, including the scope of data transmission, resource allocation, failure handling and segmentation.

2 Discussion

2.1 Scope of Data transmission

The overall AS procedures can be formulated as below [2] to support the detailed use case of "inventory-only", "inventory and command" and "command-only".

Figure 1: Overall AS procedures between A-IoT device and reader

The overall AS procedures can be formulated as:

-	Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond. See clause 6.3.3.

-	Step B: D2R data (device ID) transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure. See clause 6.3.4 (and 6.3.5).

-	Step C1: Possible R2D data transmission (e.g., for sending the command). See clause 6.3.5.

-	Step C2: Possible D2R data transmission (e.g., the corresponding response to command). See clause 6.3.5.

As highlighted in green, the data transmission includes D2R data transmission in step B and R2D data transmission/D2R data transmission in step C.

Observation 1: Data transmission includes D2R data transmission in step B and R2D data transmission/D2R data transmission in step C.

D2R data transmissions in Step B and C2 aim to transmit the upper layer data (e.g., device ID and the response of the downlink command) which could only include data of a single device. The R2D transmission in Step C1 could be used for sending the A-IoT command at least for single device completed the RA procedure. It is not clear whether there is command for a group of devices which may depend on service type of SA2 and security solution of SA3.

To discuss issues of data transmission, it is proposed RAN2 to take R2D/D2R data transmission for single device as the baseline.

Proposal 1: RAN2 to consider the D2R/R2D data transmission for single device as the baseline. Whether supports R2D data transmission targeted for more than one device depends on SA2/SA3.

2.2 Resource Allocation for D2R transmission

The A-IoT air interface is shared by all the devices, and it should be the reader to decide the transmission order and allocate the resource for efficient transmission. RAN1 has agreed to indicate the scheduling information explicitly/implicitly to the device via the corresponding PRDCH.

Since there will be a R2D data transmission firstly to trigger the following D2R transmission. It is efficient to send the scheduling information with the triggering R2D data, i.e., R2D data transmission in step C1. The scheduling information could be sent via L1 command or MAC layer. It is RAN1 to decide the detailed scheduling information.

Proposal 2: The R2D data transmission could include the scheduling information for the following D2R data transmission in step C, e.g., via L1 command or MAC layer.

A short AS ID is beneficial for D2R scheduling and R2D reception especially when there are more than one transmission rounds, e.g., multiple segments in A-IoT buffer. The device should acquire the ID before the beginning of the data transmission. Generally, there is paging/RA procedure before data transmission and could be used to assign the AS ID for following communication, e.g., via paging or MSG2.

Proposal 3: AS ID should be assigned for D2R scheduling and R2D reception before data transmission phase, e.g., via paging or MSG2.

As discussed before, the AS ID could be long-term ID or one-shot AS ID. If the AS ID is a long-term ID, it will require the A-IoT device to store the ID and use when there is A-IoT service which is quite complex for A-IoT device. Meanwhile, the length of AS ID will be longer than one-shot AS ID which may impact capacity of L1 command if it is include in L1 command.

Proposal 4: AS ID could be used in one procedure and will become invalid after energy depleted or receiving paging for the device.

2.3 Failure handling without segmentation

The table below lists the possible failure case and solutions for data transmission failure handling.

| Failure Type | Detected by device | Detected by reader | Failure handling | Spec impact |
| --- | --- | --- | --- | --- |
| R2D | Incorrect decoding | No Response in allocated resource or indicated by device. | The reader will re-transmit the R2D message. | The device should report the R2D failure. |
|  | Not received | No Response in allocated resource | The reader will re-transmit the R2D message. | NA |
| D2R | Receive re-transmitted R2D message. | No Response in allocated resource | The reader will re-transmit the R2D message (because the reader may not know whether the device transmitted the D2R message or not). | NA |
|  | Receive re-transmitted R2D message or re-transmission indicator. | Incorrect decoding | The reader will re-transmit the R2D message or re-transmission indicator. | The reader should indicate the D2R failure. |

It can be seen that the reader could detect the R2D failure if there is no D2R response in the specified resource. For the incorrect decoding case, the reader could detect the failure without receiving the response or indicated by the device in the allocated resource. However, there will be energy consumption and interference for the device to send failure indicate to the reader and will not bring any benefits.

After detecting R2D failure, the reader could re-transmit the R2D message to deal with the failure case.

Observation 2: The reader could detect the R2D failure via not receiving the D2R response in the allocated resource and could re-transmit the R2D message again to complete the A-IoT service.

For D2R failure, the reader could know the D2R lost or received incorrectly through detecting the expected D2R response in allocated resource. If there is no D2R data received, the reader should re-transmit the R2D to trigger the D2R transmission. Compared with re-transmission directly, there could be minor benefits for the reader to indicate D2R re-transmission (without upper layer data) in the incorrectly decoding case. That is., the D2R re-transmission indicator may consume less R2D resource as no upper layer data re-transmission. However, the benefit brought by the re-transmission indicator is questionable since the upper layer data is small in some cases.

Observation 3: The reader could detect the D2R failure via not receiving the D2R response in the allocated resource or not decoding the D2R response correctly and will transmit the R2D message again to complete the A-IoT service.

Proposal 5: The reader could detect the R2D/D2R data transmission failure and R2D message re-transmission could fix all the failure case.

2.4 Segmentation

The TB size of D2R data depends on D2R message size (i.e., upper layer data), target coverage, device availability, etc. Segmentation should be performed in device side if the allowed TB size can’t accommodate the whole D2R message waiting for transmission.

The reader may not know the D2R message size as CN is not required to provide the information mandatory. It will lead to resource wastage if the reader allocates more resources for small D2R data, and unnecessary service failure for energy depleted if there are lots of devices in the queue of data transmission. Or the reader may allocate less resources for large D2R data and there will be more segments for the D2R message which may increase the probability of service failure.

Observation 4: Without D2R message size information, the reader can’t allocate resource accurately which may increase the probability of the A-IoT service failure.

To support accurate resource allocation, the device should be able to report the left message size to the reader at least with the first segment. Therefore, it is proposed to include the left message size information with the segments, e.g., indicated in the header of MAC Data PDU.

Proposal 6: The left message size should be reported to the reader along with the D2R segments, e.g., the first segment or all segments.

For segmentation, the reader may discard all received segments when detects segment lost and treat the reception of the A-IoT message as failure if there is no feedback for D2R segments. This will lead to resource wastage, energy consumption and unnecessary re-triggering of the A-IoT service. It is necessary to introduce failure handling of D2R segments.

Observation 5: Failure handling of D2R segment is necessary to avoid resource wastage, energy consumption and unnecessary re-triggering of the A-IoT service.

It is assumed that all segment(s) are stored in upper layer(s) of A-IoT device and no AS layer buffering introduced for A-IoT segmentation. The A-IoT device will fetch the next segment or re-transmit the lost one when scheduled by the reader. Therefore, the device needs to know how to fetch the data.

As one option, the device could record the position of already transmitted data and fetch from the position when scheduled. However, this will require the device to store the position and calculate every time which will introduce complexity in A-IoT device.

From reader side, it will record the received bytes naturally and the device could fetch upper layer data accordingly if this information could be sent from the reader.

Proposal 7: To support (re-)transmission of segment, the position of expected received segments should be send to the device from the reader, e.g., bits received in reader side.

3 Conclusions

In this contribution, we analyze the data transmission procedure for A-IoT device and made the following observations and proposals:

Observations

Observation 1: Data transmission includes D2R data transmission in step B and R2D data transmission/D2R data transmission in step C.

Observation 2: The reader could detect the R2D failure via not receiving the D2R response in the allocated resource and could re-transmit the R2D message again to complete the A-IoT service.

Observation 3: The reader could detect the D2R failure via not receiving the D2R response in the allocated resource or not decoding the D2R response correctly and will transmit the R2D message again to complete the A-IoT service.

Observation 4: Without D2R message size information, the reader can’t allocate resource accurately which may increase the probability of the A-IoT service failure.

Observation 5: Failure handling of D2R segment is necessary to avoid resource wastage, energy consumption and unnecessary re-triggering of the A-IoT service.

Proposals

Proposal 1: RAN2 to consider the D2R/R2D data transmission for single device as the baseline. Whether supports R2D data transmission targeted for more than one device depends on SA2/SA3.

Proposal 2: The R2D data transmission could include the scheduling information for the following D2R data transmission in step C, e.g., via L1 command or MAC layer.

Proposal 3: AS ID should be assigned for D2R scheduling and R2D reception before data transmission phase, e.g via paging or MSG2.

Proposal 4: AS ID could be used in one procedure and will become invalid after energy depleted or receiving paging for the device.

Proposal 5: The reader could detect the R2D/D2R data transmission failure and R2D message re-transmission could fix all the failure cased.

Proposal 6: The left message size should be reported to the reader along with the D2R segments, e.g., the first segment or all segments.

Proposal 7: To support (re-)transmission of segment, the position of expected received segments should be send to the device from the reader, e.g., bits received in reader side.

4. References

[1]	RP-243326

[2]	TR 38.769-v1.0.0, Study on solutions for ambient IoT (Internet of Things), 2024-09