3GPP RAN WG2 Meeting #129	R2-2500645

# Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.4.2

Source:	HONOR

Title:	Discussion on LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

1. Introduction

In RAN#103 meeting, the following objectives of Rel-19 LP-WUS work item were agreed [1]:

- For IDLE/INACTIVE modes (RAN1, RAN2, RAN3, RAN4) - Specify procedure and configuration of LP-WUS indicating paging triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point.

The following agreements on LP-WUS monitoring conditions and subgrouping were made [2] in RAN2#128 meeting:

- FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session. - FFS on the following options - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - Option 2: The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - Option 3: The formula for UE_ID based PEI subgrouping is reused.

In this contribution, we focus on the discussion of the following issues for LP-WUS in RRC_IDLE/INACTIVE.

- Subgrouping

- LP-WUS and emergency PDU session

- UE paging probability information

- Monitoring condition of LP-WUS

2. Discussion

2.1. Subgrouping

Three options regarding LP-WUS subgrouping formula have been summarized in last RAN2 meeting, of which Option1 and Option2 consider the case of joint subgrouping between LP-WUS and PEI.

FFS on the following options - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - Option 2: The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - Option 3: The formula for UE_ID based PEI subgrouping is reused.

Some companies argue that power consumption introduced by false alarm could be further reduced if LP-WUS and PEI are used jointly. In our opinion, since there is no support from simulation results, the benefits are not clear. In addition, from the RAN1 agreement, joint use of LP-WUS and PEI seems not necessary.

RAN1 has been discussing the maximum number of subgroups per PO supported based on false alarm rate and power consumption and has agreed that the maximum number of subgroups per PO supported in Rel-19 is 32 [3], which has sufficient power saving gain based on simulation results.

The maximum number of subgroups per PO supported in Rel-19 is 32.

Moreover, RAN1 has defined the offset of a LO with PO/PF as the reference point [4]. If LP-WUS and PEI are used jointly, that implies the offset(s) of a LO is with respect to the MO of PEI, which is not consistent with RAN1 agreement.

On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured.

Therefore, we suggest reusing UE_ID based PEI subgrouping formula for LP-WUS, which is Option 3, instead of considering the case of joint subgrouping in UE_ID based LP-WUS formula.

Proposal 1: The formula for UE_ID based PEI subgrouping is reused for LP-WUS.

2.2. LP-WUS and emergency PDU session

RAN2 agreed to FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In our view, whether UE disables UE-ID based subgrouping when it has an emergency PDU session should depend on the additional paging latency.

For PEI, the additional paging latency can be determined by parameter pei-FrameOffset-r17, which is 0~160ms. The additional paging latency introduced by PEI is deemed tolerable. For LP-WUS, RAN1 has defined three candidate values for the wake-up delay that UE support, which are {[80ms], [500ms] and [900ms]}. Namely, the additional paging latency introduced by usage of LP-WUS are {[80ms], [500ms] and [900ms]}. Compared to additional paging latency introduced by PEI, at least the additional paging latency of 80ms should be deemed tolerable. The other two paging latency are much larger than those introduced by PEI, which are unacceptable.

In our view, when UE satisfies entry conditions and has an emergency PDU session, it can determine whether to disable UE-ID based subgrouping based on UE wake-up delay.

Proposal 2：UE can determine whether to disable UE-ID based subgrouping based on the wake-up delay it supports when it has an emergency PDU session.

2.3. UE paging probability information

Following agreement has been made in RAN2 127bis meeting.

For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion.

In Rel-17 PEI, it is specified that the UE may include its UE paging probability information in the Requested PEIPS assistance information IE if the UE has set the NR-PSSI bit to "NR paging subgrouping supported" in the 5GMM capability IE. The granularity of the paging probability is 5%.

TS 24.501

Table 9.11.3.80.1: PEIPS assistance information element

(omit)

Bit

5 4 3 2 1                        UE paging probability information value

0 0 0 0 0                                          p00

0 0 0 0 1                                          p05

0 0 0 1 0                                          p10

0 0 0 1 1                                          p15

0 0 1 0 0                                          p20

0 0 1 0 1                                          p25

0 0 1 1 0                                          p30

0 0 1 1 1                                          p35

0 1 0 0 0                                          p40

0 1 0 0 1                                          p45

0 1 0 1 0                                          p50

0 1 0 1 1                                          p55

0 1 1 0 0                                          p60

0 1 1 0 1                                          p65

0 1 1 1 0                                          p70

0 1 1 1 1                                          p75

1 0 0 0 0                                          p80

1 0 0 0 1                                          p85

1 0 0 1 0                                          p90

1 0 0 1 1                                          p95

1 0 1 0 0                                          p100

On the other hand, paging probability is usually less than 5% (e.g., 3% or 1% or <%1) in RAN1 simulation for LP-WUS power consumption. The reason is little power saving gain and even negative power saving gain may be introduced by LP-WUS if the UE paging probability is high due to the extremely high MR transition energy (15000uint ~ 40000uint) [5].

Obviously, the granularity of paging probability defined for PEI is not suitable for LP-WUS. RAN2 consider more finer granularity of paging probability (e.g., 1% or <1%) for reporting to CN, and then inform CT1 our conclusion.

Proposal 3: RAN2 consider more finer granularity of paging probability (e.g. 1% or <1%) for LP-WUS for reporting to CN.

2.4. Monitoring condition of LP-WUS

Frequent switching between LP-WUR and MR may happen when UE moves back and forth at the edge of LP-WUS coverage. As MR transition energy is huge (15000uint ~ 40000uint) [5], ping-pong between LP-WUR and MR may have a severe impact on UE power consumption. RAN2 needs to consider how to avoid ping-pong, e.g., add a hysteresis timer in entry condition.

Proposal 4: RAN2 discuss how to avoid ping-pong between LP-WUR and MR at the edge of LP-WUS coverage, e.g., add a hysteresis timer in entry condition.

The following agreement has been achieved in RAN1 118bis meeting.

- Confirm the following working assumption for iDRX: For each UE, the periodicity of LO is the same as its iDRX cycle

According to the working assumption, the following issue has been observed which is showed in the below figure.

For example, the wake-up delay supported by UE is 800ms, the I-DRX cycle configured by gNB is 640ms. When a wake-up indication received within LO1, UE would miss corresponding PO1 after it wakes up. It does not make sense to monitor LP-WUS in this scenario. It is proposed that if the wake-up delay supported by UE is larger than I-DRX cycle, UE should not monitor LP-WUS.

Proposal 5: If the wake-up delay supported by UE is larger than I-DRX cycle, UE should not monitor LP-WUS.

3. Conclusions

In this contribution, we discussed Subgrouping, LP-WUS and emergency PDU session, UE paging probability information, Monitoring condition of LP-WUS. The proposals are summarised as below.

Proposal 1: The formula for UE_ID based PEI subgrouping is reused for LP-WUS.

Proposal 2：UE can determine whether to disable UE-ID based subgrouping based on the wake-up delay it supports when it has an emergency PDU session.

Proposal 3: RAN2 consider more finer granularity of paging probability (e.g. 1% or <1%) for LP-WUS for reporting to CN.

Proposal 4: RAN2 discuss how to avoid ping-pong between LP-WUR and MR at the edge of LP-WUS coverage, e.g., add a hysteresis timer in entry condition.

Proposal 5: If the wake-up delay supported by UE is larger than I-DRX cycle, UE should not monitor LP-WUS.

4. References

[1] RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] Report of 3GPP TSG RAN WG2 meeting #128

[3] Final Report of 3GPP TSG RAN WG1 #118b v1.0.0

[4] Final Report of 3GPP TSG RAN WG1 #118 v1.0.0

[5] TR 38.869 Study on low-power wake up signal and receiver for NR